3GPP TSG-RAN WG4 Meeting #116bis            	R4-2513400

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.13.3

Source:	Samsung

Title:	View on BS demodulation requirement for MIMO with 3Tx

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WI was endorsed to support NR MIMO phase 5. One of objective related with uplink as following in WID [1]

- Objective of Core part - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Note: UL full power transmission mode 1 and mode 2 are not supported - Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching

In the last RAN4#116 meeting, the test scope of performance part of NR MIMO phase 5 was under discussed, the related agreement was captured into WF in [2] as following

- General scope - Clarify if demodulation requirements are needed for 3-antenna-port codebook-based UL transmissions - FFS on RAN4 should specify PUSCH demodulation requirements for 3-antenna-port non-coherence codebook-based UL transmissions - Interested companies are welcomed to provide simulation results next meeting - Test assumptions for PUSCH demodulation requirements (if introduced) - Duplex mode - Both FDD and TDD - Number of Rx antennas - Both 4Rx and 8Rx - SCS and CHBW for FDD - 15kHz SCS, 5MHz, 20MHz, 50MHz as starting point, further decision based on simulation results. If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage. - SCS and CHBW for TDD - 30kHz SCS, 10MHz, 40MHz, 100MHz as starting point, further decision based on simulation results. If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage. - Number of layers - only 3 layers with TPMI index 0 - Number of DMRS symbols - 1+1 - DMRS ports - Rel-15 DM-RS port as {0,1,2} - Propagation channel and MCS level - TDLB100-400, MCS 2 - TDLC300-100, MCS 16 - TDLA30-10, MCS 20 - PUSCH mapping type - Both mapping type A and type B - Other parameters - existing simulation assumption for 4Tx requirement can be reused as reference

In this contribution, the view on the test scope and test setup of BS demodulation requirement was provided.  Meanwhile, the initial simulation results are provided for feasibility study.

# 2	Discussion

## 2.1	General scope

NR supports 1-port, 2-port, 4-port and 8-port PUSCH. Nonetheless, current commercial mobile UEs are generally equipped with only 1 or 2 Tx antennas. To enhance the UL performance and anticipating advancements in hardware and design technology, the emergence of UEs equipped with 3 Tx antennas is a foreseeable development in the near future. Consequently, it is advantageous to augment the NR standards to support 3-port PUSCH supporting up to 3 layers.

In Rel-19, RAN1 has introduced the non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmission with following matrix

Table 6.3.1.5-48: Precoding matrix W for single-layer transmission using three antenna ports with 4portSRS_3TX configured. TPMI index; Precoder matrix W (ordered from left to right in increasing order of TPMI index). 0 – 2; 13100; 13010; 13001; -; -; -; -; -. Table 6.3.1.5-49: Precoding matrix W for two-layer transmission using three antenna ports with 4portSRS_3TX configured. TPMI index; Precoder matrix W (ordered from left to right in increasing order of TPMI index). 0 – 2; 13100100; 13100001; 13001001; -. Table 6.3.1.5-50: Precoding matrix W for three-layer transmission using three antenna ports with 4portSRS_3TX configured. TPMI index; Precoder matrix W (ordered from left to right in increasing order of TPMI index). 0; 13100010001; -; -; -.

Compared with the non-coherent codebook for 3 layers with 4Tx transmission as following, the power scaling factor is different.

Table 6.3.1.5-6: Precoding matrix W for three-layer transmission using four antenna ports with transform precoding disabled. TPMI index; (ordered from left to right in increasing order of TPMI index). 0 – 3. 4 – 6; -.

Observation 1: Compared with non-coherent codebook of 3 layers with 4Tx transmission, different power scaling factor is used for the non-coherent codebook of 3 layers with 3 Tx transmission

Meanwhile, the PUSCH requirement with supporting 4Tx scenario targeting for FWA or CPE devices was introduced in Rel-18 NR FR1, only 4-layer requirement was specified for 4Tx transmission, there is no specific 3-layer requirement for 4Tx transmission.

Observation 2: PUSCH requirement with supporting 4Tx transmission scenario targeting for FWA or CPE devices, only 4 layers requirement was specified for 4Tx transmission in Rel-18. There is no specific 3-layer requirement for 4Tx transmission.

Therefore, considering the difference of non-coherent codebook for 3-layer for 3Tx and 4Tx transmission, and there is also no requirement for 3-layer transmission, we think it is necessary to introduce PUSCH demodulation performance requirements for UE which support 3Tx with 3- layer transmission for guaranteeing the test coverage

Proposal 1: RAN4 should specify PUSCH requirement with 3Tx with 3-layer transmission with non-coherent codebook.

## 2.2	Test Setup

In Rel-15, RAN4 has introduced PUSCH requirements with UE supporting 1Tx and 2Tx scenarios. And PUSCH requirement with supporting 4Tx in Rel-18 NR FR1 WI. In general, the existing test configurations can be considered as reference for specifying PUSCH requirements with 3Tx transmission.

SCS and BW

In the last meeting, RAN4 agreed to perform the simulation campaign to check the performance with different CBW for both 15KHz and 30KHz SCS with following combination

- 15kHz SCS, 5MHz, 20MHz, 50MHz

- 30kHz SCS, 10MHz, 40MHz, 100MHz

Based on our evaluation in FDD with 15KHz SCS, we observed that there is 1dB performance degradation for 50MH scenario compared with 5MHz and 20MHz CBW scenarios, while there is minor difference for 5MHz and 20MHz scenarios for different MCS. The similar observation is also applied for TDD with 30KHz SCS.

Considering the test coverage and performance difference for different CBWs, we would like to propose to define the PUSCH requirements with 3 Tx-3 layers transmission for the minimum and maximum CBW for each SCS.

Proposal 2: Consider the following SCS and channel bandwidth for specifying PUSCH requirement with 3Tx with 3 layers transmission

- 15kHz SCS, 5MHz, 50MHz

- 30kHz SCS, 10MHz, 100MHz

# 3	Simulation Results

In this section, the initial simulation results with 3Tx-3 layers transmission were provided for alignment purpose.

Table 1: Initial simulation results of PUSCH with 3Tx- 3layer transmission, 15KHz SCS

| Case | Tx/Rx | SCS | BW | Channel | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3Tx/4Rx | 15KHz | 5MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.432 |
| 2 | 3Tx/4Rx | 15KHz | 5MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.594 |
| 3 | 3Tx/4Rx | 15KHz | 5MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.016 |
| 4 | 3Tx/4Rx | 15KHz | 20MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.413 |
| 5 | 3Tx/4Rx | 15KHz | 20MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.835 |
| 6 | 3Tx/4Rx | 15KHz | 20MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.679 |
| 7 | 3Tx/4Rx | 15KHz | 50MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.521 |
| 8 | 3Tx/4Rx | 15KHz | 50MHz | TDLC300-100 | A | (2,11) | MCS16 | 14.7 |
| 9 | 3Tx/4Rx | 15KHz | 50MHz | TDLA30-10 | A | (2,11) | MCS20 | 17.252 |

Table 2: Initial simulation results of PUSCH with 3Tx- 3layer transmission, 30KHz SCS

| Case | Tx/Rx | SCS | BW | Channel | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3Tx/4Rx | 30KHz | 10MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.669 |
| 2 | 3Tx/4Rx | 30KHz | 10MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.712 |
| 3 | 3Tx/4Rx | 30KHz | 10MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.090 |
| 4 | 3Tx/4Rx | 30KHz | 40MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.645 |
| 5 | 3Tx/4Rx | 30KHz | 40MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.773 |
| 6 | 3Tx/4Rx | 30KHz | 40MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.820 |
| 7 | 3Tx/4Rx | 30KHz | 100MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.724 |
| 8 | 3Tx/4Rx | 30KHz | 100MHz | TDLC300-100 | A | (2,11) | MCS16 | 14.371 |
| 9 | 3Tx/4Rx | 30KHz | 100MHz | TDLA30-10 | A | (2,11) | MCS20 | 16.900 |

Compared with different CBWs for FDD with 15KHz SCS, we observed that there is minor different for MCS 2/16 /20 under 5MHz and 20MHz scenarios, while there is about 1dB performance degradation in 50MHz scenario.

Similarly, there is minor different for MCS2/16/20 under 10MHz and 40MHz scenarios, while there is also 1dB performance degradation in 100MHz scenario.

Observation 3: Compared with 5MHz and 20MHz results for 15KHz SCS, there is minor different for MCS 2/16/20, while there is about 1dB performance degradation for 50MHz CBW scenario.

Observation 4: Compared with 10MHz and 40MHz results for 30KHz SCS, there is minor different for MCS2/16/20, while there is also 1dB performance degradation for 100MHz CBW scenario.

# 4	Conclusion

In this contribution, the view on the test scope and test setup of PUSCH requirement with 3Tx transmission was provided. Meanwhile, the initial simulation results are provided for different MCS and channel bandwidth for alignment purpose

Observation 1: Compared with non-coherent codebook of 3 layers with 4Tx transmission, different power scaling factor is used for the non-coherent codebook of 3 layers with 3 Tx transmission

Observation 2: PUSCH requirement with supporting 4Tx transmission scenario targeting for FWA or CPE devices, only 4 layers requirement was specified for 4Tx transmission in Rel-18. There is no specific 3-layer requirement for 4Tx transmission.

Proposal 1: RAN4 should specify PUSCH requirement with 3Tx with 3-layer transmission with non-coherent codebook.

SCS and BW

Proposal 2: Consider the following SCS and channel bandwidth for specifying PUSCH requirement with 3Tx with 3 layers transmission

- 15kHz SCS, 5MHz, 50MHz

- 30kHz SCS, 10MHz, 100MHz

Observation 3: Compared with 5MHz and 20MHz results for 15KHz SCS, there is minor different for MCS 2/16/20, while there is about 1dB performance degradation for 50MHz CBW scenario.

Observation 4: Compared with 10MHz and 40MHz results for 30KHz SCS, there is minor different for MCS2/16/20, while there is also 1dB performance degradation for 100MHz CBW scenario.

# References

RP-242394, WID revision: NR MIMO Phase 5, Samsung

R4-2512527, Way Forward for [116][327] NR_MIMO_demod , Samsung