3GPP TSG-RAN WG4 Meeting #116bis           	R4-2513407

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.7.3

Source:	Samsung

Title:	View on demodulation requirement for NR_NTN_Ku_bands

Document for:	Discussion

- Introduction

In the RAN#107 meeting, the revised WID on introduction of Ku bands for NR NTN was approved with the following objectives [1]

- The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - The objective of the performance part is to update as appropriate for Ku band: - RRM performance requirements and test cases - UE demodulation and CSI reporting requirements - For 15 kHz subcarrier spacing, the requirements are only applicable for GSO and low mobility scenarios. - Satellite access node demodulation requirements - Satellite access node conformance tests

According to the meeting agenda and work plan for this WI,  this is the 1st meeting to discuss demodulation requirements for SAN and UE to support Ku band.

In this contribution, the view on the test scope on SAN and UE demodulation requirement was provided.

# 2	General Scope

In this section, the view on general scope of demodulation requirement for SAN and UE is provided.

According to the WID, two Ku bands #1a and #1b for downlink block and uplink block B were defined. For FR1 numerology, the following SCS and BW combination are supported in band #1a and #1b

- 15KHz SCS: 10MHz, 15MHz, 25MHz, 35MHz, 50MHz

- 30KHz SCS: 10MHz, 20MHz, 25MHz, 35MHz, 50MHz, 70MHz and 100MHz

And for FR2 numerology, the following SCS and BW combination are supported

- 120KHz SCS: 50MHz, 100MHz, 200MHz and 400MHz

Meanwhile, for 15KHz SCS, the requirements are only appliable for GSO and low mobility scenarios.

2.1	UE demodulation requirement

PDSCH requirement

In Rel-17, RAN4 defined the limited test cases for UE supporting NTN operation. For FR1-NTN, only 4 cases were introduced including optional UE feature related with number of HARQ process and HARQ feedback disable features for 15KHz SCS with the typical channel bandwidth as 10MHz. Similar as for FR2-NTN, 4 cases were introduced for 120KHz SCS with the typical channel bandwidth as 200MHz.

Observation 1: RAN4 has already introduced the PDSCH requirement for 15KHz SCS and 10MHz CBW in FR1-NTN for FDD operation, and 120KHz SCS and 200MHz CBW in FR2-NTN.

In Rel-17 NTN demodulation requirement, for UE side, the demodulation requirement mainly focus on LEO part, and the SAN requirements could be applied for both GSO and NGSO considering the UE pre-compensation capability of NTN UE.

Observation 2: For NTN demodulation requirement, only the requirement with LEO scenario was defined for UE side, and one set of requirements was defined for SAN requirement for both GSO and NGSO scenario.

For FR1-NTN with 15KHz SCS, as indicated in the WID, the requirements are only applied for GSO and low mobility scenario. For GSO scenario, the relative motion to NTN UE is smaller, and the targeting scenario is low mobility scenario. Thus, the doppler experienced by NTN-UE is smaller. From the demodulation requirement perspective, it is less challenge compared with Rel-17 NTN for NGSO scenario. In Rel-17 NTN demodulation requirement, the UE speed is limited to 120km/h in Rel-17 discussion, and relative doppler is around 200Hz. For Ku band, since it is targeting for low mobility, to consider the test coverage, the PDSCH requirement with 15KHz could be considered.

Proposal 1: RAN4 could consider the introduction of NR NTN PDSCH requirement for 15KHz SCS with targeting for GSO scenario for Ku-band

For 30KHz SCS, only TDD requirements was introduced for TN UE, there is no FDD requirement introduced. If NTN-UE supporting NGSO scenario, there is no corresponding test defined. Meanwhile, the VSAT UE may not need to support TN network. To ensure the test coverage, it is necessary to introduce the UE demodulation requirement with 30KHz SCS with FDD operation

Proposal 2: RAN4 could consider the introduction of NR NTN PDSCH requirement for 30KHz SCS with FDD operation for Ku-band

For FR2-NTN scenario, the channel bandwidth introduced with 400MHz for 120KHz SCS only. For UE demodulation perspective, the requirement was defined in band-agnostic way with selecting the typical channel bandwidth. In current FR2-NTN, the PDSCH requirement with 200MHz was defined, although the new NTN satellite bands n512/n511/n510 is introduced with {50, 100, 200, 400} MHz bandwidth configuration for 120kHz SCS. Therefore, we think there is no UE demodulation requirement introduced for FR2-NTN supporting Ku-band.

Proposal 3: No UE PDSCH demodulation requirement introduced for NTN with supporting Ku-band

PDCCH requirement

In Rel-17 NTN discussion, assuming the NTN UE as handled UE with supporting both NTN and TN network, if the NTN UE can meet the PDCCH requirement for TN network specified in 38.101-4, RAN4 agreed to not define the PDCCH requirement in FR1-NTN.  For Rel-18, the VSAT UE is the targeting UE for above 10GHz, it may not support the TN network. Considering the test coverage, RAN4 has defined the PDCCH requirement for FR2-NTN.  For Ku-band, it also was targeting VSAT UE. Beside 60KHz and 120KHz SCS, 15KHz and 30KHz SCS are also supported with Ku band#1a and #1b, therefore, to ensure the test coverage, the PDCCH demodulation requirement with 15KHz and 30Hz SCS should be considered.

Proposal 4: RAN4 could consider the PDCCH requirement for both 15KHz and 30KH SCS for Ku-band. No new PDCCH requirements with 120 SCS for Ku-band.

CSI reporting requirement

For the CSI requirement, in both Rel-17 and Rel-18 NTN demodulation discussion, there is no CSI reporting requirement introduced, considering that the CSI may be outdated due to the large satellite RTT delay. Therefore, there is no corresponding CSI reporting requirement for both FR1-NTN and FR2-NTN. For Ku-band, we think the similar collusion can be drawn.

Proposal 5: No CSI reporting requirement for NTN with Ku-band.

2.2	SAN demodulation requirement

For SAN side, the current requirement should be applied for both GSO and NGSO scenario. In Rel-18 NTN SAN demodulation requirement discussion for FR2-NTN, RAN4 have introduced the following requirements

- PUSCH requirement

  - PUSCH requirement with CP-OFDM waveform

  - PUSCH requirement with DFT-s-OFDM waveform

  - PUSCH requirement with repetition type A

- PUCCH requirement

  - PUCCH format 0

  - PUCCH format 1

  - PUCCH format 2

  - PUCCH format 3

  - PUCCH format 4

- PRACH requirement

  - PRACH requirement with format B4 and C2

Observation 3: RAN4 has introduced the SAN demodulation requirement for FR2-NTN with 120KHz SCS

Therefore, for Ku-band with applying the FR2 numerology with 120KHz, we think the existing requirements can meet the test purpose, there is no need to introduced the new SAN demodulation requirement for Ku-band with 120KHz SCS. Regarding the 60KHz SCS, although it is supported in FR2-NTN, RAN4 only defined requirement with 120KHz SCS.

Proposal 6:  No new SAN demodulation requirements introduced for Ku-band with 120 KHz SCS and 60KHz SCS.

In Rel-17 NTN discussion, RAN4 have introduced the following requirements with both 15KHz and 30KHz SCS for both conducted and OTA testing.

- PUSCH requirement

  - PUSCH requirement with CP-OFDM waveform

  - PUSCH requirement with DFT-s-OFDM waveform

  - PUSCH requirement with repetition type A

  - PUSCH requirement with UL timing adjustment

- PUCCH requirement

  - PUCCH format 0

  - PUCCH format 1

  - PUCCH format 2

  - PUCCH format 3

  - PUCCH format 4

  - PUCCH format 2 with multi-slot

- PRACH requirement

  - PRACH requirement with format 0 and 2

For both 15KHz and 30KHz, the requirement with minimum CBW was defined if FR1-NTN, and applying the test applicability rule with different channel bandwidth to test other CBW supported. From demodulation point of view, we think there is no different with 15KHz and 30KHz SCS in Ku-band compared with FR1-NTN scenario.

Observation 4:  From demodulation point of view, no different with 15KHz and 30KHz SCS in Ku-band compared with FR1-NTN scenario.

According to WID, the minimum CBW for two Ku bands #1a and #1b is 10MHz for both 15KHz and 30KHz SCS. For 15KHz SCS, the current CBW for requirement definition in Rel-17 SAN requirement is 5MHz. If SAN declared to support the Ku band#1a, then, the test can only be proceeded to fill the 5M resource into the middle of 10MHz based on current test applicability rule. Therefore, we think there is also need to introduce the new SAN demodulation requirement both Ku-band with 15KHz and 30KHz. While from test coverage perspective, we are open to further discuss whether it is necessary to define the demodulation requirement for Ku-band with 15KHz SCS.

Proposal 7:  No new SAN demodulation requirements introduced for Ku-band with 30KHz SCS

Proposal 8:  FFS to define the SAN demodulation requirement for Ku-band with 15KHz SCS

# 3	Test Setup

In this section, the view on the test setup for SAN and UE demodulation requirement to support Ku-band operation was provided.

3.1	UE demodulation requirement

Antenna configuration

Although both FR1 and FR2 numerology are supported in Ku-band, the targeting UE type is VSAT UE with 1Rx structure. Similar as FR2-NTN, the typical antenna assumption should be 1Rx. Therefore, RAN4 could define the UE demodulation requirement with 1Rx.

Proposal 9:  Define PDSCH requirement with 1Tx1Rx for 15KHz SCS and 30KHz SCS

Channel Model

As mentioned, 15KHz is targeting the GSO and low mobility scenario.  In current requirement for NGSO with 15KHz SCS, both NTN-TDLA100-200 and NTN-TDLC5-200 were considered for requirement. For GSO scenario, the Doppler should be lower, RAN4 should further discuss the doppler value for defining requirement.

For 30KHz SCS, since there is no limitation for GSO scenario, the existing channel model can be reused for requirement with Ku-band as NTN-TDLC5-1200.

Proposal 10:  FFS on the doppler when defining the requirement with 15KHz. For 30KHz SCS, the channel model with NTN-TDLC5-1200 can be reused.

CBW for PDSCH requirement

For 15KHz SCS, the typical channel bandwidth with 10MHz was selected in FR1-NTN. And for 30KHz SCS, 40MHz CBW was considered for requirement specified in current TN specification. For Ku-band with supporting 15KHz SCS, the same CBW as 10MHz can be considered. As for 30KHz, we prefer to consider largest CBW as 100MHz for requirement definition.

Proposal 10:  RAN4 could consider the following SCS and CBW for defining PDSCH requirement as

- 15KHz SCS: 10MHz

- 30KHz SCS: 100MHz

Test cases for PDSCH requirement

As for the test cases for PDSCH requirement with 15KHz and 30KHz SCS, the following cases can be considered

table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| 1-2 | TBD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| 1-3 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| 1-4 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 100 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| 1-2 | TBD | 100 / 30 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| 1-3 | TBD | 100 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| 1-4 | TBD | 100 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Proposal 12:  RAN4 could consider the following test cases for defining PDSCH requirement with 15KHz and 30KHz in Ku-band with FDD operation

table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| 1-2 | TBD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| 1-3 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| 1-4 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-TBD | 1x1 | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 100 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| 1-2 | TBD | 100 / 30 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| 1-3 | TBD | 100 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| 1-4 | TBD | 100 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Test cases for PDCCH requirement

As for the test cases for PDCCH requirement with 15KHz and 30KHz SCS, the following cases can be considered

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 48 | 1 | 8 | TBD | NTN-TDLC5-TBD | 1x1 Low | 1 | TBD |

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 100 | 132 | 1 | 8 | TBD | NTN-TDLC5-1200 | 1x1 Low | 1 | TBD |

Proposal 13:  RAN4 could consider the following test cases for defining PDCCH requirement with 15KHz and 30KHz in Ku-band with FDD operation

# 4	Conclusion

In this contribution, the overview on test scope of SAN and UE demodulation requirement to support Ku-band operation.

PDSCH requirement

Observation 1: RAN4 has already introduced the PDSCH requirement for 15KHz SCS and 10MHz CBW in FR1-NTN for FDD operation, and 120KHz SCS and 200MHz CBW in FR2-NTN.

Observation 2: For NTN demodulation requirement, only the requirement with LEO scenario was defined for UE side, and one set of requirements was defined for SAN requirement for both GSO and NGSO scenario.

Proposal 1: RAN4 could consider the introduction of NR NTN PDSCH requirement for 15KHz SCS with targeting for GSO scenario for Ku-band

Proposal 2: RAN4 could consider the introduction of NR NTN PDSCH requirement for 30KHz SCS with FDD operation for Ku-band

Proposal 3: No UE PDSCH demodulation requirement introduced for NTN with supporting Ku-band

PDCCH requirement

Proposal 4: RAN4 could consider the PDCCH requirement for both 15KHz and 30KH SCS for Ku-band. No new PDCCH requirements with 120 SCS for Ku-band.

CSI reporting requirement

Proposal 5: No CSI reporting requirement for NTN with Ku-band.

Observation 3: RAN4 has introduced the SAN demodulation requirement for FR2-NTN with 120KHz SCS

Proposal 6:  No new SAN demodulation requirements introduced for Ku-band with 120 KHz SCS and 60KHz SCS.

Observation 4:  From demodulation point of view, no different with 15KHz and 30KHz SCS in Ku-band compared with FR1-NTN scenario.

Proposal 7:  No new SAN demodulation requirements introduced for Ku-band with 30KHz SCS

Proposal 8:  FFS to define the SAN demodulation requirement for Ku-band with 15KHz SCS

# References

RP-250799, Revised WID: Introduction of Ku bands for NR NTN