3GPP TSG-RAN WG4 Meeting 116bis	R4-2513453

# Prague, Czech Republic, October 13th – 17th, 2025

Source: 	Xiaomi

Title: 	Discussion on core requirement and testing framework for AI beam management

Agenda Item:	6.11.4

Document for:	Discussion

- Introduction

In this contribution, we will discuss core part requirement and testing framework for AI based beam management.

- Discussion

      - 2.1 Measurement window

Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement

The agreement in 38.214 is as below:

For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability.

According to the definition in 38.214, when the CSI report configuration has the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and nroftimeinstance-r19 is configured, after a CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if it receives at least KBM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of KBM is indicated by UE capability.

This definition clarifies the number of transmission occasions required per CSI-RS or SSB resource for the UE to perform inference during beam prediction. As this parameter KBM is directly related to UE capability, it consequently impacts the determination of the measurement time. Therefore, defining the measurement time must fully account for variations in UE capabilities.

Observation 1: It is already defined in 38.214 that the number of measurement instances (K_BM) required for a beam management report is dependent on UE capability.

Based on the above rationale, our initial definition for the measurement time is as follows:
In Beam Management Case 2, since the UE needs to measure RSRP across multiple time instances to predict future RSRPs, a new scaling factor Q is introduced, extending the measurement time to Q times the legacy measurement period. Different AI models may require different measurement durations, which are determined by the UE implementation. Consequently, Q depends on UE capability.

Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 for BM-case 2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N*Q)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N*Q)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N*Q)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Q is up to AI BM Case-2 UE capability KBM, reflecting the scaling factor for the measurement time. |  |

Proposal 1: For BM-case 2, introduce a scaling factor Q to adjust the measurement time for RSRP predictions, where Q is determined by UE capability, to adopt to different UE implementations.

      - 2.2 Channel model

Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing) Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. Take CDL-C Umi as one example Other channel models can be further discussed

### 2.2.1 Simplified CDL model

For CDL channel, both single probe and multiple probes are possible.

We understand the intention of discussing the issue is that there is probe limitation in chamber. Number of angles of arrival (AOA) will have impact on probe number in chamber.

For CDL model, there are several clusters and for each cluster, there are 20 rays. In order to reduce the total number of AOA, for each cluster, only one ray is considered.

Additionally, the number of clusters can be further reduced using the following options:

- Angle Offset Threshold:

  - If the AOA offset between two clusters is smaller than a threshold, these clusters can be merged into one. The AOA offset threshold may depend on the angle resolution of the RX antenna and test setup limitations, which can be further discussed in RAN4.

- Power Threshold:

  - Remove clusters with low power. The power threshold can be further discussed.

- Combination of Angle and Power Thresholds:

  - Apply both angle and power thresholds to reduce the number of clusters.

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

How to choose cluster for simplified CDL channel needs to be discussed. Here, we compare two simplified CDL-C channels and the possible impact.

Case 1: Select 8 clusters with 3 probes for simplified CDL-C

- 1st probe: cluster {#2,#3,#4}with the same angle

- 2nd probe: cluster{#6,7,8}with the same angle

- 3rd probe: cluster{#1,#5} with small angle offset

Here, 8 cluster are selected due to larger power and same or small angle offset.

| Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 |
| 8 | 0.6560 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 |

Case 2: Further simplify channel. Select 3 clusters with 1 probe.

  - Here, cluster{#6,7,8} are selected due to large power and the same angle

| Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 |
| 8 | 0.6560 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 |

For AI model, it tries to learn the spatial relation between L1-RSRP for different TX beams. Therefore, we would like to compare the distribution of L1-RSRP for different TX beams for CDL-C channel and simplified CDL-C channel. However, CDL-C defined in 38.901 only represents a single channel realization. It can’t represent the statistic spatial distribution for many different AOA, AOD, ZOD, ZOA etc.

We generate multiple BS-UE link by CDL-C rotation method defined in 7.7-5 in 38.901. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling.

(7.7-5)

By translation, mean angle can be changed to  and angular spread can be changed by scaling. Here, we didn’t change the angular spread. We only generate multiple random angles with uniform distribution. The 32 TX beam is aligned with simulation assumption agreed in [2].

Then we compare the CDF curve L1-RSRP difference between CDL-C and simplified CDL-C in Fig.2.

(a) Simplified CDL-C with 8 cluster, 3 AOA         (b) Simplified CDL-C with 3 cluster, 1 AOA

Fig.1 CDF curve L1-RSRP difference between CDL-C and simplified CDL-C

Fig.1(a) compares L1-RSRP difference between CDL-C and simplified CDL-C for case 1. Here, there are totally 8 clusters with three AOAs.

From the curve, it shows, with simplified CDL-C channel model, for the best Top-K TX beams,e.g. K=4, the L1-RSRP delta between CDL-C and simplied CDL-C model is small. For example, the L1-RSRP delta between 1st and 2th beam is 1.6dB and 1.74dB for CDL-C and simplified CDL-C for 50% cases. The L1-RSRP delta between 1st and 4th beam is 4.8dB and 5dB for CDL-C and simplified CDL-C for 50% cases. While L1-RSRP delta for some weaker beams will increase. The L1-RSRP delta between 1st and 12th beam is 12dB and 15dB for CDL-C and simplified CDL-C for 50% cases. It means that weaker beams power will decrease more than strong beam. The impact on AI prediction needs further study.

Fig.1(b) compares L1-RSRP difference between CDL-C and simplified CDL-C for case 2. Here, channel model is further simplified to 3 clusters with single AOA. It can be seen that L1-RSRP delta between TX beams increased a lot. For example, the L1-RSRP between 1st and 4th beam increase to 11dB for 50% cases.

Comparing case 1 and case 2, when cluster number is further reduced, L1-RSRP delta between different beams will increase. It’s more easiler for AI to distingwish the best Top-K TX beam. RAN4 may need to discuss how to select AOA number in order to balance test complexity and prediction performance requirement.

Observation 2: For simplified CDL-C model, L1-RSRP of weaker TX beams will decrease more than strong TX beams when compared with CDL-C model. The impact on AI prediction needs further study.

Observation 3: For simplified CDL-C model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 3: For simplified CDL model, RAN4 to choose 8 clusters and 3 AOA numbers to balance test complexity and prediction performance requirement.

      - 2.3 Test procedure

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.2 test procedure

As shown in Fig.2, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.3.1 Ground truth for best beam index

If the test metric is beam index prediction accuracy, UE will report the best beam index in set A after measurement. However, as we analysed in performance metric part, UE may not distinguish for the best 1st beam index and 2nd best beam index due to small L1-RSRP delta between the best 1st beam index and 2nd best beam index.

Therefore, UE may need to report for N best beam index. RAN4 can further discuss how to decide N.

Proposal 4: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A.

### 2.3.2 SNR impact

Same as SNR difference analysed for set B, there will be great SNR difference for 32 TX beams in set A either. If SNR for the worst TX beam is set as 10dB, then SNR for best TX beam will be too high. There may be some test issue. Suppose that if SNR of best several TX beams is set as 10dB, SNR for the worst 10 TX beam can be as low as -15dB. There will be great measurement error for the worst beam.

For set A, UE only needs to identify several best beam indexes and/or corresponding L1-RSRPs, UE doesn’t need to measure for all beams accurately. The reason is that since the L1-RSRP delta between best beam and worst beam is relatively large, the measured L1-RSRP for worst beam is still quite small, which will not exceed the value of L1-RSRP of best TX beams.

Therefore, for deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Observation 4: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Therefore, RAN4 needs to discuss how to set SNR condition for measurement of set A to derive ground truth.

Proposal 5: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

### 2.3.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 6: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 7: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.3.4 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 5: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.3(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index as shown in Fig.3(b). In this way, UE can only honestly report the best measured beam in T2.

Fig.3(a) beam index and RS index mapping relation for set A in T1

Fig.3 (b) beam index and RS index mapping relation for set A for T2

Proposal 8: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

In this contribution, we provide analysis and proposals on core requirements and the testing framework for AI-based beam management. The following observations and proposals are presented in the order they appear in the discussion:

Observation 1: It is already defined in 38.214 that the number of measurement instances (K_BM) required for a beam management report is dependent on UE capability.

Proposal 1: For BM-case 2, introduce a scaling factor Q to adjust the measurement time for RSRP predictions, where Q is determined by UE capability, to adopt to different UE implementations.

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

Observation 2: For simplified CDL-C model, L1-RSRP of weaker TX beams will decrease more than strong TX beams when compared with CDL-C model. The impact on AI prediction needs further study.

Observation 3: For simplified CDL-C model, L1-RSRP delta between different TX beams will increase if cluster number and AOA number is further reduced. It's easier for AI to distinguish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 3: For simplified CDL model, RAN4 to choose 8 clusters and 3 AOA numbers to balance test complexity and prediction performance requirement.

Proposal 4: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A.

Observation 4: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Proposal 5: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

Proposal 6: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 7: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Observation 5: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it's easy for UE to pass the test by cheating.

Proposal 8: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

[1] R1-2501595, FL summary 6 for AI/ML in beam management