3GPP TSG-RAN4 Meeting #116bis	R4-2513477

# Prague, Czech Republic, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Inter-RAT SSB measurement without MG test of 3-searcher solution for Rel-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-09-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CSSF enhancement solution 3 has agreement as followings: Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing. Issue 1-4-1: UE capability for CSSF enhancement Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG. The test case of inter-RAT measurment for 3-searcher solution is need to be added in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are in following part: - Introduced the inter-RAT measurement test case of solution 3 for TS38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for Rel-19 CSSF enhancement solution 3 ineter-RAT measurement is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.3.4, A.6.6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.6.3.4	SA NR - E-UTRAN event-triggered reporting with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in non-DRX in FR1

A.6.6.3.4.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1. This test shall partly verify the cell search and measurement requirements in clauses 9. 1.5.1.2.

In each test there are four FR1 cells in the test, Cell 1 is the NR PCell, Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell, Cell 3 and Cell 4 are deactivated SCells. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCC serving cell becomes worse than threshold1 and inter RAT neighbour cell becomes better than threshold2) is to be used. In the measurement control information from SCell, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2, Cell 3 and Cell 4.

Supported test configurations are shown in table A.6.6.3.4.1-1. General test parameters are provided in table A.6.6.3.4.1-2 below. Test parameters for Cell 1, Cell 2, Cell 3 and Cell 4, valid for both time duration T1 and T2, are provided in tables A.6.6.3.4.1-4, A.6.6.3.4.1-5 and A.6.6.3.4.1-6, respectively.

Table A.6.6.3.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.1.8.4.1-1A: Supported SCell test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1

| ConfigurationSCell | Description |
| --- | --- |
| 1A | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2A | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3A | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.6.3.4.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] for PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.4.1-2 and A.6.6.3.4.1-3. |  |
| Active cell |  | NR Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | LTE Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.3.4.1-3 |  |  |  |

Table A.6.6.1.8.2-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] for SCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasCA-Scell-r17 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Deactivated SCell |  | 1A, 2A, 3A | Cell 3, Cell 4 |  |
| RF Channel Number |  | 1, 2, 3, 1A, 2A, 3A, 4A, 5A | 1: Cell 3 and 4 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.6 |  |
| measCycleSCell |  | 1, 2, 3 | 640 ms |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 3 is 3 ms later than the timing of Cell 2. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.6.3.4.1-4: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.4.1-5: NR SCell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1

| Parameter | Unit | Test configuration | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,1A | N/A |  | N/A |  |
|  |  | 2,2A | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3,3A | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,1A | N/A |  | N/A |  |
|  |  | 2,2A |  |  |  |  |
|  |  | 3,3A |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1,1A | N/A |  | N/A |  |
|  |  | 2,2A | N/A |  | N/A |  |
|  |  | 3,3A | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,1A | N/A |  | N/A |  |
|  |  | 2,2A | N/A |  | N/A |  |
|  |  | 3,3A | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3,1A,2A,3A | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1,1A | N/A |  | N/A |  |
|  |  | 2,2A | N/A |  | N/A |  |
|  |  | 3,3A | N/A |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3,1A,2A,3A | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3,1A,2A,3A | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1,1A | -98 |  |  |  |
|  |  | 2,2A | -98 |  |  |  |
|  |  | 3,3A | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1,1A | -98 |  |  |  |
|  |  | 2,2A |  |  |  |  |
|  |  | 3,3A |  |  |  |  |
|  | dB | 1,1A | -Infinity | -1.46 | -Infinity | -1.46 |
|  |  | 2,2A |  |  |  |  |
|  |  | 3,3A |  |  |  |  |
|  | dB | 1,1A | -Infinity | 4 | -Infinity | 4 |
|  |  | 2,2A |  |  |  |  |
|  |  | 3,3A |  |  |  |  |
| SSB_RP Note 3 | dBm/SCS kHz | 1,1A | -Infinity | -94 | -Infinity | -94 |
|  |  | 2,2A | -Infinity | -94 | -Infinity | -94 |
|  |  | 3,3A | -Infinity | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,1A | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2,2A | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3,3A | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1,2,1A,2A | AWGN |  |  |  |
|  |  | 3,3A | AWGN |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.4.1-6: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NPRB,c /50) | -56.12+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.6.6.3.4.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH. Meanwhile, the UE shall send one Event A6 triggered measurement report for Cell 3 and Cell 4, with a measurement reporting delay less than 5760 ms from the beginning of time period T2 for both Cell 3 and Cell 4. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 1>

<Start of Change 2>

A.6.6.3.5	SA NR - E-UTRAN event-triggered reporting with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in non-DRX in FR1

A.6.6.3.5.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with [FR1 only CA and FR1 only NR-DC 3-searcher capability] in FR1. This test shall partly verify the cell search and measurement requirements in clauses 9.1.5.1.2.

In each test there are four cells in the test, Cell 1 is the NR PCell in FR1 band, Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell in FR1, Cell 3 is deactivated SCell in FR1 inter-band and Cell 4 is deactivated SCell in FR2 band. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCC serving cell becomes worse than threshold1 and inter RAT neighbour cell becomes better than threshold2) is to be used. In the measurement control information from SCell, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2, Cell 3 and Cell 4.

Supported test configurations are shown in table A.6.6.3.5.1-1. General test parameters are provided in table A.6.6.3.5.1-2 below. Test parameters for Cell 1, Cell 2, Cell 3 and Cell 4, valid for both time duration T1 and T2, are provided in tables A.6.6.3.5.1-4, A.6.6.3.5.1-5, A.6.6.3.5.1-6 and A.6.6.3.5.1-7, respectively.

Table A.6.6.3.5.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.1.8.4.1-1A: Supported SCell test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in FR1

| ConfigurationSCell | Description |
| --- | --- |
| 1A | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2A | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3A | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4A | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 5A | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.6.3.5.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] for PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables A.6.6.3.5.1-2 and A.6.6.3.5.1-3. |  |
| Active cell |  | NR Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | LTE Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in table A.6.6.3.5.1-3 |  |  |  |

Table A.6.6.1.8.2-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] for SCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasCA-Scell-r17 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Deactivated SCell in FR1 |  | 1A, 2A, 3A | Cell 3 |  |
| Deactivated SCell in FR2 |  | 4A, 5A | Cell 4 |  |
| RF Channel Number |  | 1, 2, 3, 1A, 2A, 3A, 4A, 5A | 1: Cell 3 and 4 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.6 |  |
| measCycleSCell |  | 1, 2, 3 | 640 ms |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 3 is 3 ms later than the timing of Cell 2. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

Table A.6.6.3.5.1-4: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.5.1-5: NR SCell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in FR1

| Parameter | Unit | Test configuration | Cell 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1,1A | N/A |  |
|  |  | 2,2A | TDDConf.1.1 |  |
|  |  | 3,3A | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,1A | N/A |  |
|  |  | 2,2A |  |  |
|  |  | 3,3A |  |  |
| RMSI CORESET RMC configuration |  | 1,1A | N/A |  |
|  |  | 2,2A | N/A |  |
|  |  | 3,3A | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,1A | N/A |  |
|  |  | 2,2A | N/A |  |
|  |  | 3,3A | N/A |  |
| OCNG Patterns |  | 1, 2, 3,1A,2A,3A | OP.1 |  |
| TRS configuration |  | 1,1A | N/A |  |
|  |  | 2,2A | N/A |  |
|  |  | 3,3A | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3,1A,2A,3A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3,1A,2A,3A | SSB |  |
| Note 2 | dBm/SCS | 1,1A | -98 |  |
|  |  | 2,2A | -98 |  |
|  |  | 3,3A | -95 |  |
| Note 2 | dBm/15 kHz | 1,1A | -98 |  |
|  |  | 2,2A |  |  |
|  |  | 3,3A |  |  |
|  | dB | 1,1A | -Infinity | -1.46 |
|  |  | 2,2A |  |  |
|  |  | 3,3A |  |  |
|  | dB | 1,1A | -Infinity | 4 |
|  |  | 2,2A |  |  |
|  |  | 3,3A |  |  |
| SSB_RP Note 3 | dBm/SCS kHz | 1,1A | -Infinity | -94 |
|  |  | 2,2A | -Infinity | -94 |
|  |  | 3,3A | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,1A | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2,2A | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3,3A | -58.50 | -56.16 |
| Propagation Condition |  | 1,2,1A,2A | AWGN |  |
|  |  | 3,3A | AWGN |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.6.3.5.1-6: NR SCell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in FR2

| Parameter |  | Unit | Cell 4 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup Note 1 |  |  | Setup 3 as specified in clause A.3.15 |  |
|  |  |  | AoA1 |  |
| Beam AssumptionNote 2 |  |  | Rough |  |
| NR RF Channel Number |  |  | 1 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  | 66 |  |
| BWP BW |  | MHz | 100: NPRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |
|  | Active DL BWP |  | DLBWP.1.1 |  |
|  | Active UL BWP |  | ULBWP.1.1 |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | SR.3.2 TDD |  |
| CORESET Reference Channel |  |  | CR.3.1 TDD |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |
| Ês |  | dBm/SCS | -Infinity | -89 |
| SSB_RP Note 3 |  | dBm/SCS Note4 | -Infinity | -89 |
| BB Note5 |  | dB | -Infinity | -0.12 |
| Io Note3 |  | dBm/95.04 MHz Note6 | -Infinity | -64.41 |
| Propagation Condition |  |  | AWGN |  |
| NOTE 1: The configuration for AoA1 and AoA2 can refer to Figure A.7.6.1.5.1.-1 NOTE 2: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. NOTE 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone. |  |  |  |  |

Table A.6.6.3.5.1-7: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with [FR1+FR2 CA (PCell in FR1) 3-searcher capability] in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9 MHz | 1, 2, 3, 4, 5, 6 | -73.21+10log (NPRB,c /50) | -56.12+10log (NPRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.6.6.3.5.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84 s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH. Meanwhile, the UE shall send one Event A6 triggered measurement report for Cell 3 and Cell 4, with a measurement reporting delay less than 5760 ms from the beginning of time period T2 for Cell 3 and a measurement reporting delay less than 1000 ms from the beginning of time period T2 for Cell 4. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 2>