3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513494

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on performance requirements for low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	6.3.2

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides further analysis on performance requirements for low band carrier aggregation via switching.

Discussion

- SA event triggered reporting tests without gap

As we discussed in previous meeting, regarding the L3 measurement period, in low band CA switching gap-less case, in most cases there is an available SMTC on SDL/PCC every 40ms, as the periodicity of switching pattern is 40ms. Accordingly, the L3 measurement delay will not exceed 200ms (low boundary of existing measurement period), even if switch pattern is present. The case that measurement period is further prolonged due to LB-CA switching pattern happens when measurement gap and SDL/PCC SMTC are partially overlapped. Figure 1 provides one example which takes L3 measurement period of SDL carrier as an example. Likewise it can be applied to L3 measurement period of PCC in LB-CA scenario.

- SDL SMTC is 10ms, three SMTC locations are colliding with FDD pattern, and 1 SDL SMTC is allocated for SDL ON duration. However the SDL SMTC is colliding with measurement gap, so Kp=8 (considering both meas gap and LB-CA switching pattern) then: max(200 ms, ceil( 5 x Kp) x SMTC period)= max(200 ms, 400 ms)=400ms

Figure 1. L3 measurement period is extended due to partial overlapping with switch pattern and MG

Regarding defining test case, we have no strong view on whether to verify the case that the measurement period is extended due to switch pattern or not. However we think it is still meaningful to verify the legacy measurement period in LB-CA scenario, because LB-CA switching scheme is a new functionality, and the UE L3 measurement behaviour shall be tested under this scenario. Accordingly, the measurement accuracy test shall be also defined.

So it is straight forward to verify SA event triggered reporting test without gap on FDD and measurement accuracy test due to LB-CA switching.

As we know, in legacy the event trigger reporting test on SCC is only defined for FR2 HST CA. In R19 LB-CA switching, mobility function is based on PCC.  SCell change rarely occurs since in R19 LB-CA only two carrier aggregation is considered. It seems it is not typical to configure event on SDL for reporting. We also understand that companies would like to test the UE measurement behaviour on SDL carrier. So we have no strong view on whether to define event trigger reporting test on SDL carrier.

Proposal 1: For LB-CA switching, the following tests are to be defined:

-SA event triggered reporting test without gap on PCC

-Measurement accuracy on PCC

Proposal 2: Discuss whether to verify the extended L3 measurement period due to switch pattern or to verify the legacy L3 measurement period.

Proposal 3: Discuss on whether to define event trigger reporting test on SDL carrier for LB-CA switching.

- Interruption due to measurement on deactivated SDL SCell

The switching pattern specified in RAN1 is not applicable to deactivated carriers. It is agreed in RAN4 that the legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. As the legacy measurement requirement is reused, RAN4 not to define SA event triggered reporting test on deactivated SCC.

Moreover the interruptions on active serving cell(s) due to L3 measurement on deactivated SDL carrier is quite different with the legacy requirements. It is straight forward to specify test for interruptions during measurements on deactivated SDL carrier in FR1. In our understanding, even without defining event triggering reporting test, defining interruption test on deactivated SDL can effectively validate UE behaviour on deactivated SDL.

Proposal 4: For LB-CA switching, test case of interruption during measurements on deactivated SDL carrier is to be specified.

- SDL SCell activation delay

As per the latest conclusion, the RRC switch pattern is applied to SCell activation procedure which is quite different with the legacy SCell activation procedure. So it is straight forward to specify test case for SDL SCell activation delay.

Proposal 5: For LB-CA switching, test case of SDL SCell activation delay is to be specified.

- BFD/CBD, TCI state switch and L1-RSRP/SINR requirements for SDL SCell

Issue 1-3-6: BFD/CBD requirements for SDL SCell [Agreement]: - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching

Issue 1-3-11: TCI state switch delay requirement for SDL SCell Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Based on the above agreements in RAN4, the following test cases are to be specified for SDL SCell:

Test case for Beam Failure Detection and Link Recovery Test on SDL SCell

Test case for Active TCI state switch delay on SDL SCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Proposal 6: For LB-CA switching, the following test cases are to be specified on SDL SCell:

Test case for Beam Failure Detection and Link Recovery Test on SDL SCell (depending on conclusion of core requirements)

Test case for Active TCI state switch delay on SDL SCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

In LB CA switching, the SSB/CSI-RS based L3/L1 measurement, TCI state switching, BFD/CBD evaluation on PCC are supposed to be performed on the overlapped occasions between SMTC window and PCC duration in switch pattern. Most test specified for SDL carrier shall be also verified on PCC in LB CA via switching.

Proposal 7: For LB-CA switching, the below test cases are to be specified on FDD PCell:

Test case for Beam Failure Detection and Link Recovery Test on FDD PCell

Test case for Active TCI state switch delay on FDD PCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Conclusions

This contribution provides further analysis on performance requirements for low band carrier aggregation via switching. The following proposals are provided:

Proposal 1: For LB-CA switching, the following tests are to be defined:

-SA event triggered reporting test without gap on PCC

-Measurement accuracy on PCC

Proposal 2: Discuss whether to verify the extended L3 measurement period due to switch pattern or to verify the legacy L3 measurement period.

Proposal 3: Discuss on whether to define event trigger reporting test on SDL carrier for LB-CA switching.

Proposal 4: For LB-CA switching, test case of interruption during measurements on deactivated SDL carrier is to be specified.

Proposal 5: For LB-CA switching, test case of SDL SCell activation delay is to be specified.

Proposal 6: For LB-CA switching, the following test cases are to be specified on SDL SCell:

Test case for Beam Failure Detection and Link Recovery Test on SDL SCell (depending on conclusion of core requirements)

Test case for Active TCI state switch delay on SDL SCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Proposal 7: For LB-CA switching, the below test cases are to be specified on FDD PCell:

Test case for Beam Failure Detection and Link Recovery Test on FDD PCell

Test case for Active TCI state switch delay on FDD PCell

Test case for L1-RSRP measurement for beam reporting on SDL SCell

Test case for L1-SINR measurement for beam reporting on SDL SCell

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19

- R4-2512119, WF on RRM requirements for NR_LBCA_Sw, Apple