3GPP TSG-RAN WG4 Meeting #116bis	R4-2513503

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Further consideration on performance part for LP-WUR

Agenda Item:	6.16.3

Document for:	Discussion

Introduction

In [1], the time budget for LP-WUR WI [2] performance part is included and the TU plan is copied below for reference.

LP-WUR performance part work has already started at RAN4 115 meeting. In this contribution, we provide our considerations on the LP-WUR performance part.

Discussion

Discussion

General aspects

Issue 3-1-2 General aspects

- Proposals

  - P1: RAN4 to focus on time and delay behavior of RRM offloading, RRM relaxation and LP-WUR monitoring modes. (Apple vivo)

  - P2: Do not define a test mode for LP-WUS/WUR performance part. (Apple)

  - P3: Define test mode for LP-WUR serving cell measurement tests (oppo Nokia)

  - P4: Defining TCs for LPWUS cannot verify whether UE meet the new RRM relaxation requirements in R19 LPWUS or the legacy requirements without RRM relaxation (in both cases UE will pass the test). The TCs are redundant. (MTK)

The topic for general aspects is mainly related to whether to consider using test mode in the LP-WUR test case design. Our consideration is the test mode should be last resort when all other solutions are not able to handle the LP-WUR test which is very unlikely to be the case.

Proposal 1: Focus on test case design, the test mode should be used as the last resort.

LP-WUR test case design

Issue 3-2-1: Test case scenarios

- Proposals

  - P1: Consider test case for the following scenarios: 1: LP-WUR monitoring related test case; 2:  RRM offloading related test case; 3: RRM relaxation related test case (CATT China telecom vivo)

  - P2: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.(Ericsson Nokia)

Recommendations:

Regarding test case scenario, the only difference between P1 and P2 is whether to consider test case for the LP-WUS monitoring scenario. For the LP-WUR state, the verification on miss detection/false alarm etc. should be considered in demodulation session. For the LP-WUR related paging reception duration, the related part could be verified is the wake up delay of the MR, i.e., the time when the LP-WUR detect the paging indicator till the time when MR is ready for paging reception. However the issue is there in no directly RAN4 requirement on wake up delay hence whether and how to verify the wake up delay needs further study. We also agree that it is hard to verify LP-WUS monitoring since the paging can be monitored by MR anyway even in the LP-WUS monitoring state. On the other hand the MR wake up delay is reused in “exiting case 1 due to offloading conditions cannot be met” in [3] at RAN4 116 meeting:

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met

Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

Hence if the wake up delay can be verified in case 1 related test case, there is no need to define test case for LP-WUS monitoring scenario.

For the LP-WUR operation, when the entry conditions are satisfied, there is no mandatory on UE behavior, for example as the following:

“The gNB configures in system information entry and exit conditions to monitor LP-WUS. The UE may start monitoring LP-WUS when measurements using the MR are above the configured entry threshold(s), and the measurements using the LR are above the entry threshold(s) if configured. Entry conditions for LP-WUS monitoring are based on MR and optionally LR measurements as specified in TS 38.304.”

Hence for test cases for offloading and RRM relaxation, only the exit from one state to another state needs be verified. This means at the beginning of a test case, the UE will already in the offloading or RRM relaxation or LP-WUR monitoring mode.

Proposal 1: Define test cases for offloading and RRM relaxation, for offloading and RRM relaxation, only the exit from one state to another state needs be verified. And at the beginning of a test case, the UE will already in the offloading or RRM relaxation mode.

Proposal 2: Define test case for LP-WUR monitoring mode only for MR wake up delay purpose if the MR wake up delay verification is not covered by other test cases.

Issue 3-2-2: Test case for OOK based LR and OFDB based LR

- Proposals

  - P1: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR. (Apple oppo CMCC China telecom vivo Huawei Nokia)

  - P2: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE (Ericsson)

  - P3: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB.(Ericsson)

  - P4: Discuss how to define a test case / configuration for OFDM capable UE (FG-62-1a) which is only configured with LP-RSRP & LP-RSRQ (no SS-RSRP, SS-RSRQ configuration). (Nokia)

Regarding OOK based LR or OFDB based LR, since the reference signal is different, separate sets of test cases are needed. Of course the number of test case for each set could be further discussed. For the OOK based LP-WUR, in the test case, only the LP-SS will be sent and SSB will not be sent.

In addition, multiple {M, L} setting for LP-SS were defined and to guarantee the performance of OOK based LR, the test case should cover sufficient settings of all possible {M, L} combinations. Among the following setting of {M, L}, typical case should be considered and it is suggested that the test case should cover at least M = 1 and M > 1 cases.

{M, L} setting: {1, 6} {1, 8} {2, 12} {2, 16} {4, 16} {4, 32}

For the case when M= 1 , suggest to use {M, L} = {1, 8}; for the case when M>1, suggest to use {2,12} or both {2, 12} and {4, 32}. In addition, it does not mean separate test case should be designed to cover these suggested {M,L} settings, actually, more than one {M,L} settings can be verified within a single test case in different time period.

Proposal 3: Define separate test cases for LP-SS based LR and SSB based LR. In the test case for LP-SS based LR, only LP-SS will be sent and the SSB will not be sent.

Proposal 4: For the OOK based LR, test cases should cover at least M = 1 and M > 1 cases. Suggest to use {M, L} = {1, 8}when M = 1, and use {2,12} or both {2, 12} and {4, 32} when M > 1. More than one {M,L} settings can be verified within a single test case in different time period.

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3

- Proposals

- Entry/exit condition evaluation verification

  - Option 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions (Apple CMCC vivo Huawei QC)

  - Option 2: Define test case for entry scenario (oppo Nokia)

    - Option 2-1 (oppo)

      - Scenario 1: UE enter Case 1 from legacy and exit to legacy

      - Scenario 2: UE enter Case 3 from legacy and exit to legacy

      - Scenario 3: UE enter Case 1 from Case 3

    - Option 2-2: Use test mode / procedure for LR entry measurements (Nokia)

  - Option 3: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).(MTK)

- Detail on exit condition evaluation verification:

  - P1: Define following tests for exit criteria evaluation (CMCC)

    - LR exit criteria evaluation test when exit from LP-WUS monitoring

    - LR exit criteria evaluation test when exit from Case 1

    - FFS exit criteria evaluation test when exit from Case 3, wait the RAN2’s conclusion of exit condition

  - P2: Define test cases for evaluation of exit conditions for the following cases. (Huawei)

    - Case 1: UE exist Case 1 to legacy

    - Case 2: UE exist Case 3 to legacy

  - P3: Introduce the following test cases where the exit conditions can be tested (QC)

    - UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode

    - UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode.

As analysed above, it is suggested that only define exit conditions for case 1 and case 3. For simplicity, it is suggested to consider the proposal of P2. Other cases, such as exit from case 1 to case 3, exit from case 3 to case 1, are no need to be covered.

Proposal 5: Define the following test cases for case

  - Case 1: UE exist Case 1 to legacy state, separate cases for OOK based LR and OFDM based LR; for the LP-SS sequence for OOK based LR, {M, L} could use {1, 8} or one from { {2,12}, {4, 32} }

  - Case 2: UE exist Case 3 to legacy state, separate cases for OOK based LR and OFDM based LR, {M, L} could use {1, 8} or one from { {2,12}, {4, 32} }

  - Use different {M, L} between case 1 and case 2

Regarding the exit from case 1 or case 3 to legacy case, when the UE exit the offloading case and triggers the MR or when the UE exit from the RRM relaxation state to legacy state, actually no feedback signal was sent from a UE to network hence the total duration is untestable. A joint procedures can be considered, for example, at the beginning time point A, a UE is in the RRM offloading state or in RRM relaxation state, a particular condition can be designed to trigger the MR at time point B to move the UE to the legacy state and the MR is ready at time point C, then a procedure (for example a cell reselection procedure)which could provide feedback signal to the TE is followed immediately at the time point C. In the end at time point D the UE feedback to TE. The total delay can be derived based on the delay of each procedure the exit from offloading to legacy state can be verified.

Proposal 6: The test cases defined for UE exists case 1 to legacy state and UE exists case 3 to legacy state, could have the following time durations:

    - T1: at T1 the UE is in case 1 or case 3 and camping on cell 1, cell 2 is not detectable

    - T2: at beginning of T2 the UE exists case 1 or case 3, at the end of T2 the UE is in legacy state, camping on cell 1

    - T3: at the beginning of T3 cell 2 is detectable and the UE should finish cell reselection within T3 and at the end of T3 UE should send preambles on the PRACH on cell 2

Issue 3-2-4: Test case design for RRM relaxation

- Proposals

  - P1: For RRM relaxation, the following tests can be considered: (CATT oppo CMCC vivo Huawei Ericsson)

    - intra-frequency cell re-selection

    - inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

    - inter-RAT cell re-selection

  - P1-1: whether to introduce all the former 3 test cases for each LR type, i.e., the LP-SS based LR or the SSB based LR, could be further discussed. (vivo)

  - P2: In case 3, the test cases for cell reselection shall be considered and the specific number of test cases could be discussed in this meeting (ZTE)

  - P3: Introduce reselection test case(s) for PSS/SSS and LP-SS considering intra-frequency, inter-frequency and both, MR relaxation / offloading cases.

  - P4: RAN4 to discuss whether Rel-16 test cases for RRM relaxation based on cell reselection can be applied for Case#3. (MTK)

Recommendations:

For the RRM relaxation test case, the Rel-16 procedure could be followed and the following test case could be introduced:

  - intra-frequency cell re-selection (for OOK based LR and/or OFDM based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (for OOK based LR and/or OFDM based LR)

  - inter-RAT cell re-selection (for OOK based LR and/or OFDM based LR)

For these test cases, since the cell reselection duration is fully based on the MR, which type of LR used in the test case setup is not relevant providing the UE does not leave the RRM relaxation mode. From the test case coverage point of view, it can have one for OOK based LR and one for OFDM based LR among the three test cases.

Proposal 7: Suggest to introduce the following test cases for RRM relaxation:

  - intra-frequency cell re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (for LP-SS based LR and/or SSB based LR)

Proposal 8: From the test case coverage point of view, it can have one for OOK based LR and one for OFDM based LR among the three test cases defined for RRM relaxation.

Issue 3-2-6: Test case design for higher priority frequency layer search

- Proposals

  - Option 1: Introduce test case for high priority frequency search in case 1. (Apple oppo Huawei ZTE)

  - Option 2: For RRM measurement offloading, not to introduce test case for higher priority carrier Regarding higher priority frequency layer cell reselection, suggest no test case since the test duration is quite long. (CMCC vivo)

For the UE stays in case 1, the UE will have higher priority frequency layer search with the rate of 1 hour per frequency layer. Considering the measurement period is quite long, it is suggested not to introduce the higher priority frequency layer search test case when UE is in the offloading state.

Proposal 9: Suggest do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode.

Issue 3-2-8: On FR1/FR2 for test case

- Proposals

  - P1: RAN4 to only introduce FR1 based LP-WUS test case. (Apple China Telecom Ericsson)

Recommendations:

For the test case design for FR1/FR2, since LR requirements are only defined for OFDM based LR and the same value of N1 is reused, the existing test case and test case defined for FR1 should already cover FR2 scenario hence suggest to define test case for FR1 only.

Proposal 10: RAN4 to only introduce FR1 based LP-WUS test case.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: Define test cases for offloading and RRM relaxation, for offloading and RRM relaxation, only the exit from one state to another state needs be verified. And at the beginning of a test case, the UE will already in the offloading or RRM relaxation mode.

Proposal 2: Define test case for LP-WUR monitoring mode only for MR wake up delay purpose if the MR wake up delay verification is not covered by other test cases.

Proposal 3: Define separate test cases for LP-SS based LR and SSB based LR. In the test case for LP-SS based LR, only LP-SS will be sent and the SSB will not be sent.

Proposal 4: For the OOK based LR, test cases should cover at least M = 1 and M > 1 cases. Suggest to use {M, L} = {1, 8}when M = 1, and use {2,12} or both {2, 12} and {4, 32} when M > 1. More than one {M,L} settings can be verified within a single test case in different time period.

Proposal 5: Define the following test cases for case

  - Case 1: UE exist Case 1 to legacy state, separate cases for OOK based LR and OFDM based LR; for the LP-SS sequence for OOK based LR, {M, L} could use {1, 8} or one from { {2,12}, {4, 32} }

  - Case 2: UE exist Case 3 to legacy state, separate cases for OOK based LR and OFDM based LR, {M, L} could use {1, 8} or one from { {2,12}, {4, 32} }

  - Use different {M, L} between case 1 and case 2

Proposal 6: The test cases defined for UE exists case 1 to legacy state and UE exists case 3 to legacy state, could have the following time durations:

    - T1: at T1 the UE is in case 1 or case 3 and camping on cell 1, cell 2 is not detectable

    - T2: at beginning of T2 the UE exists case 1 or case 3, at the end of T2 the UE is in legacy state, camping on cell 1

    - T3: at the beginning of T3 cell 2 is detectable and the UE should finish cell reselection within T3 and at the end of T3 UE should send preambles on the PRACH on cell 2

Proposal 7: Suggest to introduce the following test cases for RRM relaxation:

  - intra-frequency cell re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (for LP-SS based LR and/or SSB based LR)

Proposal 8: From the test case coverage point of view, it can have one for OOK based LR and one for OFDM based LR among the three test cases defined for RRM relaxation.

Proposal 9: Suggest do not define test case for higher priority frequency layer cell reselection when UE is in offloading mode.

Proposal 10: RAN4 to only introduce FR1 based LP-WUS test case.

- References

[1] RP-250821, TU budget after RP107, RAN#107

[2] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[3] R4-2512139, WF on RRM requirements for NR_LPWUS, RAN4 116, vivo