3GPP TSG-RAN WG4 Meeting #116bis	R4-2513547

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.19.4

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for Rel-19 NR NTN Phase 3

Document for:	Discussion

# Introduction

Based on the WF [1], we provide our perspectives on the unresolved issues for UE demodulation requirements for Rel-19 NR NTN Phase 3.

- Discussion

RAN4 agreed on test cases for RedCap/eRedCap for NTN at the last meeting [1]. The remaining issues are the FRC for the agreed test cases.

Issue 2-1-2: FRC for HD-FDD RedCap demodulation Way forward: - Reuse “R.PDSCH.1-1.2 HD-FDD” in TS38.101-4 for MCS13/52PRB test. - FFS whether to define a new HD-FDD reference channel for MCS4/52PRB tests. - Note: R.PDSCH.1-1.1 HD-FDD in TS 38.101-3 configures 18 DM-RS REs. MediaTek proposes to configure 12 DM-RS REs (1+1) instead of 18 REs (1+1+1). Issue 2-2-3: FRC for eRedCap demodulation Way forward: - Reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth - FFS for other test cases.

For MCS4/52PRB tests in TS38.101-4, DMRS 1+1+1 is used for high Doppler, such as TDLB100-400. However, in 38.101-5, the DMRS is always configured as 12 for both NTN-TDLA100-200 and NTN-TDLC5-200 test cases, where the Doppler is not such high compared to TDLB100-400. Therefore, we propose to consider 12 DM-RS REs (1+1) for NTN RedCap and eRedCap UE requirements.

Proposal 1: Consider 12 DM-RS REs (1+1) for NTN RedCap and eRedCap UE requirements and introduce new FRCs for cases of MCS4/52PRB.

NTN RedCap 1Rx

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | Reference channel |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD |

NTN eRedCap 1Rx/2Rx UE without baseband bandwidth reduction capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | Reference channel |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | R.PDSCH.1-12.3 FDD R.PDSCH.1-2.4 HD-FDD |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | New FRC for FDD New FRC for HD-FDD |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | R.PDSCH.1-12.3 FDD R.PDSCH.1-2.4 HD-FDD |

NTN eRedCap 1Rx/2Rx UE with baseband bandwidth reduction capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | Reference channel |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | New FRC for FDD New FRC for HD-FDD |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD |

# Conclusion

In this contribution, we provide our view about reference channel for NTN RedCap 1Rx, NTN eRedCap 1Rx/2Rx UE without baseband bandwidth reduction capability and NTN eRedCap 1Rx/2Rx UE with baseband bandwidth reduction capability.

Proposal 1: Consider 12 DM-RS REs (1+1) for NTN RedCap and eRedCap UE requirements and introduce new FRCs for cases of MCS4/52PRB.

NTN RedCap 1Rx

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | Reference channel |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD |

NTN eRedCap 1Rx/2Rx UE without baseband bandwidth reduction capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | Reference channel |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | R.PDSCH.1-12.3 FDD R.PDSCH.1-2.4 HD-FDD |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | New FRC for FDD New FRC for HD-FDD |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | R.PDSCH.1-12.3 FDD R.PDSCH.1-2.4 HD-FDD |

NTN eRedCap 1Rx/2Rx UE with baseband bandwidth reduction capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | Reference channel |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | New FRC for FDD New FRC for HD-FDD |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | New FRC for FDD New FRC for HD-FDD |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD |

# Reference

- R4-2512644, Way Forward for [116][331] NR_NTN_Ph3_demod, Ericsson