3GPP TSG-RAN WG4 Meeting #116bis	R4-2513556

# Prague, Czech, 13th – 17th Oct., 2025

Source:	LG Electronics

Title:	Discussion on RRM performance requirements of SSB adaptation for Rel-19 NES

Agenda item:	6.15.2.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM performance requirements of SSB adaptation for Rel-19 network energy saving.

Discussion

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

RRM core requirement defined L3 and L1 measurement requirement for SSB adaptation. L3 measurement requirement has already been agreed to be defined. Likwise, L1-RSRP can be a one test case to verify SSB adaptation.

Proposal 1: RAN4 to define L1-RSRP for SSB adaptation test case

Following principle can be defined to design the SSB adaptation test case

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

Proposal 2: Following principle can be defined to design the SSB adaptation test case

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

Conclusion

In this contribution, we provide views on RRM performance requirement of SSB adaptation for Rel-19 NES

Proposal 1: RAN4 to define L1-RSRP for SSB adaptation test case

Proposal 2: Following principle can be defined to design the SSB adaptation test case

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

Reference

[1]