# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513566

# Prague, CZ, 13th – 17th Octobor, 2025

Source: 	vivo

Title: 	Discussion on HPUE co-existence study for NR_UE_RF_Ph5

Agenda Item:	7.1.3

Document for:	Approval

Introduction

In RAN#109, a new WID for UE RF enhancements for NR FR1 for Phase 5 was approved [1]. One objective is to introduce HPUE for single carrier operation, i.e. single carrier TDD to support 4Tx for CPE/FWA UE, PC1.5 with 2Tx for FDD bands for handheld and FWA UE. For single carrier TDD to support 4Tx for CPE/FWA, co-existence study is needed. In this contribution, we provide our initial views on PC1 HPUE co-existence study.

Discussion

The objective for single carrier TDD to support 4Tx for CPE/FWA is captured as follows:

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class

PC1 was introduced for band n14 for public safety service in Rel-17 [2]. It is generally acknowledged that PC1 UE for band n14 is not targeted for smartphone form factor. Then PC1 was introduced for some other bands n71, n77, n85, n100, n101, which was intended for FWA or vehicle mounted UE. NR ACLR requirements for PC1 band n14 is defined as 37dB, then extend PC1 ACLR to all bands [3]. Co-existence study was conducted in R18 LTE_NR_HPUE_FWVM_R18 and some simulation results were derived. ACLR 37 dB was suffice to NR PC1 with some room for relaxation.

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

Observation 1: In current NR specification, ACLR with 37 dB was first introduced for band n14 and extended to n71, n77, n85, n100, n101.

In theory, ACLR for 4Tx with 4*26 dBm is the same with 1Tx with 26 dBm since the leakage ratio remains constant when both transmit power and interference levels increase by 4 times. For PC2 26dBm, the ACLR is 31dB. If PC1 with 4*26 PA configurations reuse ACLR as 37dB, it means 6 dB stringent compared to PC2 ACLR and it may be hard to achieve for PA vendors.

Observation 2: If PC1 with 4*26 PA configurations reuse ACLR as 37dB, it means 6 dB stringent compared to PC2 ACLR and it may be hard to achieve for PA vendors.

Anyway, we think co-existence study is needed since the maximum output power is up to 32dBm, which is 1dB higher than the maximum nominal power for PC1. However, In Rel-18 LTE_NR_HPUE_FWVM_R18, there were some simulation studied conducted for band n41, n77 [4]. The simulation methodology and procedure can still be used in this Rel-20 HPUE co-existence study. Some simulation assumptions need to be revisited, the maximum power for the interfering UE can be up to 32dBm and the channel bandwidth for band n41 can be 100MHz. We can use following simulation parameters as shown in Table 1 for band n41 with 32 dBm HPUE.

Table 1. Simulation parameters for band n41 system with 32 dBm UE

(many parameters can refer to TR 37.829)

|  | Base Station | HPUE |
| --- | --- | --- |
| Carrier frequency | - 2600 MHz |  |
| Channel bandwidth | - 100 MHz |  |
| Inter-site distance | - Use Table A.1.3-1 |  |
| Cell layout | - Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | - 1x3x1 |  |
| Lognormal fading | - 10 dB |  |
| - Shadowing correlation | - Between cells: 0.5, between sites: 1.0 |  |
| - MCL (including antenna gain) | - 70 dB (urban and suburban areas) - 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | - 17 dBi, = 65 degrees, Am = 20 dB | - Omni-directional antenna with -3.5 dBi. |
| Noise figure | - 5 dB | - 9 dB |
| Transmit power | - 46 dBm | - 32 dBm |
| Antenna height | - 45 m | - 1.5 m |
| ACLR | - 45 dB | - Use Table 5.2 in TR 36.942 - ACLR1: 30+X, ACLR2: 43+X - Where X is 1 dB |
| ACS | - 45 dB | - 27 dB (20 MHz), 33 dB (10 MHz) |

From some simulations results for PC1 in band n41, as shown in Table 2, it can be observed that ACLR relaxation can be possible for PC1. For ACLR with 30+X dB, the throughput loss would not exceed 5% when X is 0 and 7 for many cases. For 4*26dBm to achieve maximum 32dBm power, we think it is also possible to relax ACLR thus reducing the stress of implementation complexity.

Table 2. Simulation results for band n41 with 0.75km inter-site distance in TR37.829

| ACLR offset X [dB] | 0 | 7 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 2.99% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 3.02% |  |
| Average throughput loss (31 dBm interfering UE) | 3.35% | 1.68% |
| 5%-tile throughput loss (31 dBm interfering UE) | 4.30% | 1.48% |

Observation 2: ACLR relaxation can be possible for PC1 for FWA UE according to some simulation results conducted in Rel-18 WI LTE_NR_HPUE_FWVM_R18.

Proposal 1: The co-existence simulation assumptions and procedures for PC1 in Rel-18 WI LTE_NR_HPUE_FWVM_R18 can be reused as a starting point with some exceptions:

- The interfering UE power is 32 dBm

- The channel bandwidth

Conclusion

This contribution discusses PC1 HPUE co-existence study for NR_UE_RF_Ph5. The following observations and proposals are made:

Observation 1: In current NR specification, ACLR with 37 dB was first introduced for band n14 and extended to n71, n77, n85, n100, n101.

Observation 2: If PC1 with 4*26 PA configurations reuse ACLR as 37dB, it means 6 dB stringent compared to PC2 ACLR and it may be hard to achieve for PA vendors.

Observation 3: ACLR can be relaxed for PC1 for non-handheld UE according to some simulation results conducted in Rel-18 WI LTE_NR_HPUE_FWVM_R18.

Proposal 1: The co-existence simulation assumption and procedures in Rel-18 WI LTE_NR_HPUE_FWVM_R18 can be reused as a starting point with some exceptions:

- The interfering UE power is 32 dBm

- The channel bandwidth

- References

[1] RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109.

[2] RP-210190, PC1 and PC3 Updates for Band n14, AT&T, RAN#91.

[3] R4-2206455, CR to TS 38.101-1 on PC1 MPR table, Nokia, Huawei, RAN4#102-e.

[4] TR 37.829, High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands for Rel-18.