3GPP TSG-RAN WG4 Meeting#116bis 	R4-2513577

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussions on LP-WUS UE RF

Agenda Item:	4.22.1

Document for:	Approval

- Introduction

In RAN#109 meeting, the core part WI of Rel-19 LP-WUS WI has been closed [1]. In this contribution, we discuss some improvement issues for LP-WUS UE RF.

- Discussion

  - Capability reporting for LP-WUS types

Last RAN4 meeting, there were some discussions on UE capability reporting for LP-WUR type-1/type-2 and ACS guard RB, but no consensus reached in the end. There were some discussion points as follows:

- Whether the type-1 is mapped to OOK only, and type-2 mapped to ODFM only and both OOK/OFDM?

- Whether there is a need to specify the UE capability for LP-WUR Type-1 and Type-2?

- Whether the ACS guard RB for type-1 should be reported to network?

We share the following views for each of them:

- Regarding the first question, whether there should be mapping of waveform and type-1 or type-2, the answer is quite clear: there was decision in RAN4 that no mapping is needed.

- Regarding the second question, in our understanding there is no need to specify the UE capability for network to report LR type-1 or type-2. The reason is as following.

RAN4 RF define two LP-WUR types (Type 1 and Type 2), agreed no need 1-to-1 mapping between waveform and LP-WUR type, namely, for Type 1 and Type2, UE are not limited to specific LP-WUS waveform. Besides, a UE can only support either Type-1 LR or Type-2 LR, but not both.

- SNR:

Based on link-level simulation, RAN4 agreed a single SNR value (i.e., -4.5dB) for both envelop-detection and OFDM-based receiver. Also applied to both Type-1 and Type-2 to derive REFSENS requirements.

- NF:

For the FR1 requirements targeting at bands <2.5GHz

  - The IM+NF values are

      - Set 1: 18dB

      - Set 2: 13.5dB

Send an LS to RAN1 with RAN4 decision of two sets of: NF (assume IM is 2.5dB)

- REFSENS requirements

All supported band with simplified REFSENS requirements: based on agreed REFSENS equation and absolute NF+IM value, i.e., Type 1 receiver −97.5 dBm at 15 kHz SCS and −94.5 dBm at 30 kHz SCS; Type 2 receiver −102.0 dBm at 15 kHz and −99.0 dBm at 30 kHz. Even the type-1 (worse LR) sensitivity is close to MR requirements, no big REFSENS degradation is identified for all supported bands.

Furthermore, from RRM perspective, RAN4 RRM defines two core requirements for LP-SS based or SSB based RRM, where accuracy and side conditions are same for the two core requirements, but with different delay requirements, and side condition is set to -3 dB which corresponds to large NF case, i.e.,	NF of LR = NF of MR+ 8dB, which is type-1 LR assumption. It means that the condition is easily satisfied by LP-WUR with smaller NF (i.e., type 2), thus, no need to further split the requirements for type 1 and type 2 LP-WUR.

Overall, we think specify UE capability for LP-WUR Type-1 and Type-2 is not needed.

- Regarding the 3rd question, the ACS for type-1 specify two cases, i.e., relaxation of Z = 0.0dB with 7 guard RBs for 15 kHz SCS or 2 guard RBs for 30 kHz SCS, and Z = - 5 with 0 guard RB. It was also agreed that the LR should pass both two conditions, with and without sufficient guard RBs. If there is no sufficient guard RBs for configuration, e.g, small CBW, the network should assume worse ACS performance case. Therefore, no additional capability reporting is required.

In summary, regarding additional UE capability for LR type-1 and type-2, we think it is not necessary.

Proposal 1: No need to specify UE capability for LP-WUR type-1 and type2.

  - FR2 CR editorial improvement

Last RAN4 meeting, the FR2 CR [2] was agreed in RAN4, unfortunately, it is further changed to “endorsed” by MCC after RAN4 meeting in 3GU due to the CR cover page issue. Therefore, the FR2 LP-WUS is not reflected in the latest TS 38.101-2 v19.2.0. Based on MCC guidance, a formal CR with correct cover-page should be resubmitted in RAN4#117 meeting for approval, and the content will be merged into TS 38.101-2 v19.3.0.

The CR was approved in a rush time on Friday last meeting, so few small issues were not updated due to limited time. There is a chance in RAN4 this meeting to further improve the structure of FR2 CR (assume no technical update) before implemented into spec.

For example, we suggest at least the following updates:

- Use the same suffix M as FR1, i.e., LP-WUS/WUR

- For each clause title, use the same wording as FR1: xx for LP-WUS/WUR

- For ACS and ASCS clause, use the same structure as FR1, i.e., ACS and ASCS are in the same clause

More details of CR improvement for FR2 can be further discussed during the meeting.

Proposal 2: Update FR2 LP-WUS feature CR to align as much as possible with FR1 spec structure (assume no technical updates).

  - LS to RAN5 on LP-WUS testability

Agreed last RAN4 meeting, an LS to RAN5 on LP-WUS testability is needed [3].

Issue 4-1-1 test parameters for Side condition of Rx requirements

Agreement:

The following setting will be used for Rx requirements related testing. RAN4 will inform it to RAN5 next meeting via LS.

    - LP-WUS, LPSS and SSB all share the same EPRE.

    - LPSS shares the same M and same length as LP-WUS

    - LPSS shares the same length in OFDM symbols as LP-WUS

    - LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition

    - SSB is transmitted at 20 ms periodicity

Issue 4-1-2: Whether all NR CBW requirements should be tested

Agreement:

  - RAN5 can decide.

Issue 4-1-3: Whether both OOK and OFDM should be tested, if UE support both

Agreement:

  - Make suggestion to RAN5: For the case when LR supporting both OOK and OFDM waveforms, only test with OFDM waveform.

Issue 4-1-3: Whether spherical coverage can be skipped, if UE pass REFSENS

Agreement:

  - FFS whether need to verify to confirm the spherical coverage performance

Issue 4-1-7: RAN4 Input on Test mode

Agreement:

  - Leave test mode decision to RAN5.

Issue 4-1-8: Testing time reduction consideration for LP-WUS

Agreement:

  - Make suggestion to RAN5: Testing time reduction of LP-WUS RF test cases can be considered.

Issue 4-1-9: LS to RAN5 on Testability

Agreements:

  - Send a LS to RAN5 on RF testability considerations to RAN5 next meeting.

We prepare the draft LS in [4] to provide more information to RAN5 for consideration when developing LP-WUS/WUR conformance test spec.

Proposal 3: Approve the LS to RAN5 on LP-WUS/WUR testability.

- Conclusion

For LP-WUS/WUR UE RF, we have the following proposals:

Proposal 1: No need to specify UE capability for LP-WUR type-1 and type2.

Proposal 2: Update FR2 LP-WUS feature CR to align as much as possible with FR1 spec structure (assume no technical updates).

Proposal 3: Approve the LS to RAN5 on LP-WUS/WUR testability.

- References

- RP-252854, Status Report for LP-WUS/WUR WI, vivo, NTT DOCOMO, RAN#109.

- R4-2511858, FR2 LP-WUS feature CR, QC, RAN4#116.

- R4-2511885, WF for [116][136] NR_LPWUS_UERF Testability, vivo, RAN4#116.

- R4-2513809, LS to RAN5 on LP-WUS/WUR testability, vivo, RAN4#116bis.