# 3GPP TSG-RAN WG4 Meeting #116bis	 	                           R4-2513584

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Apple

Title:	RRM core requirements and testing framework Beam Management

Agenda item:	6.11.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#114bis in the scope of Rel-19 WI NR_AIML_air ‎[1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#114bis meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Delay requirements for applicable functionality reporting

- Testing framework based beam management

Discussion

Measurement delay requirements

We are discussing if legacy 5G NR L1-RSRP measurement-delay requirements (TS 38.133 §9.5.4.2) can or can’t be reused when AI/ML-based beam-management (BM) prediction is introduced in RAN4 work. In particular, we are focusing on the values M,N,P defined in legacy measurement delay requirements.

Firstly, let’s have a conceptual recap of the values M, N, P:

- M (temporal-freshness / averaging factor)
Counts how many consecutive CSI-RS periods (after we have already made measurements with every Rx-beam once) the UE must keep gathering fresh samples before it can hand an L1-RSRP value to higher layers. The spec sets M = 1 when a higher-layer timeRestrictionForChannelMeasurement is present or for aperiodic CSI-RS; otherwise M = 3 for periodic / semi-persistent CSI-RS.

- N (spatial-sweep factor)
With repetition = ON the same Tx beam-set is sent in several CSI-RS symbols/ SSB bursts; the UE measures its Rx beam each time.  If repetition = ON for periodic CSI-RS in that resource set N = ceil( maxNumberRxBeam / N_res_per_set ), where: maxNumberRxBeam is a UE capability / handled-beam upper bound signalled and N_res_per_set is the number of CSI-RS resources in the configured resource set (i.e., how many distinct spatial “shots” we already get per period).  In many FR2 RRM test configurations and lab discussions, UEs are assumed to support up to 8 Rx beams (a practical test-capability level reflected in UE capability signalling work and used in various RAN4 RRM setups). When: maxNumberRxBeam = 8, and the network configures one CSI-RS resource in the resource set (N_res_per_set = 1), and repetition = ON (meaning the UE must get enough CSI-RS opportunities to cover all beams), then the formula gives N = ceil(8 / 1) = 8. That’s the scenario that led to  “legacy N = 8”.  Therefore,  “N = 8” is a configuration-derived value, not a spec constant.

- P (availability-loss factor)
Inflates the measurement window when some CSI-RS occasions collide with SMTC windows or measurement gaps.  P = 1 when there is no overlap; it rises according to the overlap formulas in §9.5.4.2 when collisions occur.

N tells us how many CSI-RS periods/SSB bursts in total the UE must span to complete its Rx-beam sweep.

- If N = 1, the UE measures what it needs in the very next CSI-RS occasion and we pay only one period of delay.

- If N = 4, it means we wait through four CSI-RS periods before every Rx-beam has been measured once.

When the CSI-RS resource-set is configured with repetition = OFF, each CSI-RS resource may carry a different Tx beam, but the UE only measures one Rx-beam per resource instance so by definition it never needs to cycle through its other Rx-beams to cover spatial directions. In that case the spatial-sweep factor N in the 38.133 delay formula collapses to 1.

When repetition = ON, we re trading off how many CSI-RS periods the UE spends sweeping its Rx-beams (that’s N) against how many CSI-RS symbols we load into each period (that’s N₍res_per_set₎).  Here are the tradeoffs involved:

Pros of a smaller N

- Lower latency: Fewer full CSI-RS periods means the first complete Rx-beam sweep, and thus the multi-beam measurement vector arrives more quickly, shaving milliseconds off the end-to-end prediction.

- Reduced UE power draw: The RF front end and digital beam-steering logic only turns on for a handful of symbols before reporting, which conserves battery.

- Faster beam-failure recovery: Should we need to fall back to measured beams, having a small-N sweep means we can detect link degradation sooner.

Cons of a smaller N

- Higher DL overhead: To hit a small N we must pack more CSI-RS symbols per period (larger N₍res_per_set₎), which could consume more time-frequency resources on PDSCH or special CSI-RS slots.

- Scheduler pressure: Denser CSI-RS placements can collide with data or other measurements, forcing tighter resource-planning and potentially more PDCCH/DCI juggling.

Pros of a larger N

- Lower CSI-RS footprint: With fewer symbols per period, the network can serve more UEs or carry more data, since CSI-RS “steals” less air-time.

- Simpler scheduling: Sparsely spaced CSI-RS patterns could ease the burden on scheduling

Cons of a larger N

- Increased measurement delay: Every extra period we wait, it multiplies the sweep delay by T_CSI-RS ot T_SBB risking stale inputs to your AI/ML model.

- Higher UE energy cost: The RF and beam-steering logic keep running across many periods, drawing more power and potentially delaying sleep in DRX.

- Slower reaction to channel changes: In fast-moving or deep-fade scenarios, a large N can let the serving beam degrade significantly before the next full measurement arrives.

During RAN4#116 the following agreement was reached:

Agreement:

observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report)

For case 1 reuse M,N,P from legacy measurement requirements

For case 2, use T*M, N P  (M,N,P same as legacy measurement requirement)

FFS on T value, T can also be 1. T can also be based on capability

This observation period is to be used for the prediction delay requirement

BM-Case 2 (temporal-domain prediction, observation window)

In the legacy system, the UE measures reference signals such as SSBs or CSI-RS directly and computes the RSRP using standard filtering and averaging methods. The reporting delay is therefore only due to measurement and basic signal processing. This delay is already well defined in the specifications through fixed parameters like Z3 and Z3’, which describe how much time must pass between receiving a trigger or reference signal and sending the report. Because the process is straightforward and deterministic, these timing values are uniform across UEs and guarantee interoperability.

RAN1 has already agreed to extend legacy timing parameters (Z3 and Z3’) by introducing d and d’ to capture AI/ML inference delay.These extensions ensure that additional processing time required for spatial and spatio-temporal beam prediction is reflected in the reporting timeline.RAN4 shall therefore align its specification of prediction delay with RAN1’s d’ to maintain consistency across standards.

In the AI/ML-based system, the UE still starts with the same reference signal measurements, but instead of reporting them directly, it must first run them through an AI/ML model that predicts the best beams or their RSRP values. This step introduces an additional inference delay, which depends on the model’s complexity and the UE’s processing capability. As a result, the reporting timeline must be extended beyond the legacy Z3 and Z3’ by a new parameter (d or d’). The main difference, therefore, is that legacy reporting is purely measurement-driven with fixed timing, while AI/ML reporting adds an inference stage, requiring extra time to ensure the prediction can be completed before the report is sent.

- RAN4 shall update the definition of beam prediction reporting timelines to explicitly include the inference delay introduced by AI/ML models. This delay shall be aligned with the parameter d’ defined in RAN1, which extends the legacy Z3’ timing between reference signals and L1-RSRP reporting. By referencing d’ directly, RAN4 ensures consistency with RAN1 specifications, avoids duplicate definitions, and provides a clear basis for conformance testing

Known Condition for TCI state

The known condition for TCI state is defined based on the period between TCI state switch and the last transmission of RS or RS QCLed with target TCI state. If the target TCI state is a predicted beam, and the RS was not transmitted to, measured and reported by UE, then we need to introduce a new definition for known condition. We also need to differentiate between UE side prediction and NW side prediction. For the case when target TCI state is a RS based on beam prediction -

UE side model: The UE predicts and reports the top K beams based on measurements on set B. Since the UE is predicting the beam(s) reported, it is known to the UE if a TCI state is received within a certain period. The known condition can be determined based on last transmission of set-B.

NW side model: The UE reports the measurements for Set B and target TCI state is based on NW side prediction, the TCI state is unknown to the UE. In case the RS of target TCI state is transmitted to the UE, measured and reported prior to the TCI state switch, then it is known to the UE.

The following agreement was reach during RAN4#116

Agreement:

  - On Detectability and SNR conditions:

    - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command

    - The TCI state remains detectable during the TCI state switching period

    - The SSB associated with the TCI state remain detectable during the TCI switching period

      - SNR of the TCI state ≥ -3 dB

  - Time conditions:

    - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement

  - Agreement on QCL relationship

    - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

    - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

Before a gNB can command a TCI state change to a predicted transmit beam in AI/ML-based beam management, several technical conditions must be satisfied. First, the UE must have sent at least one L1-RSRP inference report for the target TCI state. This report provides the predicted RSRP of that beam, confirming that the UE’s AI/ML model has generated a concrete prediction for the intended downlink direction.

In addition to a prediction, the SSB associated with the target TCI state must remain physically detectable during the switching period, with a measured SNR of at least –3 dB. The UE can still find and decode the PSS/SSS and PBCH of that SSB, proving the beam is present and usable. This is a coarse presence check, not a full RSRP measurement: it verifies that the signal exists and meets a minimum quality level without requiring a calibrated dBm-level RSRP reading.
These combined requirements: one inference report and sustained SSB detectability with adequate SNR, ensure that a predicted beam is both logically valid and physically sound before the network switches to it. They safeguard against acting on purely model-generated data, and give the gNB confidence that the target TCI state can immediately support downlink transmission once activated.

Time Conditions for TCI Switch
RAN4#116 specifies that a TCI switch command must be issued within 1.28 s (1280 ms) of the last transmission of the reference-signal (RS) resource that the UE used for beam-prediction reporting or measurement.This requirement effectively limits how long a network can wait before acting on an AI/ML beam prediction.

The purpose of the 1280 ms bound is to preserve the freshness of the prediction. Channel conditions, especially in mobile scenarios, can change rapidly; if too much time elapses between the UE’s last supporting measurement and the TCI switch command, the predicted RSRP and beam direction may no longer represent the actual channel. By mandating a maximum delay, it can be ensured the target TCI state is still valid and detectable when the switch occurs.

The timer starts from the final RS resource used for the prediction (for example, the last SSB or CSI-RS sample that fed the AI/ML inference). The gNB must therefore schedule the switch command, taking into account signaling, processing, and any buffering, so that it reaches the UE within this 1280 ms window, guaranteeing that the beam selection remains accurate and robust.

When the Rx Beam Is Known

RAN4#116 clarifies that the receive (Rx) beam is considered known when the predicted transmit (Tx) beam in Set A has a Type-D Quasi-Co-Location (QCL) relationship to any Tx beam whose characteristics (e.g., timing, delay, angle) have already been measured and reported. Because a Type-D QCL implies that the associated downlink signals share essentially the same spatial path, the network can safely infer the correct Rx beam from the known measured beam. In this case, no additional UE-side estimation is needed; the Rx direction is automatically determined from existing measurements.

When the Rx Beam Is Not Automatically Known
If the predicted Tx beam does not have a QCL Type-D link to any measured Tx beam, then the Rx beam cannot be assumed known by default. In this situation, whether the Rx beam can still be identified depends on UE capability. A UE that supports Rx-beam prediction or pairing may signal that it can provide the corresponding Rx beam, but this is optional and not mandated for the current work item.

When a QCL Type-D relationship exists, Rx-beam knowledge is guaranteed and can be treated as a standard requirement. Otherwise, Rx-beam determination is left to implementation-specific or future-study mechanisms.

We would like to consider allowing an Rx beam to be considered known when a predicted Tx beam is QCL Type-D to another predicted Tx beam whose Rx beam is already validated. A Type-D QCL relationship means the two Tx beams share essentially the same spatial path and delay characteristics, so the receive direction that works for one beam is equally valid for the other. If the reference predicted beam has already satisfied the existing safeguards, at least one inference report, SSB detectability, and SNR ≥ –3 dB, its Rx beam is effectively “known” in the same way as a measured one. Extending the rule to predicted-to-predicted QCL therefore supports hierarchical AI/ML beam prediction without undermining reliability.

LCM and performance monitoring related requirements

During RAN4#116 the following agreements were discussed:

Issue 2-4: Activation delay

Agreement:

- UE has to be ready to start measurements for inference after sending RRC reconfiguration complete

- Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay

- This applies to all reporting schemes

Issue 2-2: Prediction report delay

Agreement:

  - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay

    - The reporting delay includes the inference delay

  - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

Activation delay: This is the one-time interval from when the UE sends RRC Reconfiguration Complete until it is ready to start the first measurements for inference. It is basically a readiness check and does not itself include model loading or inference execution.

Inference delay: This is the time the UE takes to run the AI/ML model and produce a prediction once measurement data are available.
Activation delay ends before inference delay begins, but any model-loading work that the UE chooses to perform at inference time rather than before activation is counted inside the inference delay. So activation delay defines the start of the measurement phase, while inference delay covers the model’s run-time processing within each prediction report.

Aperiodic Reporting Timing (RAN4#116 Agreement)
For aperiodic CSI prediction, the RAN4#116 agreement clarifies that the DCI trigger defines the measurement start but not the report slot. The DCI message sent by the gNB explicitly indicates which reference-signal resources the UE shall measure, thereby marking the start of the measurement delay and ensuring both UE and network know when and what is being measured. After those measurements, the UE runs its AI/ML inference and prepares the report. Because the DCI does not specify the exact transmission slot for the report, the UE sends its prediction as soon as measurement and inference are complete, using available uplink resources. The total prediction reporting delay therefore equals the measurement delay plus the reporting delay, with the reporting delay explicitly including the inference delay (model execution and related processing) This framework ensures that for aperiodic reporting the network knows precisely when measurement begins, while allowing flexible, readiness-based timing for the final report.

For periodic reporting, the gNB provides a fixed reporting periodicity and uplink resources directly in the RRC configuration. For semi-persistent (SP) reporting, the network provides the same information in RRC and later activates it with a MAC-CE command. In both cases, the reporting schedule is fully known to both UE and network once configuration (and MAC-CE activation, if needed) is complete.

However, when a UE finishes its measurements and AI/ML inference, the next scheduled uplink slot may not yet have started. To capture this gap, RAN4 specifies that the prediction reporting delay must include the time until the first available reporting occasion, the wait from processing completion to the earliest configured report slot.

Consequently, for SP and periodic schemes the prediction reporting delay is defined as the sum of three elements: measurement delay, inference delay, and the waiting time until the first scheduled reporting opportunity. This ensures the delay KPI reflects the true end-to-end latency from measurement start to the moment the first report is actually transmitted, even though the reporting times themselves are predetermined by RRC or MAC-CE configuration

    - Performance monitoring and LCM related requirements

The following agreement was reached in RAN4#115 meeting:

Agreement from RAN4#115: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |  |
| SP report | Not support | Support | Support |  |
| P report | Support | Support | Support |  |

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

RAN1 achieved following agreement for performance monitoring mechanism and metric:

RAN1#121 Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N)

- Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is log2(N+1)

RAN1 has introduced RS-PAI (Reference-Signal Prediction Accuracy Indicator) as a new report quantity for AI/ML beam management. RS-PAI counts the number of accurate prediction instances N_p over N valid monitoring instances and therefore requires the UE to accumulate multiple reference-signal transmissions, compute the metric, and then report it. This workflow differs from legacy beam-prediction accuracy metrics, which simply compared one measured prediction with one ground truth, and it brings new timing uncertainty: the network cannot easily know when the UE will finish collecting N samples, complete computation, and send the first RS-PAI report.

Without a standardized end-to-end delay requirement, the gNB cannot confidently schedule uplink resources, set timers for fallback, or verify conformance. Similarly, accuracy requirements must be defined so that RS-PAI truly reflects prediction quality and can be reproducibly tested, at least in static radio conditions, across different UE implementations. In short, RS-PAI introduces window accumulation + computation steps that legacy “measure once → report once” rules never covered, creating a gap in RAN4’s performance-monitoring framework.

We have the following proposals:

- RAN4 should define LCM requirements for UE-assisted monitoring where the UE reports the RS-PAI metric (monitoring Type 1 Option 2) in BM-Case 1 and BM-Case 2. The UE-reported N_p/N ratio must exceed a minimum threshold. The threshold can be scenario specific.

- RAN4 to define requirements for end-to-end RS-PAI reporting delay: start when the UE either (a) receives the configuration of monitoring RS resources or (b) sends RRC Reconfiguration Complete, and end when the first RS-PAI report is received at the gNB. This delay must bound the total time for window accumulation, UE processing, and uplink scheduling, giving the network a deterministic basis for resource planning and timely fallback.

- RAN4 Requirement on LCM Testing and Trigger Criteria Using RS-PAI

RAN4 shall include Life-Cycle Management (LCM) testing in its performance requirements for AI/ML beam-management models.

- Primary KPI: Use RS-PAI (N_p / N) as the single KPI to monitor live prediction accuracy during testing.

- Trigger thresholds: Define minimum RS-PAI ratios and observation windows (e.g., sustained below X % over Y consecutive reports) that, when breached, trigger an LCM action.

- LCM actions to test: Model switching, adapter activation/re-training, or deactivation with fallback to legacy beam management.

- Extended conditions: Include criteria for persistent instability (e.g., large RS-PAI variance) and deployment changes (e.g., new SSB/CSI-RS configuration) to test the ability to pre-emptively trigger LCM.

This proposal ensures that RAN4 tests not only accuracy but the full life-cycle response—verifying that RS-PAI based thresholds reliably activate switching, deactivation, or fallback in conformance testing and field operation.

- RAN4 Requirement on LCM Delay for Model Switching, Activation and Deactivation
RAN4 shall specify end-to-end delay requirements for LCM actions in AI/ML beam-management.

- LCM actions covered: model switching, new-model activation, and model deactivation with fallback to legacy operation.

- Start and end points: The delay measurement shall start when the LCM trigger condition (e.g., RS-PAI threshold breach or deployment-change event) is detected by the gNB or UE and shall end when the target model is fully active or legacy fallback is completed.

- Maximum delay bounds: RAN4 shall define maximum acceptable delays (e.g., X ms for deactivation/fallback, Y ms for model/functionality switching) to guarantee service continuity.

- Test method: LCM delay shall be validated in conformance testing under both static and dynamic radio conditions, including the time for signaling, UE processing, model load, and uplink scheduling.

Testing framework for AI/ML based Beam Management

The testability issues for AI/ML based BM use case were highlighted by multiple companies during previous few RAN4 meetings but the discussions continued without any concrete conclusion on the topic. Below are the agreements/summary of the progress on this topic during the past RAN4 meetings which are documented in the corresponding WFs.

Agreements from RAN4#110 meeting Issue 2-3: Test setup feasibility for FR2 Companies are invited to provide further analysis on what the test setup should enable in terms of test environment Agreements from RAN4#113 meeting 2.3.2 Agreements in main session: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Agreements from RAN4#114 meeting 2.4 Testing for one sided models Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Agreements from RAN4#114-bis meeting Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

The following agreements were reached in RAN4#116:

Issue 2-6: Test system setup

Agreement:

Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing)

Single AoA tests can also be performed in the multi AoA based RRM test setup

Further analyze what spatial channel models can be emulated with this test setup

  - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup

Single AoA tests are not precluded

Issue 2-7: Test system channel model

Agreement:

Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber.

Take CDL-C Umi as one example

Other channel models can be further discussed

The Rohde & Schwarz paper  proposes a multi-AoA over-the-air chamber (feIFF) with four fixed probes arranged around the DUT. Each probe is independently driven in amplitude and phase so that their combined far-field matches the 3GPP CDL-C channel for any selected gNB Tx beam in Set B. For every Tx beam, the controller computes probe weights so that the superposed field at the UE reproduces the cluster delays, powers and AoAs defined by the CDL model. The received RSRP at the UE is therefore a joint outcome of (i) the chosen Tx beam’s intended AoD, (ii) the CDL cluster powers and phases, and (iii) the probe geometry and weights that synthesize those arrivals.

In a multi-AoA OTA test chamber such as the feIFF system described by Rohde & Schwarz, the AoA (Angle of Arrival) values that matter are those specified by the CDL channel model, and the probes are the hardware used to reproduce those AoAs. Here is how they relate and what is actually emulated:

- CDL channel AoA (from 3GPP TR 38.901/38.827)
Each cluster in a CDL channel is defined by its mean AoA (and spread). These are the target arrival directions that a real UE would experience in the field for a given gNB Tx beam.

- Probe AoA (physical chamber geometry)
The chamber contains a finite number of probes (four in feIFF) placed at fixed azimuth angles around the UE. Each probe has a known physical direction, its own “probe AoA.”

- Relationship and emulation
To make the UE “see” the CDL-specified AoAs, the test controller computes complex weights (amplitude and phase) for each probe so that the superposition of their fields creates an effective wavefront arriving from the desired CDL AoA.

  - If a CDL AoA coincides with a probe’s physical direction, most power is sent through that probe.

  - If a CDL AoA lies between probes, several probes transmit simultaneously with calibrated weights so that their combined field angle equals the target AoA.

- Which angles are truly emulated
The CDL AoAs are the angles being emulated; the probes are simply the “actuators” used to synthesize those angles in space. The AoD (Angle of Departure) of the Tx beam is part of the logical CDL model and influences how cluster powers are assigned, but physically it is not emulated as a separate outgoing direction, only the arrival geometry at the UE is reproduced

The channel model governs how much of each Tx beam’s energy reaches the UE from each direction. Clusters with higher power or favorable angles contribute more strongly to the synthesized field. As the chamber steps through Tx beams in Set B, the UE measures RSRP per beam; differences in those readings directly reflect how the CDL-defined angular spreads and fades couple with each specific Tx beam pattern.

Because several probes radiate simultaneously and can represent multiple arrival directions at once, the UE is exposed to a spatially rich scene and can freely sweep or select its own Rx beams exactly as it would in the field. This dynamic is not possible in single-AoA setups, where the UE effectively “sees” only one direction at a time.

The paper stresses that the Rx-beam index used for each Tx-beam measurement should be provided as input to the AI/ML model. Even if the UE sweeps all Rx beams and reports only the best RSRP per Tx beam, the corresponding Rx-beam ID preserves the spatial context of each measurement. Including these indices allows the AI/ML model to learn the true Tx/Rx geometry and improves prediction of future Tx beams under mobility.

In R4-2511597, Rohde & Schwarz demonstrate that a model trained on the full 24-cluster CDL-C channel can be reliably evaluated on a 6-cluster simplified channel suited for the feIFF chamber: in their preliminary system-level simulation the ground truth and training used the full CDL model, while prediction/testing ran on the simplified model, yielding ≤ 1 % loss in top-1/top-3 beam prediction accuracy, which shows that training on the full channel generalizes well to simplified test conditions.

From Anritsu’s paper: the Anritsu paper stresses that a conformance test setup must cover both AI/ML beam-management use cases—BM-Case 1, which predicts the best Set A downlink beams from current Set B spatial measurements, and BM-Case 2, which predicts future Set A beams from past Set B measurements, and also allow mixed sequences of these two modes, since in real deployments a UE may dynamically shift between spatial and temporal prediction, or chain them in series, to maintain robust beam tracking and mobility performance

Anritsu emphasizes channel-model realism, noting that field performance of AI/ML beam management depends heavily on the propagation model. They caution that it is not yet proven whether a single simplified CDL channel can capture real-world effects and argue that multiple CDL-type channel models (e.g., CDL-A/B/C) may be required to represent the full range of angular spreads, cluster dynamics, and mobility variations encountered in practice.

Anritsu underscores the need for flexibility in test hardware, comparing single-Angle-of-Arrival (1AoA) setups with multi-AoA chambers. A 1AoA system places the UE in front of a single CATR or “wireless-cable” feed and drives it with a fading simulator capable of synthesizing numerous virtual clusters, Doppler shifts, and delay taps. This makes it possible to reproduce very detailed CDL or other channel models, including complex spatially consistent mobility, without being constrained by the number or positioning of physical probes. In short, 1AoA provides exceptionally rich channel modelling and is well suited for evaluating propagation effects and mobility patterns defined in standards.

However, 1AoA lacks realistic Rx-beam sweep behaviour. The signal effectively arrives from only one physical direction at a time, meaning every receive beam is illuminated almost identically. As a result, a UE performing beam sweeping sees no genuine spatial diversity, and its beam-selection timing and decision processes cannot be evaluated accurately. In contrast, a multi-AoA chamber uses several fixed probes transmitting simultaneously at different azimuths, so the UE experiences true multi-directional arrivals and can sweep or select Rx beams naturally, but such setups cannot perfectly emulate spatially consistent mobility and are harder to adapt when multiple CDL channel models must be applied

Instead they argue for keeping both 1AoA and multi-AoA options open, because each has complementary strengths and limitations:

- 1AoA – flexible for many channel models and spatially consistent mobility but limited for UE Rx-beam sweeping.

- Multi-AoA – good for Rx-beam sweeping but limited for spatially consistent mobility and for supporting many different channel models.

Multi-AoA chambers such as eIFF or feIFF can reproduce rich instantaneous spatial conditions for Rx-beam sweeping, but they are not well suited for spatially consistent mobility as defined in 3GPP TR 38.901 §7.6.3.2. Their probes are fixed in position, so while signal amplitude and phase can be varied to mimic fading, the chamber cannot continuously shift cluster angles and delays as a UE moves. This means the smooth, geometry-driven evolution of multipath components that real mobility requires can only be approximated by power or phase changes, falling short of true spatial consistency and limiting accurate testing of temporal beam-prediction scenarios such as BM-Case 2.

Anritsu R4-2509134 highlights that how the UE’s receive-beam (Rx-beam) information is provided to the AI/ML model is still not harmonized, and that this choice directly shapes the required test setup. RAN1’s earlier agreements (referenced in the paper) list several options: (1) the UE sweeps all Rx beams for every measurement sample and reports only the best Rx beam as input to the AI model; (2) it provides measurements of specific Rx beams—either per sample (2a) or for the entire measurement set (2b); or (3) it uses random Rx beams per sample. Each option results in a different AI input structure, from a single best-beam RSRP to a richer set of {Tx beam, multiple Rx beams, RSRP} tuples, and therefore dictates how many measurements, probes, and reporting fields must be emulated in the test chamber.

Because the conformance procedure must reflect the actual inference path of the AI/ML model, RAN4 needs to reconfirm which option (or combination) is most representative of real UE behavior. For example, if the UE always provides only the best Rx beam, the test setup can focus on accurately emulating peak RSRP values; but if multiple or random Rx beams are included, the test must present genuine multi-directional arrivals and capture Rx-beam indices for each Tx-beam measurement. Anritsu therefore proposes that companies clarify and align the latest assumptions before finalizing the test environment, since the selected approach will drive measurement complexity, signaling requirements, and ultimately the validity of AI-beam prediction performance results

Anritsu proposes a two-step fallback method: If no single chamber setup can handle all requirements, allow a two-phase evaluation, first the AI model’s prediction accuracy without Rx-beam sweeping, then a separate assessment of UE Rx-beam sweeping behaviour.

We propose a two-phase testing approach: Phase 1 validates the AI/ML beam-prediction algorithm under controlled mobility without Rx-beam sweeping, and Phase 2 verifies full end-to-end performance with natural Rx-beam sweeping in a multi-AoA environment.

Phase 1 – Validate AI/ML prediction accuracy under controlled mobility (no Rx-beam sweeping).
This phase uses a single-AoA “wireless-cable” setup driven by a fading/channel emulator to generate rich CDL channel conditions and spatially consistent mobility. By fixing or emulating the receive direction, we isolate the algorithm itself, testing how well the AI/ML model predicts the best Set A beams for both BM-Case 1 (spatial) and BM-Case 2 (temporal) without any influence from UE beam-sweeping hardware. We need this phase to obtain a clean algorithmic baseline, so that if issues appear later in full multi-AoA testing we can pinpoint whether they stem from the model or from the UE’s beam-sweeping mechanics, and to allow early, lower-cost debugging before the more complex Phase 2. Run the procedure across CDL-A/B/C to capture angular/mobility diversity and ensure the AI/ML algorithm generalizes under spatially consistent mobility.  In a single-AoA “wireless-cable” rig the channel is generated entirely in the fading emulator, so we can easily and repeatably load many CDL families (A/B/C, different Dopplers, cluster spreads). Running those variations here verifies that the AI/ML model generalizes across diverse propagation environments and exposes any weaknesses before hardware-related effects come in.

Phase 2 – Verify full end-to-end performance with natural Rx-beam sweeping.
This phase is carried out in a multi-AoA chamber (e.g., feIFF) that presents true multi-directional arrivals so the UE can sweep and select Rx beams exactly as it would in the field. Using simplified CDL-C channel models tailored to the probe layout, the test measures how well the AI/ML model predicts the best Set A beams when real-time Rx-beam changes, hardware dynamics, and switching delays are all active. It also logs the Rx-beam index for each Tx-beam measurement, ensuring the AI input captures the actual {Tx,Rx} geometry. The goal is to confirm that the combined system—AI/ML algorithm plus UE beam-sweeping hardware and control logic—meets accuracy and responsiveness requirements under realistic spatial conditions. Phase 2 is focused on real Rx-beam sweeping in a multi-AoA chamber, where every new CDL family would require complex probe-weight recalibration or extra probes. That would add significant cost and time while giving little new information about sweeping behaviour.

So the variety of CDL channels is concentrated in Phase 1 for algorithm coverage, while Phase 2 keeps to the single simplified CDL-C that best matches the chamber geometry, ensuring a practical and reproducible end-to-end test.

We need these simulation approaches to determine feasibility and build confidence in the proposed two-phase testing. We will need to test all key train/test combinations—full→full, simplified→simplified, full→simplified (already proven by R&S with ≤1 pp accuracy loss), and simplified→full to verify that AI/ML models trained on one channel type generalize to the other. Cover both BM-Case 1 and BM-Case 2 and include the three agreed AI input modes (best-Rx, specific-Rx per sample, fixed-Rx subset). Use a focused set of metrics: Top-1/Top-3 beam-prediction accuracy. Next, RAN4 should (i) reconfirm which AI input modes to standardize, (ii) adopt the two-phase test method, and (iii) run a joint multi-vendor simulation round to close the full↔simplified gaps and set final pass/fail tolerances

- Two-Phase RAN4 Conformance Test for AI/ML Beam Management:  RAN4 should adopt a two-phase test to validate AI/ML beam-prediction accuracy and full UE behaviour for BM-Case 1 (spatial) and BM-Case 2 (temporal).

- Phase 1 – Algorithmic validation (single-AoA “wireless-cable” setup)

  - Fading emulator with full CDL-A/B/C channels for rich, spatially consistent mobility.

  - Tests algorithm only, independent of UE beam-sweeping hardware.

  - Verifies train/test channel combinations (full↔full, simplified↔simplified, full↔simplified) and all AI input modes.

- Phase 2 – End-to-end validation (multi-AoA chamber, e.g. feIFF)

  - Simplified CDL-C channel matched to probe layout.

  - Provides true multi-directional arrivals for natural Rx-beam sweeping.

  - Confirms that the complete system, AI/ML model plus UE hardware, meets accuracy and responsiveness targets.

Conclusion

In the paper, the following Observations and Proposals were made:

- RAN4 shall update the definition of beam prediction reporting timelines to explicitly include the inference delay introduced by AI/ML models. This delay shall be aligned with the parameter d’ defined in RAN1, which extends the legacy Z3’ timing between reference signals and L1-RSRP reporting. By referencing d’ directly, RAN4 ensures consistency with RAN1 specifications, avoids duplicate definitions, and provides a clear basis for conformance testing

- RAN4 should define LCM requirements for UE-assisted monitoring where the UE reports the RS-PAI metric (monitoring Type 1 Option 2) in BM-Case 1 and BM-Case 2. The UE-reported N_p/N ratio must exceed a minimum threshold.

- RAN4 to define requirements for end-to-end RS-PAI reporting delay: start when the UE either (a) receives the configuration of monitoring RS resources or (b) sends RRC Reconfiguration Complete, and end when the first RS-PAI report is received at the gNB. This delay must bound the total time for window accumulation, UE processing, and uplink scheduling, giving the network a deterministic basis for resource planning and timely fallback.

- RAN4 Requirement on LCM Testing and Trigger Criteria Using RS-PAI

RAN4 shall include Life-Cycle Management (LCM) testing in its performance requirements for AI/ML beam-management models.

- Primary KPI: Use RS-PAI (N_p / N) as the single KPI to monitor live prediction accuracy during testing.

- Trigger thresholds: Define minimum RS-PAI ratios and observation windows (e.g., sustained below X % over Y consecutive reports) that, when breached, trigger an LCM action.

- LCM actions to test: Model switching, adapter activation/re-training, or deactivation with fallback to legacy beam management.

- Extended conditions: Include criteria for persistent instability (e.g., large RS-PAI variance) and deployment changes (e.g., new SSB/CSI-RS configuration) to test the ability to pre-emptively trigger LCM.

This proposal ensures that RAN4 tests not only accuracy but the full life-cycle response—verifying that RS-PAI based thresholds reliably activate switching, deactivation, or fallback in conformance testing and field operation.

- RAN4 Requirement on LCM Delay for Model Switching, Activation and Deactivation
RAN4 shall specify end-to-end delay requirements for LCM actions in AI/ML beam-management.

- LCM actions covered: model switching, new-model activation, and model deactivation with fallback to legacy operation.

- Start and end points: The delay measurement shall start when the LCM trigger condition (e.g., RS-PAI threshold breach or deployment-change event) is detected by the gNB or UE and shall end when the target model is fully active or legacy fallback is completed.

- Maximum delay bounds: RAN4 shall define maximum acceptable delays (e.g., X ms for deactivation/fallback, Y ms for model/functionality switching) to guarantee service continuity.

- Test method: LCM delay shall be validated in conformance testing under both static and dynamic radio conditions, including the time for signaling, UE processing, model load, and uplink scheduling.

- Two-Phase RAN4 Conformance Test for AI/ML Beam Management:  RAN4 should adopt a two-phase test to validate AI/ML beam-prediction accuracy and full UE behaviour for BM-Case 1 (spatial) and BM-Case 2 (temporal).

- Phase 1 – Algorithmic validation (single-AoA “wireless-cable” setup)

  - Fading emulator with full CDL-A/B/C channels for rich, spatially consistent mobility.

  - Tests algorithm only, independent of UE beam-sweeping hardware.

  - Verifies train/test channel combinations (full↔full, simplified↔simplified, full↔simplified) and all AI input modes.

- Phase 2 – End-to-end validation (multi-AoA chamber, e.g. feIFF)

  - Simplified CDL-C channel matched to probe layout.

  - Provides true multi-directional arrivals for natural Rx-beam sweeping.

  - Confirms that the complete system, AI/ML model plus UE hardware, meets accuracy and responsiveness targets.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.

- R4-2511597, “Discussion on test setup for AI/ML-based beam management”, Rohde & Schwarz, RAN4 #116 meeting, 25th – 29th August 2025, Bengaluru, India.

- R4-2509134, “FR2 test setup for AI/ML beam management evaluation”, Anritsu Corporation, RAN4 #116 meeting, 25th – 29th August 2025, Bengaluru, India.