# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513588

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.9.2

Source:	Apple

Title:	Discussion on General Aspects of AI/ML for NR Air Interface Phase 2

Document for:	Discussion

# 1	Introduction

RAN#108 has approved a new WI on AI/ML for NR air interface enhancements in Rel-20 [2], where the objective includes CSI feedback enhancement with two-sided model and inter-vendor training collaboration for two-sided model.

# 2         Inter-vendor training collaboration and interoperability for two-sided AI/ML

In RAN Meeting #109 a revised WID was captured in RP-252445 with the following objectives:

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

- NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

AI/ML-based CSI compression needs a single, trusted reference point to guarantee inter-vendor interoperability and consistent minimum performance. By studying a fully specified test decoder together with a standardized reference encoder, RAN4 can provide this anchor for laboratory conformance testing (Option 3), for field deployments (Direction C), and for inter-vendor training collaboration under Direction A 3a-1. Such a unified model specification would let every vendor benchmark and validate their encoders against the same known baseline, ensuring reproducibility and enabling inter-vendor training collaboration through secure parameter exchange and joint model development.

Combining test and deployment requirements in one standardized decoder/encoder pair avoids duplication and simplifies 3GPP specifications. The same model can serve as the official conformance object for Option 3 and as the field reference side for Direction C, while the standardized encoder structure alone supports inter-vendor parameter exchange for Direction A. This dual use helps vendors and operators maintain a single interface and accelerates adoption by lowering testing and integration costs.

Even with a stable reference model, long-term model health must be monitored and controlled. RAN4 should therefore examine lifecycle management (LCM) requirements, covering activation, switching, and fallback so that AI/ML CSI-compression models remain reliable as radio conditions evolve. The focus is on identifying which KPIs best indicate model fidelity and network impact, and on defining high-level principles for criteria to trigger LCM actions, including delay requirements for switching or activating and observation windows on KPIs to avoid unnecessary model toggling.

LCM requirements must address timing and monitoring frequency so that model changes, such as switching, activation, or fallback, do not occur too frequently or abruptly. By specifying observation and hold times, along with test methods and validation procedures, RAN4 can ensure that model changes are deliberate, measurable, and interoperable, helping maintain overall network performance.

- Workplan: Standardized/reference models and Lifecycle Management for AI/ML-based CSI Compression

RAN4 is invited to develop a workplan to ensure inter-vendor interoperability, field reliability, and long-term stability of AI/ML-based CSI compression. The workplan combines standardized reference modeling with LCM requirements as follows:

- Unified reference model – Specify a fully defined test decoder with a standardized reference encoder serving as:
– the Option 3 conformance benchmark for laboratory testing,
– the Direction C standardized reference side for field deployment, and
– the structural anchor for Direction A 3a-1 inter-vendor training collaboration through a common encoder architecture and parameter format.

- Lifecycle management requirements – Define high-level requirements for activation, switching, and fallback so deployed models stay robust as conditions evolve.
– Establish LCM triggering rules by identifying key KPIs and use filtered or historical KPI criteria to trigger LCM  but avoid unnecessary toggling.
– Set minimum observation and hold times and appropriate monitoring frequency, plus test methods and validation, to ensure each model change is deliberate, measurable, and interoperable, while RRM functions remain stable.

# Conclusion

- The following proposal outline a RAN4 workplan to secure interoperability, field reliability, and controlled lifecycle management for AI/ML-based CSI compression.

- Workplan: Standardized/reference models and Lifecycle Management for AI/ML-based CSI Compression

RAN4 is invited to develop a workplan to ensure inter-vendor interoperability, field reliability, and long-term stability of AI/ML-based CSI compression. The workplan combines standardized reference modeling with LCM requirements as follows:

- Unified reference model – Specify a fully defined test decoder with a standardized reference encoder serving as:
– the Option 3 conformance benchmark for laboratory testing,
– the Direction C standardized reference side for field deployment, and
– the structural anchor for Direction A 3a-1 inter-vendor training collaboration through a common encoder architecture and parameter format.

- Lifecycle management requirements – Define high-level requirements for activation, switching, and fallback so deployed models stay robust as conditions evolve.
– Establish LCM triggering rules by identifying key KPIs and use filtered or historical KPI criteria to trigger LCM  but avoid unnecessary toggling.
– Set minimum observation and hold times and appropriate monitoring frequency, plus test methods and validation, to ensure each model change is deliberate, measurable, and interoperable, while RRM functions remain stable.

# References

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm.

- RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2