# 3GPP TSG-RAN WG4 Meeting #116bis		R4-2513593

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Apple

Title:	On RAN4 Impacts for Measurement Event Prediction in AIML Mobility

Agenda item:	7.12.4

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis within the scope of Rel-19 SI ‎[1]‎ and ‎[2], with a dedicated agenda item assigned to measurement Event prediction in AI/ML Mobility.

In this paper, we provide our views on identified measurement event prediction related issues which need further discussions.

Discussion

Requirements Case A vs Case B

Following agreement was reached in RAN4#114bis During RAN4#114 bis the following agreements were reached:

Issue 3-1-1: Prioritization of cases for event prediction

Agreement:

- RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization

Issue 3-1-2: Performance metric for event prediction use case

Agreement:

- Potential RRM requirement impact for event prediction use case - for indirect case:

- RAN4 to study the requirement for the predicted event triggered reporting

  - RAN4 to study the requirement for [delay and accuracy]

  - RAN4 to discuss the metric for the event prediction

- Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time],

  - Candidate options:

    - Option 1: RAN4 will define requirement for the predicted RSRPs

    - Option 2: RAN4 will not define requirement for the predicted RSRPs

In the last RAN2 meeting (RAN2 #128 [1]), the simulation framework, assumptions, and parameters for Measurement Event Prediction for handover events were discussed. Among the agreements captured in the Chair notes [1], definitions were agreed for 4 indirect measurement event prediction cases:

- Indirect measurement event prediction for temporal domain case A: Future measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at a future time occurs, without further involvement of an AI/ML model

- Indirect measurement event prediction for temporal domain case B: Future measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at a future time occurs, without further involvement of an AI/ML model

- Indirect measurement event prediction for frequency domain: Measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at a time instance occurs, without further involvement of an AI/ML model​

- Indirect measurement event prediction for spatial domain: Measurement result(s) is predicted by an RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

Under the indirect measurement-event prediction approach for intra-frequency scenarios (temporal case A, temporal case B, or the frequency domain), the process shown in  Figure 1, applies. First, a Radio Resource Management (RRM) prediction model, possibly using AI or ML, estimates future measurement values. These predicted values, optionally combined with recent actual measurements from the same cell, are then assessed to determine whether a measurement event will occur at a specified future moment. The final decision step is fully deterministic and does not involve any further AI or ML processing.

Figure 1: Indirect measurement event prediction.

In Temporal Case A the UE continues to measure every period in FR2 but now reports a forecast that the A3 threshold will be crossed before it actually is. The network can exploit this early warning either to prepare the target cell while still waiting for a legacy A3 report (Option 2) or, more aggressively, to launch the handover as soon as the prediction is deemed trustworthy (Option 3). The timing margin of the prediction therefore has direct impact on handover-failure probability and interruption time: a prediction that is too late behaves like legacy, while a prediction that is too early may waste resources if it proves wrong. RAN4 must therefore decide what minimum lead time and what minimum prediction reliability are acceptable for mobility-performance enhancement in this case.

Figure 2 provides an example of time domain case-A RRM measurement prediction.

Figure 2: Time domain case A RRM measurement prediction

In both options, the decision for an early handover preparation may be done based on the predicted measurement event.

In RAN2#128, RAN2 discussed the assumption on how to improve HO KPI by measurement event prediction and the following two options are agreed as baseline HO operation for Case A prediction in FR2.

2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

Option 2: At the current time t₀, Event A3 is predicted to occur at a future time. Based on this prediction, the network (NW) can initiate handover (HO) preparation in advance. However, it will not transmit the HO command until the UE reports the actual measurement event for the same neighbouring cell. Once the UE reports the real Event A3 at t₂, the NW can immediately issue the HO command without incurring the typical preparation delay. The primary benefit of this option is the reduction of HO preparation time. Moreover, because the HO decision is still based on the actual measurement report from the UE,as in the legacy approach, there is minimal risk of degrading HO-related KPIs.

Option 3: At time t₀, the network predicts that Event A3 will occur in the future. Using this prediction, the NW begins HO preparation ahead of time, but it waits to send the HO command until the Event A3 entering condition is satisfied based on real-time measurement results. Once the entering condition is met and the predicted TTT (Time-To-Trigger) condition is satisfied for the same neighbouring cell, the UE can report Event A3 entering, prompting the NW to send the HO command immediately. This approach not only saves HO preparation time but may also enable an earlier handover compared to the legacy method. However, because the HO command may be triggered before the full TTT expiry, there is a potential risk of HO KPI degradation due to premature handovers.

In RAN2#129, RAN2 discussed the assumption on how to improve HO KPI by measurement event prediction and the following two options were agreed as shown below:

In time domain case B RRM measurement prediction, the number of required downlink radio measurements is reduced by the UE skipping specified measurements periods and predicting the skipped measurements from the past measurements using a ML model. Two examples for skipping patterns are shown in Figure 3.

Figure 3: Examples of time domain case B measurement prediction

In Figure 4, at the current time T1, the UE uses the Case A prediction model to predict future L3 RSRP values within the prediction window (PW, T1–T4) based on historical L3 RSRP measurements from the serving and neighboring cells during the observation window (OW, T0–T1). Subsequently, we evaluate whether an Event A3 report would be triggered by analyzing the predicted L3 RSRP values within the PW.

Figure 4. Indirect measurement event prediction

In the example provided, the Event A3 entering condition is predicted to be first satisfied at T2 and sustained throughout the time-to-trigger (TTT) period (T2–T3). Consequently, the output of the Event A3 prediction is labeled ‘predicted at T3’. If no Event A3 condition is predicted within the PW, the output defaults to ‘not predicted’.

In Temporal Case B the UE intentionally omits certain downlink measurements in FR1, reconstructing the missing RSRP samples with a model trained on historical data. The goal is to cut measurement overhead without degrading mobility. Because the prediction window is short and not every period is forecast, early-handover benefits are limited; instead the primary risk is that inaccurate predictions will cause either false A3 triggers (extra signalling) or missed A3 triggers (late handover). Consequently, the acceptable error margin in the reconstructed RSRP, and hence the derived event decision , is tighter than in Case A, yet the required lead time may be much smaller. These different emphases mean that a single metric cannot serve both cases.

RAN4 should define separate performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, reflecting their different objectives and risk profiles.

For Case A, where the UE predicts a future event (e.g., A3) while still performing all periodic measurements, the priority is enabling earlier and more reliable handover preparation. Requirements should therefore emphasize prediction lead time and accuracy metrics that minimize both late predictions (legacy-like performance) and premature predictions (wasted resources or degraded HO KPIs).

For Case B, where the UE skips certain measurements and reconstructs the missing RSRP values, the goal is to reduce measurement overhead without harming mobility. Here, requirements should focus on the accuracy of reconstructed RSRPs and the correctness of the derived event decision, with tighter error margins than Case A but smaller lead-time expectations. This separation ensures that each case is evaluated against metrics aligned to its intended mobility and efficiency benefits.

RAN4 should set similar performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, but with different emphasis. Case A should focus more on prediction lead time and reliability to enable earlier, safe handover preparation. Case B should focus more on RSRP reconstruction accuracy and correctness of the derived event decision to ensure measurement-overhead reduction without mobility degradation

Requirements and Performance metrics for indirect measurement event prediction with temporal domain

Based on TS 38.133, the UE shall send one Event A3-triggered measurement report within the defined measurement reporting delay. The UE shall not send event-triggered measurement reports if the reporting criteria are not met. The rate of correct events observed during repeated tests shall be at least 90%.

However, in RAN2, F1 score with the following details has been agreed as the evaluation metric for AI/ML measurement event prediction [TR 38.744].

The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

A key challenge in defining performance metrics for AI/ML-based A3 measurement event prediction is the lack of clarity in how the F1 score is computed when the prediction method is either periodic or event-based. In the periodic approach, predictions are inferred at every measurement sampling interval regardless of whether an A3 entry condition is close to being met. This can result in a large number of predictions being made under conditions where the event is far from occurring, skewing the counter values and thus the F1 score. Conversely, in the event-based approach, predictions are only inferred when a specific trigger, such as the A3 entry condition, is satisfied, leading to fewer but more targeted prediction instances. The difference in starting points and update frequencies between these two methods makes direct comparison and consistent scoring difficult.

A related ambiguity arises when there is more than one target cell candidate for the event. It is unclear whether the prediction accuracy counters (true, false, and missed events) should be computed for each target cell individually or whether the evaluation should consider only the case where the correct target cell is predicted. Evaluating each target cell individually treats all cells as independent samples, while including correct target-cell identification in the metric couples event detection accuracy with target-cell selection accuracy. This difference can significantly affect reported F1 scores and needs to be clarified for consistent testing and interpretation.

Assume max ETD = 50 ms and ground-truth A3 events at t = 200, 500, 800 ms (three GT events). We use one-to-one matching: each predicted event can match at most one GT event if ∣tpred−tGT∣≤50 ms.

Periodic predictor (predicts every 100 ms): predictions at 100, 200, 300, 400, 500, 600, 700, 800, 900 ms.
Matches: 200↔200, 500↔500, 800↔800 → n3 = 3 (true).
All other predictions (100, 300, 400, 600, 700, 900) are unmatched → n1 = 6 (false).
Every GT is matched → n2 = 0 (missed).
Precision =n3/(n1+n3)=3/(6+3)=1/3≈0.333=n3/(n1+n3)=3/(6+3)=1/3≈0.333.
Recall =n3/(n2+n3)=3/(0+3)=1.0=n3/(n2+n3)=3/(0+3)=1.0.
F1 =2PRP+R=2×0.333×10.333+1≈0.5=P+R2PR​=0.333+12×0.333×1​≈0.5.
→ Lots of “spam” predictions far from any GT event tank precision → lower F1.

Event-based predictor (fires only near likely A3): predictions at 195, 205, 505, 780 ms.
Greedy match: 205↔200, 505↔500, 780↔800 → n3 = 3.
The extra 195 ms doesn’t have a remaining GT to match → n1 = 1.
All GT matched → n2 = 0.
Precision =3/(1+3)=0.75=3/(1+3)=0.75. Recall =3/(0+3)=1.0=3/(0+3)=1.0.
F1 =2×0.75×10.75+1≈0.857=0.75+12×0.75×1​≈0.857.
→ Fewer, targeted predictions boost precision → higher F1, even with identical GT and ETD.

Two predictors with identical true detections can yield very different F1 scores solely because of their reporting style: a periodic predictor fires on every sampling instant, generating many unmatched predictions that inflate the false-positive counter n1, lower precision, and drive F1 down, whereas an event-based predictor reports only near likely A3 triggers, keeping n1 small and precision, and thus F1,much higher. This demonstrates that F1 intertwines true predictive ability with reporting frequency, so RAN4 conformance tests based only on F1 could rate equally accurate models very differently depending on whether they use periodic or event-based reporting

Even if the metric definition is resolved, practical testing with the F1 score is resource-intensive. Obtaining a statistically reliable F1 score requires large datasets with accurately labeled events across varied radio conditions, UE mobility patterns, and network deployments. This means extensive simulations and/or field trials to capture enough cases of true positives, false positives, and false negatives. The complexity of setting up such tests, combined with the need for repeated measurements under controlled conditions, creates a high overhead for both equipment vendors and test labs.

These challenges suggest that while the F1 score is conceptually useful for capturing the trade-off between precision and recall, it may not be the most practical or unambiguous metric for RAN4 conformance testing. The variability introduced by prediction mode (periodic vs event-based), multi-target scenarios, and the high cost of data collection calls for simpler, more deterministic metrics that can still ensure prediction quality without excessive testing burden.

In order to calculate F1 = 2 · (Precision · Recall)/(Precision + Recall) we need both precision and recall.

- Precision counts only the UE’s predictions, so the tester inspects a short window around each prediction.

- Recall, however, is the share of all real A3 events that were successfully predicted. To obtain that denominator the tester must first locate every ground-truth A3 trigger in the entire trace, even during long stretches where the UE made no predictions.

Formula:
Precision = True Positives / (True Positives + False Positives), Recall = True Positives / (True Positives + Missed Events)

For defining RAN4 requirements for indirect measurement-event prediction we propose the following:

Precision requirement
For indirect measurement-event prediction the UE shall achieve a windowed precision ≥ P_min within a defined lead-time window. Precision is calculated as the number of correctly predicted A3 events divided by the total number of predicted events, where a correct prediction is one that matches a ground-truth trigger inside the permitted Event-Time-Deviation and lead-time bounds. This single metric is both meaningful and efficient: it directly limits false positives (false alarms) that would otherwise waste resources on unnecessary handover preparation, and it can be tested by checking only the short time windows around each UE prediction, avoiding the full-trace scanning required for recall or F1. Precision is also sufficient for safety, because if a UE misses some true events the worst case is that the ordinary measurement process still triggers the A3 event and performs the handover. Such missed predictions merely reduce the early-handover benefit without risking dropped connections

Lead-time window requirement
Every accepted prediction shall arrive inside a lead-time window [L_min, L_max], meaning it is reported at least L_min before the true A3 trigger but no more than L_max ahead. The lower bound L_min guarantees that each prediction gives the gNB enough preparation time for resource reservation, beam setup, and signaling; predictions later than L_min bring no gain over legacy reporting. The upper bound L_max ensures predictions are not so early that they become stale or unreliable, avoiding premature handover preparation or resource holding. By enforcing this two-sided window, RAN4 ties the UE’s predictive reporting to a practical, actionable time frame, ensuring predictions are both timely and trustworthy for mobility optimization.

Figure 5. Lead-time window requirement

Predicted-RSRP accuracy requirement
If the UE includes predicted Layer-3 RSRP values together with the event flag, these predictions shall meet the same absolute and/or relative accuracy limits that apply to conventional event-triggered RSRP measurements. Accurate predicted RSRPs give the gNB confidence in both the magnitude and the slope of the expected signal change, allowing it to rank target cells, fine-tune handover timing, and monitor model health by comparing predicted and subsequent measured values. This requirement ensures that the added information in the report is as reliable as today’s measured RSRP, enabling richer network decisions without compromising interoperability.

RAN4 Key Performance Metrics for Indirect Measurement-Event Prediction

Windowed precision ≥ P_min – Precision is the ratio of correctly predicted A3 events to all predicted events, counting only matches within the allowed lead-time window. This directly limits false positives (false alarms) that would otherwise waste handover resources

Lead-time window [L_min, L_max] – Each accepted prediction must arrive at least L_min but no more than L_max before the actual A3 trigger. The lower bound ensures sufficient time for resource reservation and signalling; the upper bound prevents stale or premature predictions.

Predicted-RSRP accuracy – UE reports predicted Layer-3 RSRP values with the event. Those predictions shall meet the same absolute and/or relative accuracy limits defined for conventional A3 measurements, enabling correct target-cell ranking and handover timing.

These KPIs and performance requirements apply to both time-domain Case A and Case B, but individual target values may differ to match their distinct objectives: Case A emphasises early-warning mobility optimisation with stronger minimum lead-time and stricter precision, while Case B focuses on measurement-overhead reduction with shorter or zero minimum lead-time and tighter emphasis on RSRP reconstruction accuracy.

Measurement Reporting and Model Monitoring

When the UE performs indirect A3-event prediction it already computes a vector of future Layer-3 measurements (RSRP, optionally RSRQ/RS-SINR). If the UE reports only the binary outcome “A3 will trigger” the gNB loses visibility into these predicted values. Lacking this context the network cannot judge urgency, or apply cell-specific offsets , thus limiting the practical benefit of AI-aided mobility.

Including the predicted measurements in the event-triggered report would give the gNB richer information. The network could rank multiple UEs’ requests, decide whether to advance or delay the handover, and compare the prediction with subsequent actual measurements to detect drift or model faults. When the UE appends its predicted RSRP (and optionally RSRQ / RS-SINR or Δ-RSRP) to the A3-prediction report, the gNB acquires a quantitative picture of the upcoming radio environment instead of a single yes/no flag. With numeric forecasts for both serving and neighbour cells, the scheduler can rank simultaneous requests: a UE that predicts a –3 dB margin in 40 ms clearly needs faster attention than one forecasting a –0.5 dB margin in 150 ms. This enables smarter admission control, uplink-grant prioritization, and handover-preparation sequencing, particularly in hot-spot cells where dozens of UEs could approach the A3 threshold concurrently.

Moreover, the gNB can use the predicted values to fine-tune handover timing. If the forecast indicates only a shallow crossing of the threshold, the network may defer the command to avoid ping-pong; if it shows a steep, rapidly widening delta, it can expedite execution. After the handover, the gNB can cross-check the predictions against the UE’s subsequent actual measurements. Systematic discrepancies may reveal model drift, or context the model failed to capture (e.g., sudden interference)

For indirect measurement-event prediction use case, the report should include the predicted RSRPs, and RAN4 shall assume similar RSRP accuracy requirements as those applied to predicted measurement RSRPs. The applicable accuracy levels may be further refined considering key factors such as observation-window length, prediction-window duration, and other configuration parameters.

Conclusion

In this paper we provide our view on the Rel-19 AI/ML mobility measurement event prediction scenario to be studied in RAN4 working group.

The following Observations and Proposals were made:

- Two predictors with identical true detections can yield very different F1 scores solely because of their reporting style: a periodic predictor fires on every sampling instant, generating many unmatched predictions that inflate the false-positive counter n1, lower precision, and drive F1 down, whereas an event-based predictor reports only near likely A3 triggers, keeping n1 small and precision, and thus F1,much higher. This demonstrates that F1 intertwines true predictive ability with reporting frequency, so RAN4 conformance tests based only on F1 could rate equally accurate models very differently depending on whether they use periodic or event-based reporting

- RAN4 should set similar performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, but with different emphasis. Case A should focus more on prediction lead time and reliability to enable earlier, safe handover preparation. Case B should focus more on RSRP reconstruction accuracy and correctness of the derived event decision to ensure measurement-overhead reduction without mobility degradation

RAN4 Key Performance Metrics for Indirect Measurement-Event Prediction

Windowed precision ≥ P_min – Precision is the ratio of correctly predicted A3 events to all predicted events, counting only matches within the allowed lead-time window. This directly limits false positives (false alarms) that would otherwise waste handover resources

Lead-time window [L_min, L_max] – Each accepted prediction must arrive at least L_min but no more than L_max before the actual A3 trigger. The lower bound ensures sufficient time for resource reservation and signaling; the upper bound prevents stale or premature predictions.

Predicted-RSRP accuracy – UE reports predicted Layer-3 RSRP values with the event. Those predictions shall meet the same absolute and relative accuracy limits defined for conventional A3 measurements, enabling correct target-cell ranking and handover timing.

For indirect measurement-event prediction use case, the report should include the predicted RSRPs, and RAN4 shall assume similar RSRP accuracy requirements as those applied to predicted measurement RSRPs. The applicable accuracy levels may be further refined considering key factors such as observation-window length, prediction-window duration, and other configuration parameters.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, Source: OPPO, InterDigital, Beijing, China, Sept. 15–18, 2025, Document for: Approval, Agenda Item: 9.3.2.1.