# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513600

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.13.2.1

Source:	Apple

Title:	On RRM Performance Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#116 the RRM performance requirements for Rel-19 MIMO Phase 5 was discussed and way forward [1] was agreed. In this contribution we present our views on the performance requirements impact and test cases for UE initiated beam management.

# 2. Discussion

For UE-initiated beam reporting, the following table needs to be further discussed:

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

TCs for different Events

The core requirements have primarily been discussed based on Event 2 and in RAN4#115 there was agreement that the same design would be used for requirements with Event 1 and 7. The events differ in the conditions that trigger the event, while the reporting delay requirements are the same. Also, the beams measured for each of the events are different. For event 1 only current beam – RS of the indicated TCI state is measured; For Event 2 current beam and new beam are measured; For Event 7 the current beam – RS of the qth best activated TCI state and new beams are measured. The main aspects are measurement and comparison against a threshold. It would be sufficient to define performance test case for Event 2 as nothing different is tested by introducing test cases for event 1 and event 7.

- Introducing test case for Event 2 is sufficient and doesn’t test anything different with test cases for Event 1 or Event 7.

Introduce test case for Event 2 alone.

For testing event 7 we would need to configure multiple TCI states and at a minimum require 4 AoAs simultaneous transmission for the test set up to be feasible. The testing framework doesn’t support this currently and would be infeasible for it to be tested. Hence, we propose not to define requirement for event 7.

- For Event 7 at a minimum 4 simultaneous AoAs are required for test set-up  to support it.

Do not introduce test case for Event 7 as it is not feasible to be tested.

TCs for Mode A/B

The ending point is defined as when UE send first PUCCH which is the same for Mode A and Mode B. There is nothing different tested by introducing test cases for Mode A and Mode B. Only define test for either Mode A or Mode B, Mode A would be preferred.

- Given the ending point of first PUCCH transmission, the requirements are the same for Mode A and Mode B.

Introduce test case for either Mode-A (preferred) or Mode-B.

TCs for different scenarios

The requirements are applicable for inter-cell and intra-cell scenario. For SSB based, we explicitly have inter-cell and intra-cell scenario, while for CSI-RS based they cannot be differentiated. In order to cover both scenarios and minimize tests, we propose to introduce requirements for SSB based with inter-cell and CSI-RS based without explicitly saying inter-cell or intra-cell.

Introduce test cases for (1) CSI-RS based event trigger and (2) inter-cell SSB based event trigger.

TCs for time window-based evaluation

In RAN4#116 it was agreed to introduce core requirement for window-based evaluation with eventInstanceCount-r19 > 1. This is an optional feature and doesn’t test anything more than UEs ability to detect / count multiple trigger events. We propose not to define a test case for the same.

Do not introduce test cases for time window-based evaluation with eventInstanceCount-r19 > 1.

Applicable frequency range

The core requirements are defined for FR2 with priority and the test case shall also be defined for FR2 alone.

Introduce test cases in FR2 alone.

TCs for beam switching delay

In RAN4#115 it was agreed that existing requirements for TCI state switch delay are applicable for UE-initiated/event-triggered beam management. Also, there is no enhancement to beam switching delay with UE-initiated/event-triggered beam management. We don’t need to introduce a test case for the same.

Do not introduce test cases for TCI state switching/ beam switching delay for UE-initiated/event-triggered beam management.

# 3. Conclusion

In this paper, we present our views on the performance part impact and test cases for UE initiated beam reporting. Our observations and proposals are captured below:

- Introducing test case for Event 2 is sufficient and doesn’t test anything different with test cases for Event 1 or Event 7.

- Introduce test case for Event 2 alone.

- For Event 7 at a minimum 4 simultaneous AoAs are required for test set-up  to support it.

Do not introduce test case for Event 7 as it is not feasible to be tested.

- Given the ending point of first PUCCH transmission, the requirements are the same for Mode A and Mode B.

Introduce test case for either Mode-A (preferred) or Mode-B.

Introduce test cases for (1) CSI-RS based event trigger and (2) inter-cell SSB based event trigger.

Do not introduce test cases for time window-based evaluation with eventInstanceCount-r19 > 1.

Introduce test cases in FR2 alone.

Do not introduce test cases for TCI state switching/ beam switching delay for UE-initiated/event-triggered beam management.

# Reference

- R4-2508271, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.