# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513602

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.13.3

Source:	Apple

Title:	On UE Demodulation and CSI Reporting Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#116 the UE demodulation and CSI reporting requirements for MIMO Phase 5 WI were discussed and way forward [1] was agreed. In this contribution we present our views on the UE demodulation and CSI reporting requirements for this WI.

# 2. Discussion

## General/ Scope

Based on the discussion in RAN4#115 and RAN4#116, the table below summarizes of scope of UE demod and CSI reporting requirements for MIMO Phase 5 WI.

| Topic | Description | UE Demod Requirements Impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | No |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | Yes – PMI reporting requirements for Type I SP |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | FFS – PMI reporting requirements for eType II and eType II-Doppler codebooks |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | No |
|  | SRS Port grouping | FFS – PDSCH demod requirements |
| CJT Calibration reporting | Aperiodic standalone CJT calibration reporting | FFS –CSI reporting requirements |
| 3TX | 3-antenna-port codebook-based UL transmission | No |
| Asymmetric DL sTRP/UL mTRP | Asymmetric DL sTRP/UL mTRP | No |

### CSI Enhancements

For CSI enhancements, the following are introduced in RAN1:

- PMI reporting for 48, 64, 128 ports for the following codebooks

  - Rel-15 Type I-SP (Single panel)

  - Rel-15 Type I-MP (multi panel)

  - Rel-16 eType II

  - Rel-17 FeType II PS (port selection)

  - Rel-18 Type II-Doppler

  - Rel-18 Type II-CJT

- CRI reporting enhancement for up to 128 ports for Type I and eType II codebooks

- Fixed SRS port group mapping for SRS antenna switching when > 4 layers/ 2 2CW is supported

Codebook enhancement

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19

Way forward:

-	FFS on RAN4 should specify PMI reporting requirements for eTypeII-r19

-	Interested companies are welcomed to provide simulation results for next meeting

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

Way forward:

-	FFS on RAN4 should specify PMI reporting requirements for eTypeII-Doppler-r19

For codebook refinement for up to 128 ports it has been agreed to define PMI reporting requirements for typeI-SinglePanel-r19. And FFS if requirements are introduced for eType II and Type II-Doppler codebook for > 32 ports. It is not necessary to define requirements for all codebooks with increased number of TX. Hence, we propose not to define requirements for eType II and Type II-Doppler codebook with larger number of TX. The current field deployments heavily use Type -I codebook and we expect the same for larger number of CSI-RS ports. In case there is deployment for eType II and Type II-Doppler codebook for > 32 portsRAN4 can discuss defining requirements.

- It is not necessary to introduce PMI reporting requirements for all codebooks with increased number of TX ports

- Current field deployments heavily utilize Type-I codebook, and can be expected to continue with > 32 TX ports

Do not define requirements with eType II or Type II-Doppler codebook with > 32 TX ports in Rel-19.

SRS Port grouping

Issue 1-1-3: Clarify if UE demodulation/CSI reporting requirements should be specified for SRS port grouping

Proposals:

-	Option 1: RAN4 to discuss to reuse an existing3-layer, 4Rx demodulation requirement with a simple set up for SRS port grouping feature.

-	Option 2: RAN4 should not introduce UE demodulation/CSI reporting requirements for SRS port grouping

-	Option 3: Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19

Way forward:

-	Encourage supporters for introducing SRS port grouping related test requirements to provide more details on the test setup.

It is FFS if requirements should be introduced for SRS port grouping. SRS port grouping feature introduces fixed port grouping for low complexity 6RX /8RX receiver for > 4 MIMO layers. To properly verify this the test setup will need to have the following –

- SRS antenna switching

- Reciprocity based SVD precoder for PDSCH

Neither of these are introduced for demod requirements so far.

We cannot reuse the exiting requirements for testing SRS port grouping in our understanding.

- For testing SRS port grouping with UE demod requirements, we would need to introduce (1) SRS antenna switching (2) Reciprocity based SVD precoder which have not been done so far for UE demodulation requirements.

- Existing requirements cannot be reused to test SRS port grouping.

We can further discuss the feasibility of the test set up and introduction of requirements to test SRS port grouping in Rel-19.

Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19.

### CJT Reporting Enhancement

Issue 1-1-4: Clarify if UE demodulation requirements should be specified for UE reporting enhancement for CJT

Agreement:

-	RAN4 does not introduce demodulation requirements for reporting enhancements for CJT deployments.

Issue 1-1-5: Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT

Proposals:

For new CSI reporting for time/frequency/phase offsets:

-	Option 1: RAN4 should not introduce CSI reporting requirements for UE reporting enhancements for CJT

-	Option 2: RAN4 to further discuss the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test.

For PMI reporting:

-	Option 1: Further discussion on introducing new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation.

Way forward:

-	Encourage supporters for introducing CJT related CSI reporting test requirements to provide testable test setup.

Requirements for CJT reporting enhancements is discussed in RRM with evaluation for number of samples for defining delay and accuracy requirements for delay and frequency offset. For phase offset reporting RRM requirements will be de-prioritized in Rel-19

Agreements in RAN4#115 for MIMO Phase 5 RRM [2]:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

Agreements in RAN4#116 for MIMO Phase 5 RRM [3]:

#### Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? Agreement: - RAN4 to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, further discuss the accuracy and SNR in the performance part.

Introduction of RRM requirements for delay and accuracy would be sufficient to verify CJT calibration reporting feature. We don’t see the necessity to introduce a CSI reporting or PMI reporting requirement for the same as the test procedure would be complicated and we still need to evaluate if there is a reasonable and significant performance difference observed in link level simulations with compensation. It must also be pointed out that in RRM discussion there was an evaluation of achievable accuracy for frequency and delay offset reporting and there was no alignment in results even at higher SNR.

- RAN4 will define RRM requirements for delay and accuracy for time and frequency offset reporting for CJT calibration

- RRM requirements would be sufficient to verify implementation of this feature

- CSI/ PMI reporting requirements would need further study on test procedure and evaluation if performance delta is observed with compensation in link level simulations.

- In RRM the evaluation of accuracy for frequency and delay offset reporting doesn’t show good alignment

Do not introduce UE demodulation requirements for CJT calibration reporting in Rel-19.

## Test assumptions for typeI-SinglePanel-r19

Many of the test assumptions for Type I codebook with > 32 ports were agreed in [1].

Issue 1-2-2: N1 and N2 configuration for 64 CSI-RS ports Agreement: - (N1, N2) = (8,4) Issue 1-2-3: portMappingMethod for 64 CSI-RS ports Agreement: - Method1 Issue 1-2-4: MCS level for 64 CSI-RS ports Agreement: - Start simulation work with both MCS20 and MCS22. Down select further based on simulation results. Issue 1-2-7: Rule for CQI/RI/PMI delay Agreement: - RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing Capability 2. Issue 1-2-8: CQI/RI/PMI delay for FDD 64CSI-RS ports Agreement: - Apply slot n+4, PMI delay (from the PMI estimation slot to the PMI apply slot) 11ms, Report slot offset = [7], for FDD SCS=15kHz - Interested companies are welcomed to provide results when the reported PMI is applied at slot n+7 Issue 1-2-9: CQI/RI/PMI delay for TDD 64CSI-RS ports Agreement: - Apply slot n+6, PMI delay (from the PMI estimation slot to the PMI apply slot) 11.5ms, Report slot offset = 17, for TDD SCS=30kHz Issue 1-2-11: Correlation configuration Agreement: - XP-medium as starting point for simulation results alignment.

The issue comes from one company’s observation that the throughput with random precoding is very low. The issue stems from the fact that we use MIMO correlation factors not suitable for larger number of antenna. With larger number of TX and any correlation creates a strong spatial preference resulting in random precoder giving very low throughput. In case the same correlation coefficient as existing spec is used for 64 TX, then we recommend using low correlation for random precoding. Another alternative is to use a correlation coefficient suitable to large number of antennas.

- The MIMO correlation coefficients currently in spec are not suitable for large number of antenna and is expected to result in low throughput with random precoder.

To address the issue with very low TP with random precoder discuss the following options:
Option 1: Use the same correlation coefficients as legacy and modify the test to use low correlation with random precoder.
Option 2: Update the correlation coefficient to be suitable for larger number of TX.

Issue 1-2-10: Scaling factors for MIMO correlation matrices

Proposals:

-	Option 1: 0.00026 for 64 (8,4,2) × 2, 0.00050 for 64 (8,4,2) × 4, 0.00053 for 128 (16,4,2) × 2, 0.00101 for 128 (16,4,2) × 4

-	Option 2: RAN4 to use scaling factor a=0.00010 for both 64 CSI-RS ports

-	Option 3: do not use scaling factors

-	Other options are not precluded

Issue 1-2-11: Correlation configuration

Agreement:

-	XP-medium as starting point for simulation results alignment.

The two issues need to be discussed together. It was agreed to use XP-Medium as a baseline for alignment. As mentioned in earlier issue the current MIMO correlation coefficients are not suitable for larger number of TX and should be updated for requirements with larger number of TX ports.

RAN4 to discuss updating the correlation coefficient to be suitable for larger number of TX.

The scaling factor might need to be added to ensure the correlation matrix is positive definite. The scaling factor needs to be chosen based on the MIMO correlation and antenna configuration. We propose to discuss this alogn with the correlation coefficient.

The ‘a’ scaling factor should be chosen based on MIMO correlation coefficient and antenna config.

If it is agreed to introduce PMI reporting requirements with eType II and eTypeII-Doppler codebooks with > 32 ports the above proposals/ considerations shall also apply to them.

If PMI reporting requirements for eType II and eTypeII-Doppler codebooks with > 32 ports the proposals/ considerations on MIMO correlation, test metric shall also apply to those.

# 3. Conclusion

In this paper, we present our views on the scope of UE demodulation and CSI reporting requirements for MIMO Phase 5 WI. Our observations and proposals are captured below:

- It is not necessary to introduce PMI reporting requirements for all codebooks with increased number of TX ports

- Current field deployments heavily utilize Type-I codebook, and can be expected to continue with > 32 TX ports

- Do not define requirements with eType II or Type II-Doppler codebook with > 32 TX ports in Rel-19.

- For testing SRS port grouping with UE demod requirements, we would need to introduce (1) SRS antenna switching (2) Reciprocity based SVD precoder which have not been done so far for UE demodulation requirements.

- Existing requirements cannot be reused to test SRS port grouping.

Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19.

- RAN4 will define RRM requirements for delay and accuracy for time and frequency offset reporting for CJT calibration

- RRM requirements would be sufficient to verify implementation of this feature

- CSI/ PMI reporting requirements would need further study on test procedure and evaluation if performance delta is observed with compensation in link level simulations.

- In RRM the evaluation of accuracy for frequency and delay offset reporting doesn’t show good alignment

Do not introduce UE demodulation requirements for CJT calibration reporting in Rel-19.

- The MIMO correlation coefficients currently in spec are not suitable for large number of antenna and is expected to result in low throughput with random precoder.

To address the issue with very low TP with random precoder discuss the following options:
Option 1: Use the same correlation coefficients as legacy and modify the test to use low correlation with random precoder.
Option 2: Update the correlation coefficient to be suitable for larger number of TX.

RAN4 to discuss updating the correlation coefficient to be suitable for larger number of TX.

The ‘a’ scaling factor should be chosen based on MIMO correlation coefficient and antenna config.

If PMI reporting requirements for eType II and eTypeII-Doppler codebooks with > 32 ports the proposals/ considerations on MIMO correlation, test metric shall also apply to those.

# Reference

- R4-2512527, “Way Forward for [116][327] NR_MIMO_demod”, Samsung.

- R4-2508273, “WF on RRM requirements for NR_MIMO_Ph5_Part2”, MediaTek.

- R4-2512130, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung