3GPP TSG-RAN WG4 Meeting #116bis																R4-2513635

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	5.10.2

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirements for NR NTN Ku bands

Document for:	Discussion

# Introduction

The RRM core requirements were finalized at the last meeting, and the BigCR was agreed as in [1]. It builds on the Release 18 RRM requirements for Ka‑band FR2‑NTN and introduces new items, including satellite switching and timing accuracy requirements for NGSO mobile VSAT. This paper discusses RRM test cases for requirements verification.

# Test cases for NR VSAT Ku band with FR1-NTN and FR2-NTN

RRM requirements are typically band-agnostic, especially most VSAT RRM requirements are defined based on SAN FR1 with an intra-satellite assumption, but FR1-NTN/FR2-NTN distinctions may apply in test configurations.

To verify RRM requirements for Ku‑band VSAT, most of the existing test cases can be applied directly (FR2‑NTN uses FR2‑NTN cases; FR1‑NTN uses FR1 SAN), but new satellite‑switching test cases are required due to requirements that differ from FR1 SAN. Additionally, an FR1‑NTN intra-frequency inter-satellite handover test case is needed, as the requirements differ from FR1 SAN.

Proposal 1: introduce the following new test cases for Ku-band VSAT

- Satellite‑switching for both FR1-NTN and FR2-NTN

- Intra-frequency inter-satellite handover for FR1-NTN

Table 1 summarizes FR2‑NTN RRM test cases and their FR1 SAN counterparts. We recommend defining the RRM test cases using this table and reusing most existing cases rather than creating new ones.

Table 1 Existing RRM test cases for FR2-NTN and their FR1 SAN counterparts.

| Requirement |  | Exiting test cases | Comment for VSAT in Ku band |
| --- | --- | --- | --- |
| Re-selection 4.2C.1 | FR1-NTN | A.14.1.1 Cell reselection to FR1 intra-frequency NR case A.14.1.5 Cell reselection to FR1 inter-frequency NR case | Reuse the existing FR1 test cases. No need to add new test cases but clarify with test applicability. |
|  | FR2-NTN | missing | n.a. |
| Handover 6.1C.1.3 | FR1-NTN | A.14.2.1.1 Intra-frequency SAN Handover from FR1 to FR1 | - intra-satellite: Reuse the existing FR1 test cases. - inter-satellite: define new test cases |
|  | FR2-NTN | A.14.2.1.7 Intra-frequency intra-satellite Handover from FR2-NTN to FR2-NTN A.14.2.1.9 Intra-frequency inter-satellite handover from FR2-NTN to FR2-NTN | Reuse the existing FR2-NTN test cases. |
| Satellite switching 6.1C.3.3 | FR1-NTN, FR2-NTN | n.a | New tests for both FR1-NTN and FR2-NTN. |
| UE transmit timing 7.1C.2 | FR1-NTN, FR2-NTN | A.14.3.1.1 NR UE Transmit Timing Test for FR1 A.14.3.1.2 NR UE Transmit Timing Test for FR2-NTN | Reuse the existing FR1/ FR2-NTN test cases. |
| Timing Advance 7.3C | FR1-NTN, FR2-NTN | A.14.3.2.1 SA FR1 timing advance adjustment accuracy A.14.3.2.3 SA FR2-NTN timing advance adjustment accuracy | Reuse the existing FR1/ FR2-NTN test cases. |
| Radio Link Monitoring 8.1C | FR1-NTN, FR2-NTN | A.14.4.1.1 Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode A.14.4.1.2 Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode A.14.4.1.9 Radio Link Monitoring Out-of-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode A.14.4.1.10 Radio Link Monitoring In-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode | Reuse the existing FR1/ FR2-NTN test cases. |
| Measurement Procedure 9.2C | L3 FR1-NTN, FR2-NTN | A.14.5.1.1 SA event triggered reporting tests without gap under non-DRX A.14.5.1.7 SA event triggered reporting test with SSB time index reading without gap under non-DRX for FR2-NTN | Reuse the existing FR1/ FR2-NTN test cases. |
| 9.5C.7 | L1 FR1-NTN, FR2-NTN | A.14.5.3.1 SSB based L1-RSRP measurement for satellite access when DRX is not used A.14.5.3.5 SSB based L1-RSRP measurement when DRX is not used in FR2-NTN | Reuse the existing FR1/ FR2-NTN test cases. |
| Measurement performance 10.1.2C 10.1.3C | L3 FR1-NTN, FR2-NTN | A.14.6.1.1 SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell A.14.6.1.3 SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | Reuse the existing FR1/ FR2-NTN test cases. |

Additionally, while reusing existing cases, it would be beneficial to add a table clarifying applicability for VSATs using FR1‑NTN numerology, as not all FR1 SAN tests apply. Proposal 2 provides an example.

Proposal 2: Add a table clarifying test applicability for FR1‑NTN VSATs, as shown below.

| Requirement | Applicable test cases for VSAT with FR1-NTN numerology |
| --- | --- |
| Re-selection | A.14.1.1, A.14.1.5 |
| Handover | A.14.2.1.1 |
| Satellite switching | TBA |
| UE transmit timing | A.14.3.1.1 |
| Timing Advance | A.14.3.2.1 |
| Radio Link Monitoring | A.14.4.1.1, A.14.4.1.2 |
| Measurement Procedure | A.14.5.1.1, A.14.5.3.1 |
| Measurement performance | A.14.6.1.1 |

# Summary

Our observations and proposals in this paper are summarized as below:

Proposal 1: introduce the following new test cases for Ku‑band VSAT

- Satellite-switching for both FR1-NTN and FR2-NTN

- Intra-frequency inter-satellite handover for FR1-NTN

Proposal 2: Add a table clarifying test applicability for FR1‑NTN VSATs, as shown below.

| Requirement | Applicable test cases for VSAT with FR1-NTN numerology |
| --- | --- |
| Re-selection | A.14.1.1, A.14.1.5 |
| Handover | A.14.2.1.1 |
| Satellite switching | TBA |
| UE transmit timing | A.14.3.1.1 |
| Timing Advance | A.14.3.2.1 |
| Radio Link Monitoring | A.14.4.1.1, A.14.4.1.2 |
| Measurement Procedure | A.14.5.1.1, A.14.5.3.1 |
| Measurement performance | A.14.6.1.1 |

# References

- R4-2512351, Big CR on RRM requirements of Ku band for NR NTN, R4#116.