3GPP TSG-RAN WG4 Meeting #116bis																R4-2513639

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.21.2

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirements for IoT-NTN TDD mode

Document for:	Discussion

# Introduction

The RRM requirements for NB-IoT frame structure type 1 for NTN-TDD has been introduced in [1]. In this paper, the corresponding RRM tests cases to verify the requirement are discussed.

# RRM test cases for NB-IoT for frame structure type 1 for NTN-TDD

The table below summarizes potential RRM tests for the new core requirements.

| Test case | Core requirement | Description | Reference |
| --- | --- | --- | --- |
| Re-selection | 4.6B | Intra-frequency case, in normal coverage | A.13.1.1 |
|  |  | Inter-frequency case, in normal coverage | A.13.1.4 |
| RRC Re-establishment | 6.5B | Intra-frequency RRC Re-establishment, under normal coverage | A.13.3.1 |
| Random Access | 6.6B | Contention Based Random Access, in normal coverage | A.13.3.2.1 |
| UE transmit timing | 7.20B | UE Transmit Timing Accuracy, in normal coverage | A.13.4.1.1 |
| Timing Advance | 7.22B | UE Transmit Timing Advance Adjustment Accuracy, in normal coverage | A.13.4.2.1 |
| Radio Link Monitoring | 7.23B | out-of-sync / in-sync test, with/without DRX, 4 tests in total | A.13.4.3.1/4/5/7 |
| Measurements | 8.14B | Intra-frequency neighbour cell measurement, in normal coverage | A.13.5.1 |
|  |  | Inter-frequency neighbour cell measurement, in normal coverage | A.13.5.2 |
| Downlink channel quality reporting accuracy | TBA | Downlink channel quality reporting accuracy, in normal coverage | A.13.6.2.1 |
|  |  | Downlink channel quality reporting accuracy on non-anchor carrier, in normal coverage | A.13.6.2.4 |

Proposal 1: Work split for the RRM tests cases in the provided list.

Additionally, the existing test cases can serve as references, but the requirements and test configurations need to be updated. For example, only the NGSO configuration is applicable, and the GSO configurations should be removed.

Proposal 2: Only NGSO configuration is applicable.

Besides, measurement accuracy requirements needs to be defined, including NRSRP, NRSRQ, and downlink channel quality reporting accuracy, similar to the existing 9.1.22A.1-8.

Proposal 3: Define measurement accuracy for IoT NTN TDD, including NRSRP, NRSRQ, and downlink channel quality reporting accuracy.

# Summary

Our observations and proposals in this paper are summarized as below:

Proposal 1: Work split for the RRM tests cases in the provided list.

| Test case | Core requirement | Description | Reference |
| --- | --- | --- | --- |
| Re-selection | 4.6B | Intra-frequency case, in normal coverage | A.13.1.1 |
|  |  | Inter-frequency case, in normal coverage | A.13.1.4 |
| RRC Re-establishment | 6.5B | Intra-frequency RRC Re-establishment, under normal coverage | A.13.3.1 |
| Random Access | 6.6B | Contention Based Random Access, in normal coverage | A.13.3.2.1 |
| UE transmit timing | 7.20B | UE Transmit Timing Accuracy, in normal coverage | A.13.4.1.1 |
| Timing Advance | 7.22B | UE Transmit Timing Advance Adjustment Accuracy, in normal coverage | A.13.4.2.1 |
| Radio Link Monitoring | 7.23B | out-of-sync / in-sync test, with/without DRX, 4 tests in total | A.13.4.3.1/4/5/7 |
| Measurements | 8.14B | Intra-frequency neighbour cell measurement, in normal coverage | A.13.5.1 |
|  |  | Inter-frequency neighbour cell measurement, in normal coverage | A.13.5.2 |
| Downlink channel quality reporting accuracy | TBA | Downlink channel quality reporting accuracy, in normal coverage | A.13.6.2.1 |
|  |  | Downlink channel quality reporting accuracy on non-anchor carrier, in normal coverage | A.13.6.2.4 |

Proposal 2: Only NGSO configuration is applicable.

Proposal 3: Define measurement accuracy for IoT NTN TDD, including NRSRP, NRSRQ, and downlink channel quality reporting accuracy.

# References

- R4-2511349, (IoT_NTN_TDD) Big CR 36.133 R19 UE RRM, R4#116.

# Appendix

Core Requirement summary for frame structure type 1 for NTN-TDD [1].

- 4.6B	Cell Selection and Re-selection Requirements for UE category NB-IoT for frame structure type 1 for NTN-TDD

- 6.5B	RRC Re-establishment for NB-IoT UEs for frame structure type 1 for NTN-TDD

- 6.6B	Random Access for UE category NB-IoT for frame structure type 1 for NTN-TDD

- 6.9B	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for frame structure type 1 for NTN-TDD

- 7.20B	UE transmit timing for NB-IoT for frame structure type 1 for NTN-TDD

- 7.21B	UE timer accuracy for NB-IoT for frame structure type 1 for NTN-TDD

- 7.22B	Timing Advance for NB-IoT for frame structure type 1 for NTN-TDD

- 7.23B	Radio Link Monitoring for Category NB-IoT UE for frame structure type 1 for NTN-TDD

- 8.14B	Measurements for UE category NB-IoT for frame structure type 1 for NTN-TDD