# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513650

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.13.3.2

Source: 	CMCC

Title: 			 Discussion on RRM performance impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on RRM performance requirements for UE-to-UE CLI handling, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

How to handle the collision between SSB/CSI-RS based measurement and UL transmission is discussed in RRM core part. This issue is handled case by case. For SSB based measurement, for SBFD-aware UE, existing requirements apply for SSB based serving cell and neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. For CSI-RS colliding with UL transmission, for aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission. Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement. Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach. It is necessary to define test cases to guarantee UE performance. Especially for the collision between periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement and dynamically scheduled UL transmission, UL transmission is prioritized, and measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended.

Proposal 1: it is proposed to define test cases for the handling of the collision between SSB/CSI-RS based L1 measurement and UL transmission

- SSB based measurement is collided with UL transmission

- L1 measurement based on aperiodic CSI-RS resource is collided with UL transmission

- L1 measurement based on periodic/semi-persistent CSI-RS resource is collided with UL transmission

For CSI-RS L1 measurement, there are mainly three issues: CSI-RS colliding with UL transmission, CSI-RS puncturing in frequency domain in SBFD symbols, and CSI-RS distributed over SBFD and non-SBFD symbols. The first issue is discussed in Proposal 4. Follwoing provide discussion on the latter two issues.

For CSI-RS puncturing in frequency domain in SBFD symbols, both DU configuration and DUD configuration are considered. For DU configuration, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. For DUD, there are three cases. Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2. For case 3, for L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs. In the test, not model the interfering UE. Since UE behaviour is different for different case, it is necessary to define test cases.

- Case 1: X1 >= X3 and X2 >= X3

- Case 2: X1 >= X3 and X2 < X3

- Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

- where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

Proposal 2: it is proposed to define test cases for CSI-RS puncturing in frequency domain in SBFD symbols

- DU configuration

- DUD configuration

- Case 1: X1 >= X3 and X2 >= X3

- Case 2: X1 >= X3 and X2 < X3

- Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

For the measurement that CSI-RS is distributed over SBFD and non-SBFD symbols, the core part agreements are duplicated as following. For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network. And the measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. For RLM/BFD/CBD, RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type, and existing measurement period apply. It is necessary to define test cases to guarantee the performance.

- Agreements in RAN4 #114bis - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19 - Agreements in RAN4 #115 - For L1-RSRP and L1-SINR - Measurement period is extended by using R16 NR-U approach. - For RLM/BFD/CBD - Confirm the following for defining the requirements - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type. - Existing measurement period apply.

Proposal 3: it is proposed to define test cases on CSI-RS L1 measurement with CSI-RS resources distributed over SBFD and non-SBFD symbols

- Conclusion

This contribution provides discussion on RRM performance impacts for SBFD operation. The proposals are:

Proposal 1: it is proposed to define test cases for the handling of the collision between SSB/CSI-RS based L1 measurement and UL transmission

- SSB based measurement is collided with UL transmission

- L1 measurement based on aperiodic CSI-RS resource is collided with UL transmission

- L1 measurement based on periodic/semi-persistent CSI-RS resource is collided with UL transmission

Proposal 2: it is proposed to define test cases for CSI-RS puncturing in frequency domain in SBFD symbols

- DU configuration

- DUD configuration

- Case 1: X1 >= X3 and X2 >= X3

- Case 2: X1 >= X3 and X2 < X3

- Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

Proposal 3: it is proposed to define test cases on CSI-RS L1 measurement with CSI-RS resources distributed over SBFD and non-SBFD symbols

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2502633, Updated summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting