# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513666

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.14.5

Source: 	CMCC

Title: 			 Discussion on demodulation performance requirements for Ambient-IoT BS

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on demodulation performance requirements for Ambient-IoT BS, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

Issue 3-1: Whether to define PDRCH demodulation requirement for Ambient-IoT - Way forward - Option 1: Define PDRCH demodulation requirement. - Option 2: Not define PDRCH demodulation requirements

For the open issue on whether to define PDRCH demodulation requirements for Ambient-IoT BS, we prefer to follow legacy way to define PDRCH demodulation requirement to guarantee the performance.

Proposal 1: it is proposed to define PDRCH demodulation requirements for Ambient-IoT BS to guarantee the performance.

Issue 3-2: D2R message type for testing - Way forward - Option 1: Define PDRCH demodulation requirements with Message 1 reception - Option 2: Define PDRCH demodulation requirements with Message 3 reception Issue 3-16: Test methodology - Agreement - 10% BLER with BLER=1− Number of responsed Message 2Number of transmitted Message 1 for testing - During countering the number of responded Message 2, the TE should check if the responded Message 2 is the correct response of Message 1.

For issue 3-2, both options are workable. From the perspective of test simplification, it is preferred to define PDRCH demodulation requirements with Message 1 reception. On the other hand, issue 3-2 and issue 3-16 are combined. With the agreements in issue 3-16, it is natural to define PDRCH demodulation requirements with Message 1 reception in issue 3-2

Proposal 2: it is proposed to define PDRCH demodulation requirements with Message 1 reception. .

Issue 3-11: SFO assumption - Way forward - Option 1: 104 ppm~105 ppm - Other options

105 PPM as assumed for device 1 in RF discussion. It is proposed to follow RF conclusion to adopt 104 ppm~105 ppm as SFO assumption to define PDRCH requirements.

Proposal 3: for SFO assumption, it is proposed to use 104 ppm~105 ppm.

Issue 3-15: Channel model - Way forward - Option 1: TDL-A30-10 - Other options are precluded

According to the WID, for Rel-19 Ambient IoT, the target scenario is indoor deployment scenario (D1). it is expected that device is stationary or in very low mobility. It is valid to adopt TDLA30-10 to define PDRCH demodulation requirements.

Proposal 4: it is proposed to adopt TDLA30-10 to define PDRCH requirements.

Issue 3-17: Specification structure - Way forward - Option 1: Specify the Ambient IoT BS demodulation requirements in “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception” and “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing” if RAN4 agreed to define the Ambient IoT BS demodulation requirements. - Option 2: The demodulation requirements can be captured in clause 10 of TS 38.194/195.

We do not think these two options are contradict. TS 38.194 is for Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception, and can be used to capture demodulation requirements for Ambient IoT Base Station (BS).

Proposal 5: the demodulation requirements for Ambient IoT Base Station (BS) is captured in TS 38.194.

- Conclusion

This contribution provides discussion on demodulation performance requirements for Ambient-IoT BS. The proposals are:

Proposal 1: it is proposed to define PDRCH demodulation requirements for Ambient-IoT BS to guarantee the performance.

Proposal 2: it is proposed to define PDRCH demodulation requirements with Message 1 reception. .

Proposal 3: for SFO assumption, it is proposed to use 104 ppm~105 ppm.

Proposal 4: it is proposed to adopt TDLA30-10 to define PDRCH requirements.

Proposal 5: the demodulation requirements for Ambient IoT Base Station (BS) is captured in TS 38.194.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2512603, Way Forward for [116][335] A-IoT_demod, Huawei, CMCC