3GPP TSG-RAN WG4 Meeting #116b                                                                       R4-2513689

Prague, Czech, Oct 13th – 17th, 2025

Title:	General Aspects - Post-Deployment Validation based on performance monitoring

Source:	Korea Testing Laboratory

Agenda item:	6.11.2

Document for:	Discussion

- Introduction

At Bengaluru meeting #116, TR 38.843[1] was updated to (i) recognize RAN4’s role in considering the need and feasibility of requirements and tests that ensure consistency and accuracy of UE-reported monitoring metrics (and other monitoring-related data), and (ii) prioritize Option 2 (“post-deployment post-activation functionality testing based on performance monitoring”). This prioritization allows reliable and testable monitoring procedures to be established across use cases, while detailed performance-monitoring design remains anchored in other RAN groups.

This contribution addresses the consistency and accuracy of monitoring around model update or fine-tuning events. We show that, in Beam Management, the lack of a clear update boundary can contaminate monitoring windows (mixing pre- and post-update occasions), leading to misclassification and unstable LCM decisions. To resolve this with minimal change, we propose a lightweight “effective-from” indication that reuses RAN1-agreed performance-monitoring timing structure without redefining metrics.

The proposal preserves current monitoring mechanisms, avoids changes to agreed metric definitions, and improves testability under RAN4’s mandate to ensure consistency and accuracy of UE-reported monitoring data. It also enables unambiguous classification of the first post-update occasion and, within Option-2, supports opening a short, bounded validation window if desired.

In Bengaluru meeting #116 Agenda Item: 7.18.1 Source: CAICT, NTU, Ericsson, Qualcomm, APPLE, Huawei, Hisilicon, OPPO, CATT, CMCC, NTT DOCOMO, INC., Vivo, Nokia, Xiaomi, Mediatek, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, Korea Testing Laboratory Title: Proposed update for TR 38.843 with RAN4 part Document for: Discussion 7.3.2.9 Post deployment handling [1] When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable For dealing with potential changes in the performance of functionalities, two options may be available. The options are not mutually exclusive: - Option 1 (Post-deployment pre-activation functionality/configuration update testing): Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - Other aspects not precluded. - Option 2 (Post-deployment post-activation functionality testing based on performance monitoring): Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Option2 is prioritized. Reliable and testable monitoring procedures can be established for all cases.

- Consistency and accuracy issues

This contribution identifies a gap in the current post deployment approach where Option 2 is prioritized -- “Option 2 (Post deployment post activation functionality testing based on performance monitoring) is prioritized; it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases.” These words frame the problem and motivate our analysis within the existing RAN4 framework.

Under the present arrangement, consistency and accuracy of statistics cannot be guaranteed across vendors or deployments. The core issue is that the RAN1 agreed monitoring machinery is strongly time-anchored and windowed, yet there is no common boundary when an updated AI/ML behaviour becomes effective; consequently, post-change observations are mixed with pre-change observations in ways that depend on scheduling and transport rather than the model’s behaviour.

RAN1 has made the Option 2 timing explicit. For BM Case 1, “the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.” And “Predefines a threshold X = 64 for the minimal slot offset, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report”. (RAN 1 #121) This creates a deterministic linkage on the decision time axis, not on delivery/transport timestamps.

Observation 1: If the first report under the updated model arrives before a network-known decision-time effective-from boundary, it cannot be deterministically placed on either side of the transition. The ensuing RS-PAI window may mix pre/post monitoring occasions and make pass/fail scheduler-phase dependent.

Agreement in RAN1 #121 At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X = 64 for the minimal slot offset, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately - The associated working assumption made in RAN1#120b will not be confirmed.

The accuracy metric itself is equally well defined and windowed: “For beam prediction accuracy report for monitoring, the report quantity RS-PAI is  Np (0 ≤ Np≤ N). Where N = 1, 3, 7, 15 is configured and the size of CSI field associated with the RS-PAI is log2(N+1).” (RAN 1 #121)

Importantly, RAN1 also spells out the counting condition for accurate instances. (RAN 1 #121) The RS‑PAI condition counts a monitoring occasion as accurate only if (i) the occasion has a linked inference report and (ii) at least one of the nrofBestBeamforMonitoring‑r19 resources maps to one of the nrofreportedpredictedrs‑r19 P‑CRI(s)/P‑SSBRI(s) of the linked inference report; otherwise, it is not counted. Thus, every pass/fail decision is computed over the latest N occasions that are no later than the CSI reference resource of the monitoring report.

Observation 2: Without an explicit effective-from boundary, the network cannot reliably separate pre-vs post-transition window; additionally, since RS-PAI conveys only  Np (not the used link-offset k), it cannot audit which occasions were linked to which inference reports.

Agreement in RAN1 #121 For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1)

The same construct is extended to BM Case 2: “for UE sided model, for inference report for BM Case2, a time instance for prediction is defined as a slot.”  and, for monitoring per-instance linkage is again minimal slot-offset with a threshold, “the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. Predefines a threshold X = 64. Otherwise, the transmission occasion for monitoring has no linked time instance.” (RAN 1 #121)

Observation 3: BM-Case-2 requires at least support to report one configured time instance; under slot-based linking (minimal offset, X=64), this single-instance view cannot disambiguate pre- vs post-transition reports across prediction horizons

Agreement in RAN1 #121 For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance. - Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP The associated working assumption made in RAN1#120b will not be confirmed.

In addition, BM Case 2 timing is parameterized as “time gap is [10ms, 20ms, 40ms, 80ms, 160ms]; N = [1, 2, 4, 8]” and Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource.” (RAN 1 #120, and “for inference, AP CSI RS for Set B is not supported.” (Conclusion, #120b).

Observation 4: In BM-Case-2, the earliest f-instance is tied to the last Set-B occasion (no later than the reference) and Set-B is fixed per report; therefore, time instances cannot be aligned to the transition moment; hence pre-/post-transition reports remain indistinguishable without an explicit decision-time boundary.

Agreement in RAN1 #120 For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement in RAN1 #120b Conclusion For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

These agreements, taken together, specify which occasions belong in each window and how those occasions are linked to the corresponding inference output. Because all of these rules operate strictly on the decision-time axis, the lack of a shared “effective-from” boundary for model activation makes validation results sensitive to incidental timing. Consider a simple BM-Case-1 update: the UE activates a fine-tuned model, promptly generates the first inference report under the new logic, but there is no clear indication of the exact moment when the behaviour changed. Under the RAN1 rules, the network must still compute RS-PAI over the “N latest transmission occasion(s)” that are “no later than” the monitoring report’s reference resource (RAN1 #121) and link each occasion to an inference report by “minimal slot offset” with  k≤64 (RAN 1 #121) but without a boundary, that N-window will mix pre-change and post-change occasions in deployment specific proportions.

Two networks applying identical Option 2 configurations could admit different mixtures solely due to scheduling, leading to different NP counts and conflicting pass/fail conclusions for the same update.

For example, consider two deployments (Network-A and Network-B) using identical Option-2 parameters (e.g., BM-Case-1, N=7, minimal slot-offset with X=64, same P/SP/AP report combination). The UE activates the updated model at time teff​ and promptly sends an inference report generated under the new logic. Both networks then compute RS-PAI over the “latest N occasions no later than the monitoring report’s CSI reference resource” and link each occasion to the closest inference report by the minimal slot-offset rule. However, due solely to scheduler phase (i.e., different RS transmission-opportunity timing), Network-A’s 7-occasion window may contain, say, 5 post-change and 2 pre-change occasions (with valid links), while Network-B’s window contains 3 post-change and 4 pre-change occasions (and possibly some link failures when  k≤64. Because RS-PAI counts “accurate” occasions in that window, the two networks obtain different NP values and can reach conflicting pass/fail conclusions for the same model update -- even though their Option-2 configurations are identical.

The ambiguity is even greater in BM-Case-2. Each validation must evaluate one or more prediction horizons, but the monitoring minimum guarantees only “at least support to report one beam prediction accuracy for one configured time instance” (RAN1 #121). At the same time, linkage still relies on the minimal slot-offset rule with X = 64 (RAN1 #121), and the prediction times are slot-based and separated by a configured time gap (RAN1 #120). The earliest time instance is tied to “the most recent occasion of the CSI-RS/SSB resource in Set B … no later than” the inference report’s reference resource (RAN1 #120), and AP CSI-RS is not supported for Set-B (Conclusion, RAN1 #120). Together, these constraints make the composition of the post-change occasion set highly sensitive to when the network believes the change occurred. In effect, the same updated model can be evaluated on different subsets of occasions and different f-instances simply because the boundary is not clearly defined.

Observation 5: Absent a network-known decision-time effective-from boundary, identical updates produce different RS-PAI windows across deployments, yielding divergent  Np and conflicting pass/fail (good/moderate/bad) outcomes under the same Option-2 configuration.

RAN1 supports both cases where the monitoring set is a subset of Set A and where it has the same size as Set A for a 1:1 mapping, but this leads to inconsistencies in the mapping definitions. When the monitoring set is a subset of Set A, the correspondence is defined by an RRC-configured bitmap (RAN1 #121). In contrast, for the 1:1 case where the monitoring set has the same size as Set A, the n-th monitoring resource links to the n-th resource in Set A (RAN1 #120b).

These inconsistencies become more problematic when Set A itself, its ordering, or the monitoring subset changes around the update time but is not aligned to a common effective-from moment. In such cases, a single RS-PAI window can span two different mappings, thereby degrading comparability.

Observation 6:  Model generalization and measurement fairness are orthogonal. Even if a model is well-generalized, changes to the monitoring set (subset bitmap or 1:1) alter the distribution of monitoring occasions; therefore, an effective-from-aligned switch-over and decision-time gating are required so that RS-PAI windows for performance monitoring report never mix pre-/post-change monitoring occasions.

Observation 7:   To preserve comparability across deployments, align any fine-tuning or model update to a single decision-time effective-from boundary. If the monitor-to-Set-A mapping changes (subset bitmap or 1:1), perform an effective-from-aligned switch-over at the effective-from time so that no RS-PAI window spans two different mappings.

Agreement in RAN1 #121 For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model, when the size of the set for monitoring is smaller than the size of Set A, - support the mapping of the resources in the set for monitoring to resources in Set A is configured via RRC, support - A X-bit bitmap with Y non-zero bits is configured by the RRC in CSI Report Config for monitoring, where X is the size of Set A and Y is the size of the set for monitoring - The x-th MSB of the bitmap corresponds to x-th resource in Set A - The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the set for monitoring, 1≤y≤Y

Agreement in RAN1 #120b For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A

RAN1 also clarifies how BM-Case-2 reports are structured so that the decision-time anchoring is explicit across prediction instances. If the number of time instances N is greater than 1, the report includes a time-instance indicator whose field size is the ceiling of is log2N. “Time instance #1” is the instance identified by that indicator, and “time instances #2 to #N” correspond to the remaining instances ordered by time. For each instance, CRI or SSBRI index k maps to the RSRP field with the same index k. Within time instance #1, RSRP #1 is reported as an absolute value, and the remaining RSRP fields are reported as differentials relative to that absolute value. (RAN1 #120bis) In addition, for a single CSI report, Set B must remain fixed across all time instances. (RAN1 #119) Together, these details mean that any validation must follow the decision-time structure already agreed in RAN1, without mixing or reinterpreting instances based on delivery timing

f

Observation 8: In BM-Case-2 of UE-side model, validation must adhere to the RAN1 decision-time structure with slot-based time instances, an explicit time instance indicator, and a fixed Set B per report, so instances must not be mixed or reinterpreted by delivery time variations.

D

Conclusion (RAN 1 #119) For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement in RAN1 #120b For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams Time instance indicator. CRI or SSBRI #1 of time instance #1. CRI or SSBRI #2 of time instance #1. …. CRI or SSBRI #K of time instance #1. CRI or SSBRI #1 of time instance #2. CRI or SSBRI #2 of time instance #2. …. CRI or SSBRI #K of time instance #2. …. CRI or SSBRI #1 of time instance #N. CRI or SSBRI #2 of time instance #N. …. CRI or SSBRI #K of time instance #N. RSRP #1 of time instance #1. Differential RSRP #2 of time instance #1. …. Differential RSRP #K of time instance #1. Differential RSRP #1 of time instance #2. Differential RSRP #2 of time instance #2. …. Differential RSRP #K of time instance #2. …. Differential RSRP #1 of time instance #N. Differential RSRP #2 of time instance #N. …. Differential RSRP #K of time instance #N. - Time instance indicator exist if N > 1 - The size of CSI field for time instance indicator is log2N - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances - Time instance #1 corresponds to the time instance indicated by the time instance indicator - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s) - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

On priorities and processing, RAN1 reuses legacy method, which further separates decision time from delivery time. This approach ensures that the new inference and monitoring reports are treated with the same priority calculation as legacy beam reports (L1-RSRP/L1-SINR), using PriiCSIy,k,c,s from TS 38.214 clause 5.2.1, where k=0 assigns higher priority (lower numerical value) to minimize collision risks with other UL transmissions. These decisions emphasize that delivery-time variations (CPU load, scheduling jitter) do not justify reclassifying occasions across the decision boundary.

Observation 9: Because monitoring reuses the legacy priority rule PriiCSIy,k,c,s  with k=0, delivery-time variations (scheduler jitter, CPU load) must not be used to reclassify reports across the decision-time boundary; validation remains on the decision-time axis.

Agreement in RAN1 #121 For the determination of CSI report priority value of a CSI report for inference, the existing PriiCSIy,k,c,s is reused - k = 0 for the CSI report for inference For the determination of CSI report priority value of a CSI report for monitoring, the existing PriiCSIy,k,c,s is reused - k = 0 for the CSI report for monitoring For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1, - Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report - Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report - Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report)

To make the issue concrete, consider a live cell where an updated model becomes active at time teff. The UE immediately produces a post-change inference report; the gNB receives it and then, a little later, receives any signalling that might imply a change occurred. Under the RAN1 rules, the gNB must compute RS-PAI over the “latest N occasions … no later than” the monitoring report’s reference resource and link each to the closest inference event with k≤64. Without a shared  tef , there is no principled way to decide whether the earliest post-change report belongs to the pre- or post-side of the transition. Depending on scheduler micro-timings and DRX, the very first occasion may be treated as pre-change in one deployment (delaying the validation window) and post-change in another (giving it disproportionate weight). The results aren't comparable, and since the rules are anchored to specific times, they can't be made comparable without a shared reference point.

A minimal and fully compatible remedy is to add a UE-originated, per-function indication that pins down the boundary while leaving all RAN1 anchors, metrics, and reporting formats untouched. Concretely: “A brief uplink message that indicates the precise moment when the updated logic takes effect” allows everyone to classify occasions deterministically while keeping Option 2 unchanged; in other words, “The ‘effective-from’ uplink indication enables Option-2 post deployment validation using the existing performance monitoring framework without introducing new metrics, anchors, or procedures, and with no changes to the reporting scheme; classification remains strictly on the decision timeline.” ([2], Section 2.1, Observations 1–2) We therefore propose to discuss adopting a minimal, explicit ‘effective- from’ uplink indication that (i) identifies the relevant functionality (e.g., beam management, CSI, positioning) in terms of the RAN1 agreed use case/anchor group, and (ii) marks the decision time moment when the updated behaviour applies; the indication should arrive no later than the earliest uplink that carries any observation produced under the updated behaviour (co-carrying with that first observation is acceptable) so that the post-change validation window can be opened cleanly using the existing RAN1 linkage and windowing rules. ([2], Observation 5 and Proposal 1–2)

For completeness, when a subset mapping is used, the mapping indicated by the RRC-configured X-bit bitmap (RAN1 #121) should perform an effective-from aligned changeover at the same effective-from moment, so that a single RS-PAI window never spans two different monitor-to-Set-A relationships; this preserves the integrity of the RAN1-agreed calculation (over the N latest monitoring occasions no later than the report’s reference resource, using the minimal slot-offset rule with X = 64).

An effective-from aligned changeover of the monitor-to-Set-A mapping (subset bitmap or 1:1) is required whenever that mapping changes (e.g., a change in Set-A composition/order or in the monitoring subset/bitmap), at the slot from which the updated configuration becomes effective. If the mapping does not change (i.e., a model-behaviour update only), no mapping changeover is performed. However, a clearly signalled effective-from boundary is still required to prevent RS-PAI windows from spanning two behaviours on the decision-time axis. The effective-from boundary is the slot from which the updated behaviour becomes valid.

d

Assume Set A has eight beams A1 to A8 and the monitoring set has three resources M1 to M3. The mapping is carried by an X-bit bitmap with X =Set A = 8, read from MSB (left) to LSB (right). If the bitmap is 1 0 1 0 0 0 1 0, the 1-bits appear at positions 1, 3, and 7; by rule, the x-th MSB corresponds to the x-th resource in Set A, so those positions select A1, A3, and A7. The y-th non-zero bit (in left-to-right order) maps to the y-th entry of the monitoring-set list, so the first 1-bit maps M1 A1, the second maps M2A3, and the third maps M3A7. If you change the bitmap to 0 1 0 0 1 0 0 1, the 1-bits move to positions 2, 5, and 8, and the mapping automatically becomes M1A2, M2A5, M3A8.

fs

After a model fine-tuning or update, three practical cases illustrate how the effective-from boundary and the effective-from-aligned switch-over apply.

- Model-behavior change only (no mapping change): The Top-K order, predicted RSRP, or accuracy distribution changes, but Set A and the X-bit bitmap remain exactly the same. In this case, effective-from is still required to prevent pre-/post-change occasions from mixing inside an RS-PAI window; the atomic switch is a no-op (there is nothing to switch), so correctness is achieved simply by gating the post-change window using the effective-from boundary.

- Set-A composition/order changes (mapping implied to change): The predicted/reporting beam set (or its indices) is updated. Because the bitmap indexes Set A, a new bitmap is implied. Here, signal effective-from and apply the new bitmap atomically at that moment. This guarantees that no RS-PAI window contains occasions evaluated under two different Set-A indexings, and the minimal-slot-offset linking and N-window rules remain clean and comparable.

- Monitoring-set/bitmap change (Set A unchanged): The monitoring resources (Y) are re-selected or re-ordered relative to Set A (X), so the X-bit bitmap changes even though Set A does not. Again, signal effective-from and perform an atomic bitmap switch at that moment so that every RS-PAI window is computed under a single, consistent monitor-to-Set-A relationship.

Observation 10:  When the monitor-to-Set-A mapping (subset bitmap or 1:1) changes, perform an effective-from-aligned switch-over at the effective-from boundary. If only the model behaviour changes, keep the mapping and use an effective-from boundary so that RS-PAI windows do not mix pre-/post-change occasions.

Two clarifications reinforce that the effective‑from remedy does not encroach on other RAN1 constructs. First, Associated ID ensures consistency of network‑side additional conditions across training/inference but does not provide when a new behaviour becomes active. (RAN1 #118bis)

Observation 11:  Associated ID aligns what is used across training and inference, but it does not convey when a new behaviour becomes active; it cannot replace a decision time effective from boundary.

Agreement in RAN1 #118b Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list..

Second, for BM‑Case 2 report types and monitoring combinations, RAN1 already fixed the valid P/SP/AP pairs, so the remedy leaves those intact: “support the following combination for inference report type and monitoring report type”. (RAN1 #120b)

fs

Observation 12:  In BM-Case-2, the valid P/SP/AP inference-and-monitoring combinations are fixed, so report type pairing alone cannot distinguish pre- vs post-transition reports; a decision-time effective-from boundary is still required.

Agreement in RAN1 #120b For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support.

In short, introducing a single, explicit decision-time boundary through a UE-originated effective-from signal makes Option-2 validation deterministic and auditable. The existing RAN1 rules remain untouched, including the minimal slot-offset linking rule, the RS-PAI definition with its counting condition and window sizes, the BM-Case-2 timing and indicator rules, and both the one-to-one and bitmap mapping models. With only this boundary added, mixed windows disappear, scheduler-phase ambiguity is removed, and everyday monitoring stays lean and simple.

fs

Proposal 1:  Discuss adopting a minimal, explicit “effective-from” UE uplink indication that marks when the updated behavior applies and identifies the relevant functionality/anchor group (per RAN1-agreed use cases).

Proposal 2: Discuss minimal delivery and classification rules on the decision-time axis to ensure deterministic RS-PAI boundaries. Each RS-PAI window for a performance-monitoring report is classified solely by the effective-from boundary, and delivery-time variations shall not trigger reclassification.

Proposal 3: When the monitor-to-Set-A mapping changes (subset bitmap or 1:1), use an effective-from-aligned switch-over so that no RS-PAI window spans two mappings.

- Conclusion

In this contribution, we provided our viewpoints on Post-Deployment Validation based on performance monitoring

Observation 1: If the first report under the updated model arrives before a network-known decision-time effective-from boundary, it cannot be deterministically placed on either side of the transition. The ensuing RS-PAI window may mix pre/post monitoring occasions and make pass/fail scheduler-phase dependent.

Observation 2: Without an explicit effective-from boundary, the network cannot reliably separate pre-vs post-transition window; additionally, since RS-PAI conveys only  Np (not the used link-offset k), it cannot audit which occasions were linked to which inference reports.

Observation 3: BM-Case-2 requires at least support to report one configured time instance; under slot-based linking (minimal offset, X=64), this single-instance view cannot disambiguate pre- vs post-transition reports across prediction horizons

Observation 4: In BM-Case-2, the earliest f-instance is tied to the last Set-B occasion (no later than the reference) and Set-B is fixed per report; therefore time instances cannot be aligned to the transition moment; hence pre-/post-transition reports remain indistinguishable without an explicit decision-time boundary.

Observation 5: Absent a network-known decision-time effective-from boundary, identical updates produce different RS-PAI windows across deployments, yielding divergent  Np and conflicting pass/fail (good/moderate/bad) outcomes under the same Option-2 configuration.

Observation 6: Model generalization and measurement fairness are orthogonal. Even if a model is well-generalized, changes to the monitoring set (subset bitmap or 1:1) alter the distribution of monitoring occasions; therefore, an effective-from-aligned switch-over and decision-time gating are required so that RS-PAI windows for performance monitoring report never mix pre-/post-change monitoring occasions.

Observation 7:   To preserve comparability across deployments, align any fine-tuning or model update to a single decision-time effective-from boundary. If the monitor-to-Set-A mapping changes (subset bitmap or 1:1), perform an effective-from-aligned switch-over at the effective-from time so that no RS-PAI window spans two different mappings.

Observation 8: In BM-Case-2 of UE-side model, validation must adhere to the RAN1 decision-time structure with slot-based time instances, an explicit time instance indicator, and a fixed Set B per report, so instances must not be mixed or reinterpreted by delivery time variations.

Observation 9: Because monitoring reuses the legacy priority rule PriiCSIy,k,c,s  with k=0, delivery-time variations (scheduler jitter, CPU load) must not be used to reclassify reports across the decision-time boundary; validation remains on the decision-time axis.

Observation 10:  When the monitor-to-Set-A mapping (subset bitmap or 1:1) changes, perform an effective-from-aligned switch-over at the effective-from boundary. If only the model behaviour changes, keep the mapping and use an effective-from boundary so that RS-PAI windows do not mix pre-/post-change occasions.

Observation 11:  Associated ID aligns what is used across training and inference, but it does not convey when a new behaviour becomes active; it cannot replace a decision time effective from boundary.

Observation 12:  In BM-Case-2, the valid P/SP/AP inference-and-monitoring combinations are fixed, so report type pairing alone cannot distinguish pre- vs post-transition reports; a decision-time effective-from boundary is still required.

Proposal 1: Discuss adopting a minimal, explicit “effective-from” UE uplink indication that marks when the updated behavior applies and identifies the relevant functionality/anchor group (per RAN1-agreed use cases).

Proposal 2: Discuss minimal delivery and classification rules on the decision-time axis to ensure deterministic RS-PAI boundaries. Each RS-PAI window for a performance-monitoring report is classified solely by the effective-from boundary, and delivery-time variations shall not trigger reclassification.

Proposal 3: When the monitor-to-Set-A mapping changes (subset bitmap or 1:1), use an effective-from-aligned switch-over so that no RS-PAI window spans two mappings.

4. Reference

[1] 3GPP TR 38.843, V19.0.0 (2025-09), “Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface”.

[2] “General Aspects-Deployment post-activation functionality testing based on performance monitoring,”  R4-2510136, RAN4 #116, Bengaluru, India.