3GPP TSG-RAN WG4 Meeting #116bis	R4- 2513698

# Prague, Czech Republic, Oct. 13 – 17, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for measurement event prediction

Agenda item:	7.12.4

Document for:	Discussion

Introduction

In this contribution, we provide our high-level views for measurement event prediction use case on AI/ML mobility based on revised WID [1] in RANP#109 meeting and WF[2].

Discussion

This RAN4#116bis meeting in October is the first meeting for AI/ML Mobility WI. Although RAN4 discussed AI/ML Mobility SI during Rel-19 period, some issues agreed to discuss further in WI phase. So, We provide our views on that issues based on WF[2], and high-level views also based on the WID[1].

The objectives of RAN4 part are captured below:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

According to the WID we can categorize the issues as below three parts.

- Core requirements for RRM measurement prediction and indirect measurement event prediction

- Investigation whether the core requirements for RRM measurement prediction can be resued for indirect measurement event prediction

- LCM-related core requirements to support the LCM functionality management

- Category #1: Core requirements for measurement event prediction

According to the WID[1], RAN4 need to consider below use cases and scenarios.

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported.

For measurement event prediction use case, two methods are defined in TR38.744. Namely indirect and direct measurement event prediction as illustrated in Figure 4.3-1 and Figure 4.3-2 respectively. (Captured from TR38.744)

As described in the WID[1], RAN4 will focus on indirect measurement event prediction use case first.

Issue 3-1: Performance metric for indirect measurement event prediction

Agreement: - On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase. - Capture the candidate proposals in the WF for information purpose.

Regarding the performance metric, L3 RSRP accuracy requirement and F1 score can be baseline. F1 score is defined at TR38.744 as following:

F1 score=2×Precision ×Recall Precision+Recall

Where:

Precision= n3n1+n3

Recall= n3n2+n3

For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following:

- Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa;

- Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD];

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD].

The ETD, i.e. timing difference between ground-truth event and predicted event, is illustrated in Figure 5.3.1-1:

Proposal 1: L3 RSRP accuracy requirement and F1 score can be baseline.

- Category #2: Investigation whether the core requirements for RRM measurement prediction can be reused for indirect measurement event prediction

If there are defined the reporting procedure for RRM measurement prediction results for the indirect measurement event prediction case, then the core requirements for RRM measurement prediction can be reused for indirect measurement event prediction.

Proposal 2: If the reporting procedure for RRM measurement prediction results are defined, the core requirements for RRM measurement prediction can be reused for indirect measurement event prediction..

- Category #3: LCM-related core requirements to support the LCM functionality management

To support the LCM functionality management, the post deployment testing can be necessary. Based on the results of post deployment testing, the AI/ML model can be fallback to legacy or update to new AI/ML model. For checking the AI/ML model in the field, the case A scenario seems to be more suitable. So, periodically or by event-triggered, the case A scenario can be operated for checking the AI/ML model.

Proposal 3: RAN4 need to discuss how to check the AI/ML model in the field for measurement event prediction use case.

Proposal 4: RAN4 need to discuss how to trigger the checking operation of the AI/ML model in the field for measurement event prediction use case.

Conclusion

In this contribution, we provide our high-level views for measurement event prediction use case on AI/ML mobility based on revised WID [1] in RANP#109 meeting and WF[2].

Proposal 1: L3 RSRP accuracy requirement and F1 score can be baseline.

Proposal 2: If the reporting procedure for RRM measurement prediction results are defined, the core requirements for RRM measurement prediction can be reused for indirect measurement event prediction..

Proposal 3: RAN4 need to discuss how to check the AI/ML model in the field for measurement event prediction use case.

Proposal 4: RAN4 need to discuss how to trigger the checking operation of the AI/ML model in the field for measurement event prediction use case.

Reference

[1] RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” OPPO, Interdigital

[2] R4-2512127, “WF on AI/ML Mobility Part 1,” Moderator (Nokia)