3GPP TSG-RAN4 Meeting #116bis	R4-2513711

# Prague, Czech Republic, 13th – 17th October, 2025

Source:	LG Electronics

Title: 	On Ku-band VSAT Half-Duplex RF requirements with FR1-

Numerology

Agenda Item:	7.6.4.1

Document for:	Discussion and Approval

# Introduction

3GPP TSG RAN Meeting #109 approved a New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2 [1]. The objectives for the WI are captured below.

This document discusses the Phase 1 of the “HD-FDD for NTN Core Part” of these WI objectives and proposes a way to address the RF-requirements for Half-Duplex operation by leveraging the RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable, and the RF-requirements for Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type.

### 4.1	Objective of SI or Core part WI or Testing part WI

HPUE for NTN Core Part

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types

  - Example bands are Band 256 and 255

  - Necessary Tx requirements, at least including

    - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

  - Specify the necessary signaling to support the above objectives

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

HD-FDD for NTN Core Part

- Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies

- Phase 1: Ku band with FR1 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 2:  Ku band and Ka band with FR2 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 1 work will commence immediately.  Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability

- Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

DL intra-band intra-SAN CA for NTN Core Part

- Specify RF and RRM requirements for NR NTN CA for UE/VSAT and SAN core requirements with 2DL component carrier and 1UL

  - Contiguous carriers in Ka band (Band n512)

    - 400 MHz + 400 MHz channel bandwidths

  - Non-contiguous carriers in L band (Band n255)

    - 5 MHz + 5 MHz channel bandwidths

The DL intra-band intra-SAN CA objectives will commence upon completion of HPUE for NTN Core Part (PC1.5 for IoT-NTN) and RAN1/RAN2 availability.

# Discussion on RF-requirements for Half-Duplex operation

The new functionality that Half-Duplex FDD operation adds on top of the FDD is that a device must be capable to switch between the RX and TX modes as it is no longer expected to operate RX and TX concurrently. This switch and the time needed for it has already been defined in the specifications (for RedCap) as discussed in 2.1 and 2.2.

When it comes to other RF-requirements there is no technical reason why already defined Ku-band FDD requirements could not be applied also for HD-FDD operation. For the performance part and testing of the RX-requirements introduction of new reference channels and test cases may be necessary.

Observation 1: Capability to switch between the Rx and Tx modes is the only added functionality that HD-FDD operation adds on top of the FDD operation. Timing requirement for this switch may be necessary unless this is already indirectly addressed with other already existing RF requirements.

Observation 2: There is no technical reason why already defined Ku- and Ka-band FDD RF-requirements could not be directly applied also for HD-FDD operation in these bands, while it may be necessary to make additional clarifications into 38.101-5 to avoid any ambiguities linked with the applicability and testing of the requirements for both FDD and HD-FDD operation.

## TS38.211 Physical channels and modulation

TS38.211 Physical channels and modulation [2] states following:

4.1 General

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units Tc= 1/(df_max * N_f) , where df_max = 480 000Hz and N_f= 4096.

------ Clip --- Clip -----

4.3.2 Slots

------ Clip --- Clip -----

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3.

A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3.

------ Clip --- Clip -----

Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx

| Transition time | FR1 | FR2 |
| --- | --- | --- |
| NTx-Rx | 25600 | 13792 |
| NRx-Tx | 25600 | 13792 |

Converting the NRx-TxTc and NTx-RxTc to microseconds we get following times as shown in Table 1 and we can make following observation.

Observation 3: For FR1 numerology the 38.211 Physical channels and modulation specification allows 13.02us transition time for transition from Tx to Rx and vice versa.

Observation 4: For FR2 numerology the 38.211 Physical channels and modulation specification allows 7.01us transition time for transition from Tx to Rx and vice versa.

Table 2-1 RX-TX and TX-RX transition time in microseconds

| Transition time | FR1 | FR2 | Unit |
| --- | --- | --- | --- |
| From Tx to Rx | 13,02 | 7,01 | us |
| From TX to TX | 13,02 | 7,01 | us |

## TS38.101-5 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

## RF requirements for Tx-Rx/Rx-Tx mode switching

TS38.101-5 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 19) does not explicitly define any transition or switching time requirement for device to switch the mode between the Tx and Rx modes and vice versa.

There are, however, requirements for Transmit ON/OFF time mask through definition of Transient period between the Transmit slot and the beginning of the OFF-power requirement, for both FR1 and FR2 numerologies and for both conducted and radiated measurements.

Transmittet ON/OFF or OFF/ON switching is the most complex and most time-consuming part of the Tx-Rx/Rx-Tx mode switching and any control that is done for the receiver usually takes significantly shorter time than control of the transmitter.

For Transmit ON/OFF time mask the TS38.101-5 defines the following:

Table 2-2 Transient period in Transmit ON/OFF time mask requirements in the TS38.101-5

| Sub-clause | Sub-clause title | Conducted/Radiated | FR1/FR2 | OFF level | Transient time |
| --- | --- | --- | --- | --- | --- |
| 6.3.2/3 | Transmit ON/OFF time mask | Conducted | FR1 | -50dBm | 10us 1 |
| 9.3.2/3 | Transmit ON/OFF time mask | Radiated | FR1 | -36dBm | 10us 2 |
| 9.3.2/3 | Transmit ON/OFF time mask | Radiated | FR2 | -36dBm | 5us |

While the specified Transmit OFF power of -50dBm and -36dBm EIRP for conducted and radiated tests respectively is not enough to protect the device’s own receiver from Tx self-interference causing receiver desensitization one can assume that a device that can meet the ON/OFF time mask requirement can also protect its own receiver from Tx self-interference when switching between the Tx and Rx modes and vice versa.

Observation 5: For FR1 numerology the 38.101-5 defines 10us transition period for Transmit ON/OFF time mask.

Observation 6: For FR2 numerology the 38.101-5 defines 5us transition period for Transmit ON/OFF time mask.

Observation 7: While the specified Transmit OFF power of -50dBm and -36dBm EIRP for conducted and radiated tests respectively is not enough to protect the device’s own receiver from Tx self-interference causing receiver desensitization one can assume that a device that can meet the ON/OFF time mask requirement can also protect its own receiver from Tx self-interference when switching between the Tx and Rx modes and vice versa.

## Transmitter output power definition in the RX requirements

Observation 8: Some of the RX requirements (like Adjacent channel selectivity and Blocking) define the output power for the device’s transmitter for the duration of the RX tests to ensure that FDD device does not desensitize its own receiver with the own transmitter. While the transmitter and receiver are not concurrently operating in HD-FDD mode defining the TX power as part of the RF requirement does not cause any harm and will indirectly address the Tx-Rx/Rx-Tx mode switching capability that was discussed in earlier sections.

In addition, for all receiver tests a feedback channel to test the throughput is necessary and hence it makes sense to define a proper TX configuration including the TX output power level also for the RX requirements.

## Use of FDD term in throughout the 38.101-5 specification

The term FDD is used in the specification in many places in a way that limits the applicability of the requirements for FDD Duplex mode only. Use of FDD term in the Clauses 5, 9, 10, 11 and related annexes is described in the Table 2-3.

Observation 9: For Clause 5 the term FDD is referring to the duplex mode defined in the Sub-clause 5.22 but there does not seem to be a technical justification why same definitions could not be used for HD-FDD also.

Observation 10: For Clause 9 and 10 the term FDD is clarifying the Reference measurement channel use, which are currently only defined for FDD operation. Further study is needed to understand if and how these could also be used for HD-FDD operation and/or if new reference channels are needed.

Observation 11: There is about -0.5dB difference of REFSENS between Redcap HD-FDD and FDD in 38.101-1 while other requirements are similar.

Observation 12: For Clause 11 and Annexes the Reference measurement channels are currently only defined for FDD use. Further study is needed to understand if and how these could also be used for HD-FDD operation and/or if new reference channels are needed.

Table 2-3 Use of FDD term in the Clauses 5, 9, 10, 11

| Clause 5 Operating bands and channel arrangement |  |  |
| --- | --- | --- |
| Sub-clause 5.22 | Operating bands for FR1-NTN | Define the bands n248 and n247 as operating bands with FR1-NTN numerology for the Ku-band frequencies. This band definition clearly states that these bands are designed for FDD operation. |
| Sub-clause 5.3.6 | Asymmetric channel bandwidths | define the requirements for FDD asymmetric UL and DL channel bandwidth combinations |
| Sub-clause 5.4.2 | Channel raster defines | defines the Δshift for FDD FR1 NTN bands |
| Clause 9 Radiated transmitter requirements |  |  |
| None, but UL reference channels only defined for FDD in A.2 UL reference measurement channels There is about -0.5dB difference of REFSENS between Redcap HD-FDD and FDD in 38.101-1 while other requirements are similar. |  |  |
| Clause 10 Radiated receiver requirements |  |  |
| Sub-clause 10.3.2 | OTA reference sensitivity level Minimum requirement | references Annex A3.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 |
| Sub-clause 10.5.1 | Adjacent channel selectivity Minimum requirement | references Annex A3.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 |
| Sub-clause 10.5.2 | Adjacent channel selectivity Minimum requirement | references Annex A3.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 |
| Sub-clause 10.6.2 | Blocking characteristics Minimum requirement | references Annex A3.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 |
| Sub-clause 10.6.3 | Blocking characteristics Minimum requirement | references Annex A3.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 |
| Clause 11 Demodulation performance requirements (Radiated requirements) |  |  |
| 11.2.2 | PDSCH demodulation requirements | OP.1 FDD as defined in Annex A.5.1.1 of 38.101-4 |
| 11.2.2.1.1.1 | PDSCH demodulation requirements 1Rx requirements FDD | Requirements defined for FDD reference channel |
| 11.2.3 | PDCCH demodulation requirements | OP.1 FDD as defined in Annex A.5.1.1 |
| 11.2.2.1.1.1 | PDSCH demodulation requirements 1Rx requirements FDD | Requirements defined for FDD reference channel |
| Annex / Definition of Reference measurement channels |  |  |
| Currently reference channels are only defined for FDD operation only. |  |  |

Proposal 1: Directly apply the existing Clause 5 FR1-NTN FDD requirements for HD-FDD also (FDD  FDD / HD-FDD).

Proposal 2: Consider existing Transmit ON/OFF time mask requirement to be sufficient to ensure that HD-FDD device can also protect its own receiver from Tx self-interference.

Proposal 3: Consider applying the existing RF-requirements (Clause 9 and Clause 10) for FR1-NTN to support HD-FDD. Clarifications into 38.101-5 to avoid any ambiguities linked with the applicability and testing of the requirements for both FDD and HD-FDD operation may be necessary e.g. which reference measurement channels to use and how.

Proposal 4: For Clause 11 and Annexes the Reference measurement channels are currently defined for FDD only. Further study is needed to understand if and how these could also be used for HD-FDD operation and/or if new reference channels are needed.

# Discussion on RF-requirements for Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

The global automotive antenna market is expected to be $12.2Bn by the end of the decade and automotive satellite connectivity has a forecast value of $10.3Bn by 2033 [5]. Automotive-mounted VSAT (circa. 20 x 20 cm) are considered a key segment for Ku/Ka band NR-NTN market penetration due to large volumes.

To enable Ku-band service in vehicles, the most critical factors are size and cost. The size has been defined as 20 cm by 20 cm. In order to achieve low-cost implementation, several conditions must be met.  First, the phase shifter should be implemented with a low bit resolution, such as 1-bit. Second, instead of constructing the 20 cm by 20 cm panel as a single unit, it should be composed of multiple smaller panels combined to form the overall size. This approach can achieve the lowest possible cost [10].

However, it should be noted that while cost can be reduced in this manner, the performance of a 20 cm by 20 cm panel composed of multiple smaller panels with low-bit phase shifters (e.g., 1-bit) will be inferior compared to a single panel equipped with high-resolution phase shifters, such as7-bit to 8-bit.

In this article, performance has been estimated with consideration for low-cost and high-cost implementation.

Ku-band satellite communication systems feature an uplink center frequency of 14.125 GHz and a downlink center frequency of 11.725 GHz, with a frequency separation of approximately 2.4 GHz. For vehicle-mounted systems, the antenna array must be implemented within a compact 200 mm × 200 mm space. Using a λ/2 spacing (approximately 11.9 mm) at the 12.6 GHz center frequency, the element spacing translates to ~0.46λ (0.425 ~ 0.5λ) for the downlink band (10.7 ~ 12.725 GHz) and ~0.56λ for the uplink band (14.125 GHz). This maintains a near-0.5λ spacing across both bands, ensuring stable beam scan coverage of at least ±60 degrees. However, the 0.46λ spacing in the downlink band may introduce minor beamforming efficiency losses (0.3~0.5 dB gain reduction, assume 0.3 dB gain reduction for high cost implementation and 0.5 dB gain reduction for low cost implementation).

Despite this, the integrated TRx array structure offers significant advantages, including space savings, shared array utilization, and improved beam tracking alignment. Consequently, for vehicle-mounted systems, a unified TRx array centered at 12.6 GHz proves to be the most practical solution, effectively balancing spatial constraints and performance requirements. This design ensures the necessary beamforming gain and scan performance across all bands while operating within limited space.

For an antenna system employing array beamforming, the minimum EIRP within the specified scan range shall occur at the maximum scan angle. This is due to the reduction in effective array gain and the cosine roll-off effect as the beam is steered away from boresight. The attenuation characteristics shall be evaluated using the Cosⁿ(θ) model in Table 1 and Figure 1.

Table 3-1 Cosⁿ(θ) type attenuation model (n=1 Theoritical Value)

| degree | n = 1 | n = 1.5 | n = 2 | n = 2.5 | n = 3 | n = 3.5 | n = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| 10 | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 | -0.3 |
| 15 | -0.2 | -0.2 | -0.3 | -0.4 | -0.5 | -0.5 | -0.6 |
| 20 | -0.3 | -0.4 | -0.5 | -0.7 | -0.8 | -0.9 | -1.1 |
| 25 | -0.4 | -0.6 | -0.9 | -1.1 | -1.3 | -1.5 | -1.7 |
| 30 | -0.6 | -0.9 | -1.2 | -1.6 | -1.9 | -2.2 | -2.5 |
| 35 | -0.9 | -1.3 | -1.7 | -2.2 | -2.6 | -3.0 | -3.5 |
| 40 | -1.2 | -1.7 | -2.3 | -2.9 | -3.5 | -4.1 | -4.6 |
| 45 | -1.5 | -2.3 | -3.0 | -3.8 | -4.5 | -5.3 | -6.0 |
| 50 | -1.9 | -2.9 | -3.8 | -4.8 | -5.8 | -6.7 | -7.7 |
| 55 | -2.4 | -3.6 | -4.8 | -6.0 | -7.2 | -8.4 | -9.7 |
| 60 | -3.0 | -4.5 | -6.0 | -7.5 | -9.0 | -10.5 | -12.0 |
| 65 | -3.7 | -5.6 | -7.5 | -9.4 | -11.2 | -13.1 | -15.0 |
| 70 | -4.7 | -7.0 | -9.3 | -11.6 | -14.0 | -16.3 | -18.6 |

- Boresight direction is  0 degree

Figure 3-1 Cosine roll-off effect as the beam is steered away from boresight

Theoretically, the Effective Isotropic Radiated Power (EIRP) in a direction with beam scan angle θ decreases proportionally with Cos(θ). For instance, at boresight (θ = 0°), EIRP is maximized, whereas at a 60° tilt (θ = 60°), a reduction of approximately 3 dB occurs due to Cos(60°) = 0.5, effectively halving the perceived energy density as if the projected area of the array towards the receiver has been halved. However, in practical phased array implementations, the gain degradation often exhibits a steeper decline than the ideal Cos(θ) function. This discrepancy arises from several factors, including the inherent radiation patterns of individual radiating elements within the array, characterized by an exponent n.

Among the factors contributing to Scan Loss, the primary one is Mutual Coupling, which intensifies as the number of elements increases and the spacing between elements decreases, leading to a higher n value. To account for these practical conditions theoretically, an attenuation model of the form Cosn(θ) is employed, where n is an exponent parameter controlling the slope of the loss. Typically, various n values are observed, with patch antennas often exhibiting scan loss slopes in the range of n=1.5 to 2.5 w.r.t 48 elements [6 instead of below 57] and 64 elements [7 instead of below 62]. In the journal discussing ‘a fully integrated 384-element, 16-tile, W-band phased array system featuring self-alignment and self-test capabilities’ [8], the results indicated that the value of 'n' was over 5.

Figure 3-2 Scan Loss vs various antenna [9] and one example (W band, 384 elements) [8]

Table 3-2 and Table 3-3 present the performance comparison for Ku-band arrays of size 45 × 45 and 17 × 17, respectively, implemented using both high-cost and low-cost approaches. In the high-cost implementation, the beamforming gain is calculated using 2.9×log2(N), and the attenuation is modeled with a cosn(θ) type function, where n=2.0 for the 45 × 45 array and n=1.5 for the 17 × 17 array. In the low-cost implementation, the beamforming gain is calculated as 2.5×log2(N) for the 45 × 45 array and 2.8×log2(N) for the 17 × 17 array, with attenuation modeled using cosn(θ) where n=2.5 for the 45 × 45 array and n=2.0 for the 17 × 17 array.

Table 3-2  Ku band (45 by 45 & 17 by 17) w/ high cost implementation

|  | Ku band Frequency (GHz) | Beamforming Gain | Antenna Gain | Expected Gain @ 90 degree | Expected Gain @ 35 degree |
| --- | --- | --- | --- | --- | --- |
| 45 by 45 | 11.725 (DL) | 31.6 dB | 3.2 dBi | 34.8 dBi | 30 dBi |
|  | 14.125 (UL) | 31.9 dB | 3.2 dBi | 35.1 dBi | 30.3 dBi |
| 17 by 17 | 11.725 (DL) | 23.4 dB | 3.2 dBi | 26.6 dBi | 23 dBi |
|  | 14.125 (UL) | 23.7 dB | 3.2 dBi | 26.9 dBi | 23.3 dBi |

- Use Beamforming gain equation : (2.9) × log₂(N)

- Cosⁿ(θ) type attenuation model (n = 2 for 45 by 45 and n= 1.5 for 17 by 17)

## EIS

EISREFSENS_50MHz for 45 by 45 using high cost implementation and low cost implementation can be estimated as below

EIS of 50 MHz= PkT+10LOG10BW+NF+IM+Target SNR-Grx

=-174+76.8+3+3+-1-30=-122.2      -122 dBm for high cost implementation

=-174+76.8+3+3+-1-24.2=-116.4  -116 dBm for low cost implementation

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT  (TS 38.101-5)

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n248, n247, n509, n508 | Mobile VSAT | 6 | ≤ -122 |

Table 3-3 Ku band (45 by 45 & 17 by 17) w/ low cost implementation

|  | Ku band Frequency (GHz) | Beamforming Gain | Antenna Gain | Expected Gain @ 90 degree | Expected Gain @ 35 degree |
| --- | --- | --- | --- | --- | --- |
| 45 by 45 | 11.725 (DL) | 27 dB | 3.2 dBi | 30.2 dBi | 24.2 dBi |
|  | 14.125 (UL) | 27.5 dB | 3.2 dBi | 30.7 dBi | 24.7 dBi |
| 17 by 17 | 11.725 (DL) | 22.4 dB | 3.2 dBi | 25.6 dBi | 20.8 dBi |
|  | 14.125 (UL) | 22.9 dB | 3.2 dBi | 26.1 dBi | 21.3 dBi |

- Use Beamforming gain equation : (2.8) × log₂(N) for 17 by 17 and (2.5) × log₂(N) for 45 by 45

- Cosⁿ(θ) type attenuation model (n = 2.5 for 45 by 45 and n= 2 for 17 by 17)

EISREFSENS_50MHz for 17 by 17 using high cost implementation and low cost implementation can be estimated as below

EIS of 50 MHz= PkT+10LOG10BW+NF+IM+Target SNR-Grx

=-174+76.8+3+3+-1-23=-115.2    -115 dBm for high cost implementation

=-174+76.8+3+3+-1-20.8=-113.0  -113 dBm for low cost implementation

Assuming the EIS of VSAT Type 6 and applying the same NF + IM values to VSAT Type 7, the range of EISREFSENS_50MHz for NTN VSAT Type 7 can be estimated to be approximately −113 dBm.

Observation 13: EISREFSENS 50MHz for Mobile VSAT with small antenna (20x20) with electronically steered antenna are as follows, considering cost of antenna

- -115 dBm for high cost

- -113 dBm for low cost

## EIRP

For a 45 by 45 array operating in the Ku band, assuming an attenuation model of the form Cosn(θ) with n=2.5, this implies that the power or gain reduction as a function of the scan angle θ follows this steeper-than-ideal Cos(θ) relationship due to practical factors like mutual coupling among elements.

Table 3-4 Ku band for 45 by 45 Min EIRP for Transmit power

|  | Min EIRP | Antenna Gain @ 35 degree | Transmit Power/ Input Power |
| --- | --- | --- | --- |
| Low cost Implementation | 60 dBm @ Channel BW 200MHz | 24.7 dBi (30.7-6) | 35.3 dBm @ Channel BW 200MHz |
| High cost Implementation | 60 dBm @ Channel BW 200MHz | 30.3 dBi (=35.1-4.8) | 29.7 dBm @ Channel BW 200MHz |

To define the Minimum Effective Isotropic Radiated Power (EIRP) for a vehicle-mounted mobile VSAT, indeed, establishing the transmit power is a critical first step. Given the assumptions:

- The individual Power Amplifier (PA) output power for a reference 45x45 array is equivalent to that of each PA in the vehicle-mounted mobile VSAT.

- The reduction in EIRP due to the difference in array size (from 45x45 to 17x17) is calculated using the projection area effect, which results in a loss of approximately 10log10((17×17)/(45×45)) ≈ -8.5 dB.

<Case 1>

For the high-cost implementation, the following transmit power assumptions and calculations are applied.

When the reference transmit power Pref​ is approximately 32.7 dBm (3 dB higher due to channel BW 400MHz):

P_ref(17by17) = Pref(45by45)−8.5 dB = 32.7 dBm−8.5 dB = 24.2 dBm

PA_Pout for 45 by 45 = 32.7 – 10*log10 (45x45) = 29.7 – 33.1 = -0.4 dBm

PA_Pout for 17 by 17 = 24.2 – 10*log10 (17x17) = 29.7 – 24.6 = -0.4 dBm

<Case 2>

For the low-cost implementation, the following transmit power assumptions and calculations are applied.

When the reference transmit power Pref​ is approximately 38.3 dBm (3 dB higher due to channel BW 400MHz):

P_ref(17by17) = Pref(45by45)−8.5 dB = 38.3 dBm−8.5 dB = 29.8 dBm

PA_Pout for 45 by 45 = 38.3 – 10*log10 (45x45) = 38.3 – 33.1 = 5.2 dBm

PA_Pout for 17 by 17 = 29.8– 10*log10 (17x17) = 29.8 – 24.6 = 5.2 dBm

<Case 3>

the reference transmit power Pref​ is the maximum TRP value of 40 dBm:

P_ref(17by17) = Pref(45by45) )−8.5 dB = 40 dBm−8.5 dB = 31.5 dBm

PA_Pout for 45 by 45 = 40 – 10*log10 (45x45) = 40 – 33.1 = 6.9 dBm

PA_Pout for 17 by 17 = 31.5 – 10*log10 (17x17) = 31.5 – 24.6 = 6.9 dBm

Table 3-5 Input ref. power and PA_Pout for 45x45 and 17x17 w/ high and low cost implementation of antenna

| Scenario | Pref (dBm) | 10·log10(N×N) (dB) | PA_Pout (dBm) |
| --- | --- | --- | --- |
| Case1 (45x45) | 32.7 | 33.1 | −0.4 |
| Case1 (17x17) | 24.2 | 24.6 | −0.4 |
| Case2 (45x45) | 38.3 | 33.1 | 5.2 |
| Case2 (17x17) | 29.8 | 24.6 | 5.2 |
| Case3 (45x45) | 40.0 | 33.1 | 6.9 |
| Case3 (17x17) | 31.5 | 24.6 | 6.9 |

Notes:

- Pref​(17×17) = Pref​(45×45)−8.5dB

- PA_Pout = Pref−10log10 (N×N)

- “Case1” assumes a reference transmit power of 29.7 dBm for high cost implementaion of array antenna

- “Case2” assumes a reference transmit power of 35.3 dBm for low cost implementation of array antenna

- “Case3” assumes Max TRP of 40dBm

For a 45 × 45 array, the individual PA_Pout varies from −0.4 dBm to 6.9 dBm, and consequently, the minimum EIRP is inherently dependent on the PA_Pout capability. Table 3-6 presents the minimum EIRP variation with antenna implementation based on the various PA_Pout values.

Table 3-6  Ku band for 17 by 17 Min EIRP using adjusted Input power

|  | PA_Pout | Total PA_Pout (= PA_Pout + 10log10(17x17)) | Antenna Gain @ 35 degree | Min EIRP |
| --- | --- | --- | --- | --- |
| Case 1 | -0.4 dBm | 24.2 dBm | 21.3 dBi (Table3-3) | 45.5 dBm |
|  |  |  | 23.3 dBi (Table3-2) | 47.5 dBm |
| Case 2 | 5.2 dBm | 29.8 dBm | 21.3 dBi (Table3-3) | 51.1 dBm |
|  |  |  | 23.3 dBi (Table3-2) | 53.1 dBm |
| Case 3 | 6.9 dBm | 31.5 dBm | 21.3 dBi (Table3-3) | 55.8 dBm |
|  |  |  | 23.3 dBi (Table3-2) | 57.8 dBm |

The resulting minimum EIRP ranges are approximately,

- PA_Pout = -0.4dBm

  - 45.5 dBm to 47.5 dBm

- PA_Pout = 5.2dBm

  - 51.1 dBm to 53.1 dBm

- PA_Pout = 6.9dBm

  - 55.8 dBm to 57.8 dBm

Observation 14: Minimum peak EIRP range for Mobile VSAT with small antenna (20x20) with electronically steered antenna are as follows, considering different PA_Pout and cost of antenna implementation(low and high)

- PA_Pout = -0.4dBm

  - 45.5 dBm to 47.5 dBm

- PA_Pout = 5.2dBm

  - 51.1 dBm to 53.1 dBm

- PA_Pout = 6.9dBm

  - 55.8 dBm to 57.8 dBm

Proposal 5: Consider Mobile VSAT with small antenna of 20x20 cm with electronically steered antenna as a starting point.

# Conclusions

This document discusses the Phase 1 of the “HD-FDD for NTN Core Part” of these WI objectives and proposes a way to address the RF-requirements for Hald-Duplex operation by leveraging the RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

[RF requirements for HD-FDD]

Observation 1: Capability to switch between the Rx and Tx modes is the only added functionality that HD-FDD operation adds on top of the FDD operation. Timing requirement for this switch may be necessary unless this is already indirectly addressed with other already existing RF requirements.

Observation 2: There is no technical reason why already defined Ku- and Ka-band FDD RF-requirements could not be directly applied also for HD-FDD operation in these bands, while it may be necessary to make additional clarifications into 38.101-5 to avoid any ambiguities linked with the applicability and testing of the requirements for both FDD and HD-FDD operation.

Observation 3: For FR1 numerology the 38.211 Physical channels and modulation specification allows 13.02us transition time for transition from Tx to Rx and vice versa.

Observation 4: For FR2 numerology the 38.211 Physical channels and modulation specification allows 7.01us transition time for transition from Tx to Rx and vice versa.

Observation 5: For FR1 numerology the 38.101-5 defines 10us transition period for Transmit ON/OFF time mask.

Observation 6: For FR2 numerology the 38.101-5 defines 5us transition period for Transmit ON/OFF time mask.

Observation 7: While the specified Transmit OFF power of -50dBm and -36dBm EIRP for conducted and radiated tests respectively may not be enough to protect the device’s own receiver from Tx self-interference causing receiver desensitization one can assume that a device that can meet the ON/OFF time mask requirement can also protect its own receiver from Tx self-interference when switching between the Tx and Rx modes and vice versa.

Observation 8: Some of the RX requirements (like Adjacent channel selectivity and Blocking) define the output power for the device’s transmitter for the duration of the RX tests to ensure that FDD device does not desensitize its own receiver with the own transmitter. While the transmitter and receiver are not concurrently operating in HD-FDD mode defining the TX power as part of the RF requirement does not cause any harm and will indirectly address the Tx-Rx/Rx-Tx mode switching capability that was discussed in earlier sections. In addition, for all receiver tests a feedback channel to test the throughput is necessary and hence it makes sense to define a proper TX configuration also for RX tests.

Observation 9: For Clause 5 the term FDD is referring to the duplex mode defined in the Sub-clause 5.22 but there does not seem to be a technical justification why same definitions could not be used for HD-FDD also.

Observation 10: For Clause 9 and 10 the term FDD is clarifying the Reference measurement channel use, which are currently only defined for FDD operation. Further study is needed to understand if and how these could also be used for HD-FDD operation and/or if new reference channels are needed.

Observation 11: There is about -0.5dB difference of REFSENS between Redcap HD-FDD and FDD in 38.101-1 while other requirements are similar .

Observation 12: For Clause 11 and Annexes the Reference measurement channels are currently only defined for FDD use. Further study is needed to understand if and how these could also be used for HD-FDD operation and/or if new reference channels are needed.

Considering that

- 38.101-5 already defines Transmit ON/OFF transient time requirement shorter than the transition time for transition from Tx to Rx and vice versa as specified in 38.211

- for both FR1 and FR2 numerologies that are used on Ku- and Ka- bands

- Transmit ON/OFF time mask requirement applies to all transmissions (also in HD-FDD mode)

there does not seem to be a need to introduce any new RF requirements specifically for HD-FDD operation and all existing FDD requirements can be applied also for operation in HD-FDD mode, while it may be necessary to make additional clarifications into 38.101-5 to avoid any ambiguities linked with the applicability and testing of the requirements for both FDD and HD-FDD operation.

Proposal 1: Directly apply the existing Clause 5 FR1-NTN FDD requirements for HD-FDD also (FDD  FDD / HD-FDD).

Proposal 2: Consider existing Transmit ON/OFF time mask requirement to be sufficient to ensure that HD-FDD device can also protect its own receiver from Tx self-interference.

Proposal 3: Consider applying the existing RF-requirements (Clause 9 and Clause 10) for FR1-NTN to support HD-FDD. Clarifications into 38.101-5 to avoid any ambiguities linked with the applicability and testing of the requirements for both FDD and HD-FDD operation may be necessary e.g. which reference measurement channels to use and how.

Proposal 4: For Clause 11 and Annexes the Reference measurement channels are currently defined for FDD only. Further study is needed to understand if and how these could also be used for HD-FDD operation and/or if new reference channels are needed.

[RF requirements for Mobile-VSAT with small antenna (e.g., 20x20cm)]

Observation 13: EISREFSENS 50MHz for Mobile VSAT with small antenna (20x20) with electronically steered antenna are as follows, considering cost of antenna

- -115 dBm for high cost

- -113 dBm for low cost

Observation 14: Minimum peak EIRP range for Mobile VSAT with small antenna (20x20) with electronically steered antenna are as follows, considering different PA_Pout and cost of antenna implementation (low and high)

- PA_Pout = -0.4dBm

  - 45.5 dBm to 47.5 dBm

- PA_Pout = 5.2dBm

  - 51.1 dBm to 53.1 dBm

- PA_Pout = 6.9dBm

  - 55.8 dBm to 57.8 dBm

Proposal 5: Consider Mobile VSAT with small antenna of 20x20 cm with electronically steered antenna as a starting point.

# References

[1] RP-252958, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, 3GPP TSG RAN Meeting #109, September 2025

[2] TS38.211 Physical channels and modulation (Release 19)

[3] TS38.101-5 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 19)

[4] R4-2510831, Big CR for 38.101-5 for the addition of Ku bands, Eutelsat Group, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

[5] RP-252958 New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, TSG RAN Meeting #109, Sep 2025

[6] T. Sowlati, S. Sarkar, B. G. Perumana et al., “A 60-GHz 144- element phased-array transceiver for backhaul application,” IEEE Journal of Solid-State Circuits, vol. 53, no. 12, pp. 3640–3659, 2018.

[7] X. Gu, D. Liu, C. Baks et al., “Development, implementation, and characterization of a 64-element dual-polarized phased array antenna module for 28-GHz high-speed data communications,” IEEE Transactions on Microwave eory and Techniques, vol. 67, no. 7, pp. 2975–2984, 2019.

[8] S. Shahramian, M. J. Holyoak, A. Singh, and Y. Baeyens, “A fully integrated 384-element, 16-tile, W -band phased array with self-alignment and self-test,” IEEE Journal of Solid-State Circuits, vol. 54, no. 9, pp. 2419–2434, 2019

[9] Federico, G., D. Caratelli, G. Theis, and A. B. Smolders, “A review of antenna array technologies for point-to-point and point-to-multipoint wireless communications at millimeter-wave frequencies,” International Journal of Antennas and Propagation, Vol. 2021, No. 1, 5559765, 2021

[10] S. Kim, H.-S. Choi, and B.-W. Min, “Scalable 32 × 32 1-bit reconfigurable intelligent surfaces for upper-mid band 6G communications,” in IEEE MTT-S Int. Microw. Symp. Dig., Jun. 2024, pp. 919ow. S