3GPP TSG-RAN WG4 Meeting #116bis			                                  R4-2513743

Prague, CZ, October 13rd – 17th, 2025

Agenda item:	7.11.3

Source: 	CMCC

Title: 	General consideration on R20 A-IoT Coexistence

Document for:	Discussion

# 1. Introduction

RAN#108 meeting approved RP-251884 [1] New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices. One of the objectives of the SI is studying coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. This paper will focus on the coexistence scenario assumption.

# 2. Discussion

## 2.1. General consideration on scenario

The description for RAN4 objective in the SID is shown as below:

RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

Fortunately, it has been clearly stated in the SID that the FR1 licensed spectrum in FDD in-band is for NR and in standalone bands, with R2D in the DL spectrum and D2R in the UL spectrum. Compared with the research in R19, the uplink and downlink frequencies used for R2D and D2R have been determined. Meanwhile, to further narrow down the simulation scenarios, it is suggested that, as in R19, the coexistence study focus on the coexistence between AIOT and NR.

Observation 1: The WID has clearly defined the uplink and downlink frequencies for R2D and D2R, and R19 focused on the coexistence between AIOT and NR.

Proposal 1: It is recommended that the research in R20 also focus on the coexistence between AIOT and NR.

Figure 1. coexistence scenario between A-IoT and NR

Table 1. coexistence scenario

| Cases | device | Spectrum deploymentmode | aggressor | victim |
| --- | --- | --- | --- | --- |
| e-1 | device 2b and C | in-band | device | FDD UL |
| c-1 |  |  | reader | FDD DL |
| d-1 |  |  | FDD DL | device |
| b-1 |  |  | FDD UL | reader |
| e-2 | device 2b and C | Stand alone | device | FDD UL |
| c-2 |  |  | reader | FDD DL |
| d-2 |  |  | FDD DL | device |
| b-2 |  |  | FDD UL | reader |

Under the current conditions, it is recommended that both types of devices be considered, and both in-band and stand-alone deployment mode be included in the simulation scenarios.

Proposal 2: Table 1 lists all the coexistence simulation scenarios.

2.2. Deployment parameters

Proposal 3：Consider the deployment parameters in table 2.

Table 2. A-IoT deployment parameters parameters

| Parameter | D4T1 |
| --- | --- |
| Carrier frequency | 900 MHz (Band n8) |
| BW for NR | 10MHz with 15KHz SCS |
| BW for R2D | 180kHz, for 15kHz SCS |
| BW for D2R | 15kHz |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |
| A-IoT UL power control | Open loop power control |
| NR BS and A-IoT deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around the minimum 2D distance between macro BS and device Inter-NR BS distance: 750m NR BS height: 25m |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 1m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| NR BS Inter-site distance | MCL of 70 dB |
| Pathloss model | High priority UMa NLOS defined in TR38.901 |
| O2I penetration loss | High penetration loss as in TR 38.901 |

2.3. A-IoT device parameters

Proposal 4：Consider the A-IoT device parameters in table 3.

Table 3. A-IoT device RF parameters

| A-IoT device parameters | Device 2b | Device C |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device Tx power (dB) | choose -10dBm between -10 and -20 dBm | choose 5dBm between -3, 0, -5 dBm |
| A-IoT device noise figure (dB) | 15 | 13 |
| ACLR (dB) | 30dB as starting point | 30dB as starting point |
| ACS (dB) | 30dB as starting point | 30dB as starting point |

2.4. A-IoT reader parameters

Proposal 5：Consider the A-IoT reader parameters in table 4.

Table 4. A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | Standalone: 43 dBm In-band: choose 38dBm between 38 and 33 dBm |
| A-IoT reader receiver Noise Figure (dB) | 5 |
| A-IoT reader antenna gain (dBi) | 17 |
| ACLR | 45dB as starting point |
| ACS | 46dB as starting point |
| Antenna configuration | Same as NR BS. |

2.5. NR BS and UE parameters

Proposal 6：NR BS and NR UE parameters in TR38.769 could be reused.

Table 5.NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

Table 6. NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

# 3. Conclusions

Observation 1: The WID has clearly defined the uplink and downlink frequencies for R2D and D2R, and R19 focused on the coexistence between AIOT and NR.

Proposal 1: It is recommended that the research in R20 also focus on the coexistence between AIOT and NR.

Proposal 2: Table 1 lists all the coexistence simulation scenarios.

Proposal 3：Consider the deployment parameters in table 2.

Proposal 4：Consider the A-IoT device parameters in table 3.

Proposal 5：Consider the A-IoT reader parameters in table 4.

Proposal 6：NR BS and NR UE parameters in TR38.769 could be reused.

# 4. Reference

[1] RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

[2] Chair note RAN1#122 v14

[3] TR38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, 19.0.0