3GPP TSG-RAN WG4 # 116-bis	R4-2513763

Prague, Czech Republic, 13th October 2025 – 17th October 2025

Agenda item:	7.5.3

Source:	MediaTek Inc.

Title:	Discussion on performance requirements for FR1 SU-MIMO with spatial channel model

Document for:	Discussion

# Introduction

The Rel-19 study item [1] investigated practical spatial channel modelling methodologies for MIMO demodulation and CSI reporting. The findings of the SI were reported in TR 38.753 [2]. In the Rel-20 work item [3], the target for RAN4 is to specify new performance requirements as follows:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - Using rCDL-C1 channel model detailed in TR 38.753 as starting point

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

# Discussion

# 2.1 Angular scaling update

In TR 38.753, the angular scaling process is in a central role of the rCDL-C1 model derivation. RAN4 finally mostly ignored the process specified in TR 38.827 [4] and tried to follow TR 38.901 [5]. However, the process defined in TR 38.901 V18.0.0 Section 7.7.5.1 is ambiguous at best since the equations seem to address clusters (cluster index n) while the text talks about rays. The authors of TR 38.827 and TR 38.753 interpreted it so that scaling is applied to ray angles, not only to tabulated cluster angles.

In the latest version of TR 38.901 V19.1.0 (2025-09), Section 7.7.5.1 and the related definitions (Appendix A) have significantly changed. As a result, the scaling process has changed from the earlier versions of TR 38.901. Compared to the scaling process used in TR 38.753, at least three major differences can be identified against TR 38.901 V19.1.0:

- In TR 38.901 V19.1.0, scaling is applied only to cluster angles so that the AS (width) of each cluster cASA, ASD,ZSA,ZSD remains unchanged. In TR 38.753, we scaled the rays so that also per cluster AS cASA, ASD,ZSA,ZSD is scaled.

- In TR 38.901 V19.1.0, the scaling factor is so defined (Appendix A.5) that the desired AS is exactly achieved. In TR 38.753, the scaling factor is defined so that the process only approximately achieves the desired AS.

- In Appendix A.3 of TR 38.901 V19.1.0, a new definition for angular spread is provided so that only cluster angles are used in the calculation. The earlier definition of Appendix A.1 (as also applied in TR 38.753) takes as input the ray-specific angles.

We believe that it is feasible to follow the updates made by RAN1 in TR 38.901 V19.1.0 in this work item. The changes however need to be agreed on as soon as possible so that simulation work can start.

Observation #1: RAN1 has significantly changed the AS scaling process definition in TR 38.901 from version V18.0.0 to version V19.1.0.

Observation #2: It is feasible to follow the angular scaling updates made by RAN1 in TR 38.901 V19.1.0 in this work item.

To cross-check with other companies, we derived a new version of CDL-C. We took the original CDL-C channel model in TR38.901 and applied the angular scaling process described in TR 38.901 V19.1.0. The desired angular spread values we took from TR38.753 (ASD: 25.7620, ASA: 74.1138, ZSD: 4.8978, ZSA: 18.2050). Using the angular spread definition in Appendix A.3 of TR 38.901 V19.1.0, our model achieves all four angular spread targets.

For simplicity, no sub-cluster merging or cluster truncation was used. Our result is provided for reference in Table 1.

Proposal #1: We invite other companies to re-do the angular scaling using the updated process in TR 38.901 V19.1.0 and cross-check the results.

Table 1: Untruncated CDL-C with updated angular scaling

| Cluster # | Cluster delay [ns] |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  | -4.4 |  | -38.9 |  | -93.7 |  | 95.5 |  | 107.1 |
| 2 | 77 |  | -1.2 |  | -22.9 |  | 118.7 |  | 98.0 |  | 70.3 |
| 3 | 81 |  | -3.5 |  | -22.9 |  | 118.7 |  | 98.0 |  | 70.3 |
| 4 | 85 |  | -5.2 |  | -22.9 |  | 118.7 |  | 98.0 |  | 70.3 |
| 5 | 79 |  | -2.5 |  | -34.9 |  | -121.9 |  | 101.6 |  | 65.6 |
| 6 | 232 |  | 0 |  | -7.4 |  | 172.2 |  | 99.1 |  | 77.9 |
| 7 | 235 |  | -2.2 |  | -7.4 |  | 172.2 |  | 99.1 |  | 77.9 |
| 8 | 239 |  | -3.9 |  | -7.4 |  | 172.2 |  | 99.1 |  | 77.9 |
| 9 | 240 |  | -7.4 |  | 41.5 |  | 50.1 |  | 109.8 |  | 59.2 |
| 10 | 290 |  | -7.1 |  | -50.9 |  | 61.9 |  | 92.1 |  | 50.7 |
| 11 | 300 |  | -10.7 |  | 46.3 |  | -37.6 |  | 111.4 |  | 68.7 |
| 12 | 341 |  | -11.1 |  | -72.8 |  | 41.1 |  | 88.9 |  | 42.8 |
| 13 | 448 |  | -5.1 |  | -44.7 |  | 63.6 |  | 107.1 |  | 114.2 |
| 14 | 478 |  | -6.8 |  | -50.8 |  | -59.4 |  | 108.0 |  | 41.9 |
| 15 | 792 |  | -8.7 |  | -60.3 |  | 77.8 |  | 88.0 |  | 44.0 |
| 16 | 989 |  | -13.2 |  | 61.4 |  | 23.9 |  | 108.0 |  | 138.2 |
| 17 | 1554 |  | -13.9 |  | 59.1 |  | -26.1 |  | 91.4 |  | 47.1 |
| 18 | 1679 |  | -13.9 |  | 52.1 |  | -4.7 |  | 88.2 |  | 57.5 |
| 19 | 2004 |  | -15.8 |  | -76.0 |  | -23.3 |  | 86.6 |  | 24.8 |
| 20 | 2047 |  | -17.1 |  | 54.4 |  | 1.6 |  | 112.5 |  | 45.9 |
| 21 | 2302 |  | -16 |  | 55.1 |  | -2.8 |  | 88.0 |  | 22.4 |
| 22 | 2423 |  | -15.7 |  | 64.1 |  | -8.0 |  | 87.9 |  | 45.7 |
| 23 | 2571 |  | -21.6 |  | 72.7 |  | -32.0 |  | 109.8 |  | 36.9 |
| 24 | 3158 |  | -22.8 |  | -90.8 |  | 26.9 |  | 114.4 |  | 34.5 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

# 2.2 Delay scaling after cluster truncation

In TR 38.753, the derivation process of rCDL-C1 consists of angular scaling, sub-cluster merging + cluster truncation down to 12 clusters, and post-truncation delay scaling. The rationale of the delay scaling step is to retain the RMS delay spread of the original untruncated channel model also in the truncated model. The applied scaling factor is approximately 1.7.

In Figure 1 we illustrate the power delay profile (PDP) of three model variants: the untruncated 24-cluster model (aka A2), the truncated 12-cluster model without delay scaling, and the final delay-scaled 12-cluster model rCDL-C1 (aka A4). In Figure 2, for the same three models, we show the measured frequency coherence (FC) as defined in TR 38.753 Section 6.4.1.

As can be seen in Figure 1, the PDP of the 24-cluster model includes a long tail consisting of very weak delay taps that have relatively small impact on the channel response or receiver performance. However, the impact of the tail to the calculated RMS delay spread is so significant that when truncating the tail, noticeable delay scaling is needed to maintain the same level of RMS delay spread.

As can be seen in Figure 2, the post-truncation delay scaling significantly alters the frequency coherence and selectivity properties of the channel. In our view, the truncated model without delay scaling retains the FC properties of the original channel better than the delay-scaled model. Thus, we believe that RMS delay spread alone is not an adequate measure to characterize the PDP of a MIMO channel and its impact on demodulation performance. Therefore, we propose to omit the additional delay scaling step in channel derivation. Alternatively, methods other than linear scaling and/or metrics other than RMS delay spread could be investigated to find more suitable delay scaling methods.

Observation #3: Post-truncation delay scaling significantly alters the frequency selectivity of the truncated rCDL-C1 model compared to the original untruncated model.

Proposal #2: We propose RAN4 to consider omitting the post-truncation delay scaling step in rCDL model derivation. Alternatively, methods other than linear scaling and/or metrics other than RMS delay spread could be investigated to find more suitable delay scaling methods.

Figure 1: PDP in rCDL-C1 derivation

Figure 2: FC in rCDL-C1 derivation

# 2.3 CDL model alignment

During the Rel-19 SCM study item it became clear that aligning the link-level CDL models between companies is particularly difficult. This is partly due to the lengthy documentation in TR 38.901 that focuses on system level and geometric model descriptions so that it takes lot of effort to clarify the intended implementation details and parameters of the link level model.

Demodulation and CSI performance simulations are complicated and challenging to align between companies even with the simple TDL channel models. When introducing CDL models into RAN4 tests, it is desirable to avoid any unnecessary additional uncertainty/ambiguity in the simulation alignment. As RAN4 channel models, CDL models will have to be implemented and simulated by a wider range of companies and institutions than before.

To help CDL channel model alignment and to minimise model ambiguity, we propose RAN4 to consider the following approach: measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4. For easy and repeatable alignment, reference statistics for each specified channel model variant should be provided by RAN4.

Observation #4: As observed during the Rel-19 SCM study item, aligning the link-level CDL models between companies is particularly difficult.

Proposal #3: To improve CDL model alignment, we propose to measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

# 2.4 PMI bias

The spatial properties of the existing link-level CDL models are constant since the cluster directions (AOA, AOD, ZOA, ZOD) are fixed (tabulated) to certain angles that do not change over time or from one simulation realization to another. As a result, the optimal PMI may remain almost constant so that a UE could in the worst case pass a PMI reporting test just by reporting a single fixed PMI.

We see two alternative approaches to improve the PMI test coverage with CDL channel models:

- The first approach is to apply time-variant mean angles (AOA, AOD, ZOA, ZOD) to the model. As a result, the angles of all clusters and all rays would be varying over time. This approach would mean that the ray-specific parameters such as antenna radiation pattern (power) responses, directional array responses, and Doppler shifts would have to be dynamically updated during the simulation. This approach was discussed during the Rel-19 SCM SI, but no progress was made. This approach requires major internal changes to the existing CDL model implementations and would probably be challenging to align between companies.

- The second and simpler approach to improve PMI test coverage is to directly apply the existing beam steering approach specified in TS 38.101-4 Annex B (B.2.3.2.3). In beam steering, the MIMO channel matrix is phase-rotated column-wise in time-variant manner so that the optimal transmit precoder beam directions vary. The beam steering approach is not based on a physical model, and it does not affect the degrees of freedom of the MIMO channel. However, this approach is readily available and can be applied on top of any MIMO channel model. Note that there is a dual-cluster variant of beam steering (B.2.3.2.3A) as well. It requires implementing two underlying MIMO channel models, and we believe its applicability to CDL models would require further study.

Observation #5: We see two alternative approaches to alleviate PMI bias with CDL models: time variant mean angles (AOA/AOD/ZOA/ZOD) and beam steering.

Proposal #4: To reduce PMI bias, we propose RAN4 to consider applying the existing simple beam steering approach specified in TS 38.101-4 Annex B (B.2.3.2.3).

# 2.5 CDL and AAV parameters

During Rel-19 SCM SI, certain high-level CDL channel parameters such as antenna array orientations and UE speed vector were agreed. Those parameters are listed in Table 6.1-2 of TR 38.753. We propose to use those parameters as baseline.

Proposal #5: We propose to use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline.

During Rel-19 SCM SI, it was discovered that antenna array virtualization (AAV) has minor impact on link-level simulations. Thus, for simplicity, we propose to consider only pass-through AAV in this work item so that Ms=Ns=1.

Proposal #6: We propose to consider only pass-through AAV in this work item.

# 2.6 PDSCH demodulation test cases

During Rel-19 SCM SI, PDSCH demodulation cases for 4TX-4RX-4L and 8TX-8RX-8L with MCS 13 were extensively simulated for alignment. We believe those simulation cases can be used as basis for new tests and performance requirements. The common simulation case parameters are listed in Table 6.1-1 of TR 38.753.

For 8TX-8RX-8L, two different Type1 precoding methods were simulated: random and fixed PMI. According to our simulations [6], an appropriately selected fixed precoder (4,0,0) results in lower SNR requirement than random precoding.

For both precoding methods, the performance of CW0 is better than the performance of CW1. With fixed precoder, per-CW performance difference can be adjusted by selecting a suitable PMI, and specifically for PMI (4,0,0) the difference is quite small. Even with random precoder, it turned out that the performance difference is noticeable (more than 7dB in our results). We believe random precoding is a suitable choice for the test case, as it is expected that it is typical for the two CWs to have different quality levels.

Regardless of the precoding choice (random or fixed), the BLER difference between the codewords could be reduced by assigning different MCSs to them. This would improve the test coverage and make the use case more realistic. Therefore, we propose RAN4 to consider assigning different MCS levels to the two codewords. For example, we could choose MCS 13 for CW0 and then seek for some suitable lower MCS for CW1, with the target to achieve approximately equal BLER for the two codewords. This approach will keep the SNR required for 70% tput at a reasonable level.

Observation #6: 8TX-8RX-8L simulation results in rCDL-C1 for both fixed and random precoding showed clear performance difference between CW0 and CW1.

Proposal #7: For 8TX-8RX-8L demod, we propose RAN4 to consider random precoding and different MCS levels for the two codewords. For example, choose MCS 13 for CW0 and seek for some suitable lower MCS for CW1.

# 2.7 PMI test cases

During Rel-19 SCM SI, PMI reporting cases were simulated for alignment for ranks 2 and 4 with 8TX-4RX setup. We believe those simulation cases can be used as basis for new tests and performance requirements. The common simulation case parameters are listed in Table 6.2-1 of TR 38.753.

According to the WID, new eType2 PMI tests should be specified. The simulation results [6] showed that some gain from eType2 over Type1 can be achieved with rank 2. However, with rank4, Type1 seemed to perform equal or better than eType2. We believe a reasonable test case for advanced codebooks should be such that gain over simpler codebooks can be achieved. Therefore, we propose that the new eType2 test cases for 8TX-4RX are specified with rank 2 only.

Observation #7: According to Rel-19 SCM simulations in CDL channels, eType2 PMI feedback provides no gain against Type1 PMI feedback with 8TX-4RX rank 4.

Proposal #8: We propose that the new eType2 test cases for 8TX-4RX will be specified with rank 2 only.

During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered. However, this case is now listed in the WID as a new scenario. It is difficult to see why this case would be of additional interest, especially with eType2. Nevertheless, RAN4 needs to investigate this case.

Observation #8: During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered. This case is listed in the WID as a new scenario.

# 2.8 Frequency generalization

The WID instructs RAN4 to address the issue of generalizing channel models of TR 38.753 to carrier frequencies other than 3.5 GHz. However, the new test cases are assumed to be for FR1 only. TR 38.901 provides some tools to adjust the high-level parameters of link-level models to better match different carrier frequencies.

Observation #9: TR 38.901 provides some tools to adjust channel models to better match with different carrier frequencies.

The main purpose of RAN4 demodulation and CSI tests is to verify the performance of baseband algorithm implementations. The channel model for a test is always just a snapshot and one arbitrary realization from an infinite pool of possible scenarios that may occur in real environments. Furthermore, RAN4 demod test channels are traditionally band-agnostic. Therefore, we believe that re-defining the MIMO channel models to exactly match the properties of different carrier frequencies is not crucial. We propose to keep testing simple and not define a range of frequency-dependent models.

Proposal #9: We propose to keep testing simple and not define a range of frequency-dependent models.

# Conclusion

In this paper we provided our views on performance requirements for FR1 SU-MIMO with spatial channel model. The following observations and proposals are made:

Observation #1: RAN1 has significantly changed the AS scaling process definition in TR 38.901 from version V18.0.0 to version V19.1.0.

Observation #2: It is feasible to follow the angular scaling updates made by RAN1 in TR 38.901 V19.1.0 in this work item.

Proposal #1: We invite other companies to re-do the angular scaling using the updated process in TR 38.901 V19.1.0 and cross-check the results.

Observation #3: Post-truncation delay scaling significantly alters the frequency selectivity of the truncated rCDL-C1 model compared to the original untruncated model.

Proposal #2: We propose RAN4 to consider omitting the post-truncation delay scaling step in rCDL model derivation. Alternatively, methods other than linear scaling and/or metrics other than RMS delay spread could be investigated to find more suitable delay scaling methods.

Observation #4: As observed during the Rel-19 SCM study item, aligning the link-level CDL models between companies is particularly difficult.

Proposal #3: To improve CDL model alignment, we propose to measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

Observation #5: We see two alternative approaches to alleviate PMI bias with CDL models: time variant mean angles (AOA/AOD/ZOA/ZOD) and beam steering.

Proposal #4: To reduce PMI bias, we propose RAN4 to consider applying the existing simple beam steering approach specified in TS 38.101-4 Annex B (B.2.3.2.3).

Proposal #5: We propose to use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline.

Proposal #6: We propose to consider only pass-through AAV in this work item.

Observation #6: 8TX-8RX-8L simulation results in rCDL-C1 for both fixed and random precoding showed clear performance difference between CW0 and CW1.

Proposal #7: For 8TX-8RX-8L demod, we propose RAN4 to consider random precoding and different MCS levels for the two codewords. For example, choose MCS 13 for CW0 and seek for some suitable lower MCS for CW1.

Observation #7: According to Rel-19 SCM simulations in CDL channels, eType2 PMI feedback provides no gain against Type1 PMI feedback with 8TX-4RX rank 4.

Proposal #8: We propose that the new eType2 test cases for 8TX-4RX will be specified with rank 2 only.

Observation #8: During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered. This case is listed in the WID as a new scenario.

Observation #9: TR 38.901 provides some tools to adjust channel models to better match with different carrier frequencies.

Proposal #9: We propose to keep testing simple and not define a range of frequency-dependent models.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- TR 38.753, Study on spatial channel model for demodulation performance requirements for NR

- RP-252957, “3GPP Work Item Description: NR demodulation performance: Phase 6”, Apple

- 3GPP TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 19), V19.1.0 (2025-09)

- R4-2505594, ”Simulation results of Spatial Channel Modelling”, MediaTek