3GPP TSG-RAN WG4 Meeting #116-bis                                                          R4-2513769

Prague, Czech Republic, 13rd – 17th October 2025

Source:	Nokia

Title:	Considerations for NR_LBCA_Sw_enh

Agenda item:	7.8.3

Document for:	Approval

Introduction

The core part of WI:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

  - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

  - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw

- Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

During RAN#109 meeting following objectives for the work item NR_LBCA_Sw_enh for was recorded [1]

Comparing this to the TR 38.768 [2] concluded in Rel-19 one can make following observations about the main changes for this new work item.

- Case 3: aggregated DL reception for SDL and FDD DL bands.

- Band combinations are limited to CA_n12A-n29A and CA_n28A-n67A.

- There is no interferer expected between two carriers.

Discussion

Observing the band combination scenarios operators have sought for the switched LB CA framework in figure 1, limiting the band combinations in this work item for the two combinations makes sense as the SDL band and FDD DL bands are just adjacent each other. Aggregated reception means that there is no antenna tuning involved and DL switches between FDD DL and aggregated DL reception. There is no UL while DL aggregated reception. Aggregated total DL bandwidths are

- CA_n12A-n29A: 28MHz

- CA_n28A-n67A: 50MHz

The latter combination doesn’t need to cover full band n28 as it is associated with n67 which is EU specific band and in EU n28 DL extends only up to 788MHz.

Figure 1 Operator provided scenarios for NR low band aggregation via switching

Implementation considerations of the WI

Considering WI objectives focusing on only band combinations in close proximity each other and no interferers between the two carriers it would make sense to receive case 3 by a shared filter covering both DL bands as shown in figure 2. Receiving the two LB carriers traditional way through dedicated DL filters by using only two antennas may be infeasible as the diplexing function for the two filters that have passbands close to each other may be challenging to realize.

Figure 2 Left, RFFE arrangement and right filtering arrangement for aggregated reception

Regarding transceiver implementation we think we may reuse analysis done for fragmented carriers [3]. CA may be received through a wide band RFFE filter by two means

- Two RX chains each carrier having its own LO

- One RX chain and one LO receiving both carriers

The two methods above are illustrated in figures 3 and 4 respectively.

Figure 3. 2 LO arrangement for aggregated reception.

Figure 4. 1 LO arrangement for aggregated reception.

With 2 DL LO solution and no interferers present the requirements set for each carrier would be relaxed from a normal LB single carrier operation. UE may well utilize the same analog BB filters as for single carrier operation. The LO for FDD carrier could run constantly without any gaps in reception while the SDL carrier would be activated semi static when no UL present.

Considering the 1 LO solution it requires more attention for setting requirements. Now the analog BB filter shall cover the whole configured aggregated BW. In Rel-19 there is max 30MHz aggregated BW defined for CA_n71(2A) which is not enough to cover total aggregated BW for CA_n28A-n67A. There should be requirements set for the PCC + SCC aggregated channel bandwidth combinations. Operation would be essentially like DL NC CA without UL present. As regards the REFSENS requirements apart from implementation losses at least image rejection impairments in connection with carrier delta power should be considered.

Considering the operation from DL carrier switching point of view it is illustrated in figure 5.

Figure 5 DL carrier switching operation. 2 DL LO left and 1 DL LO right.

In 2 DL LO operation both LOs run at constant frequency. FDD RX may run continuously while SDL RX is activated/deactivated based on the RRC pattern.

In 1 DL LO operation LO frequency must switch between configured FDD DL center frequency and aggregated channel center frequency creating a need for a switching period for both FDD DL and aggregated carriers.

2 DL LO solution may operate FDD DL continuously without interruptions

1 DL LO solution leads to switching gaps for both the FDD DL carrier and the aggregated carrier

Regarding CA_n28A-n67A, there should be requirements set for the PCC + SCC aggregated channel bandwidth combinations.

For optimum network operation it would be highly beneficial to focus on scenarios and UE implementations that supports contiguous FDD DL operation without switching interruptions.

Practical implementation considerations

While everything considered above may fulfill the intent of the WI, we are somewhat skeptical that it can be guaranteed that there are no interferers in all possible network deployments thus there is a risk generating a feature that is not generic enough to be scaled for future purposes. This is illustrated in figure 7 and 8.

Figure 6 Aggregated DL operation with 2 DL LO and with interferers.

Figure 7 Aggregated DL operation with 1 DL LO and with interferers.

Considering operation with interferes, the R20 LBCA feature is very similar to the fragmented carrier operation without UL. 2 DL LO solution provides suppression for the interferers by analog BB filter while 1 DL LO solution cannot provide any suppression in analog domain.

Looking at the frequency ranges of the band combinations proposed in this work item one can notice that there is very little guard band for UL and SDL operation. This will mean that any legacy UE operating in close vicinity of the switched LBCA UE will likely block SDL band. In the work item it is not covered how the legacy gNB should serve legacy UE and Rel-20 UE in the same cell. Both aspects need alignment with companies.

Assuming that there are no interferers between carriers doesn’t necessarily lead to a generic feature that is future proof if only 1 DL solution is considered

With a 1 LO solution including a wide RFFE filter to receive both carriers, even the contiguous FDD DL may be sensitive to inter-UE interference from the FDD UL.

2 DL LO solution can withstand interferers and is thus a future proof solution

Requirements for the aggregated DL operation should be set in such a way that 2 DL LO and continuous FDD DL operation are not excluded.

For optimum network operation it would be highly beneficial to focus on scenarios where only the SDL carrier is sensitive to cross-UE interference from the FDD UL carrier, not the contiguous FDD DL carrier.

Conclusion

In this paper, we express our views on the implementation options and what are their implications to the performance and requirements.

The following observations and proposals were made:

Observation 1: 2 DL LO solution may operate FDD DL continuously without interruptions

Observation 2: 1 DL LO solution leads to switching gaps for both the FDD DL carrier and the aggregated carrier

Observation 3: Regarding CA_n28A-n67A, there should be requirements set for the PCC + SCC aggregated channel bandwidth combinations.

Proposal 1: For optimum network operation it would be highly beneficial to focus on scenarios and UE implementations that supports contiguous FDD DL operation without switching interruptions.

Observation 4: Assuming that there are no interferers between carriers doesn’t necessarily lead to a generic feature that is future proof if only 1 DL solution is considered

Observation 5: With a 1 LO solution including a wide RFFE filter to receive both carriers, even the contiguous FDD DL may be sensitive to inter-UE interference from the FDD UL.

Observation 6: 2 DL LO solution can withstand interferers and is thus a future proof solution

Proposal 2: Requirements for the aggregated DL operation should be set in such a way that 2 DL LO and continuous FDD DL operation are not excluded.

Proposal 3: For optimum network operation it would be highly beneficial to focus on scenarios where  only the SDL carrier is sensitive to cross-UE interference from the FDD UL carrier, not the contiguous FDD DL carrier.

- References

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Telus, Beijing, China, Sep 15-18, 2025

- R4-2509202, 3GPP TR 38.768 V1.1.0, Low NR Band Carrier Aggregation via Switching

- R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, Nokia, Hefei, China, 14 – 18 October 2024.