# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513770

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	SBFD BS blocking requirements

Agenda Item:	4.19.2

Document for:	Approval

1 Introduction

During RAN4#116, BS RF requirements for SBFD have been discussed [2] and the big CR for Rel-19 core specification was endorsed for TS 38.104. In this contribution, we would like to provide some discussion and update proposal for FR2 blocking requirements.

2 Discussion

In existing FR1 requirements, the ΔOTAREFSENS is defined as difference between conducted reference sensitivity and OTA REFSENS. And correspondingly the OTA blocking requirement values are offset by ΔOTAREFSENS to conducted blocking requirement, as shown in below table.

Table 10.5.2.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
|  | EISREFSENS + x dB | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±7.5 |  |
| 5, 10, 15, 20 | EISminSENS + x dB | Wide Area BS: -43 – ΔminSENS Medium Range BS: -38 – ΔminSENS Local Area BS: -35 – ΔminSENS | ±7.5 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
|  | EISREFSENS + x dB | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±30 |  |
| 25 ,30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISminSENS + x dB | Wide Area BS: -43 – ΔminSENS Medium Range BS: -38 – ΔminSENS Local Area BS: -35 – ΔminSENS | ±30 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| NOTE 1: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

When SBFD requirements are introduced, for FR1 the conducted in-band blocking and OTA blocking are correctly implemented with the offset (ΔOTAREFSENS), as well as FR1 ACS. Meanwhile for the newly introduced in-channel adjacent subband blocking requirement for FR1 the offset is missing for OTA requirements.

Proposal 1: It is proposed to correct the FR1 OTA in-channel adjacent subband blocking requirement as in the following table.

Table 12.6.9.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-O

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | EISREFSENS + 6 dB | Wide Area BS: -28 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -34 - ΔOTAREFSENS | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

The same mistake was made for FR2, during the discussion, the coupling loss is referring to the input of the active Rx as such the antenna gain should be included. However, the OTA requirements is set at Radiated Interface Boundary (RIB), i.e. the antenna gain is missing in [1]. For WA BS the antenna gain (G) has been identified for the 28GHz band to be the range 10 to 33 dB, and the OTA reference sensitivity level for FR2-1 within the range -96 to -119 dBm dB for WA [TR 38.817-02]. When the OTA Reference sensitivity is declared, the antenna gain (G) can be derived as below equation:

G = (-86) – (EISREFSENS_50M + ΔFR2_REFSENS) (dB)

Table 10.3.3.2-1 in TR 38.817-02: G assumptions for calculating FR2 WA OTA REFSENS range

| BS class | G |  |
| --- | --- | --- |
|  | 30 GHz (24.25 – 33.4 GHz) | 45GHz (37 – 52.6 GHz) |
| WA | 10 to 33 dBi | 12 to 35 dBi |

Hence for in-channel adjacent subband blocking requirement, conducted –52 dBm should be converted to the radiated interface: -52-G = EISREFSENS_50M + 34 + ΔFR2_REFSENS

Proposal 2: It is proposed to correct the FR2-1 OTA in-channel adjacent subband blocking requirement as in the following table.

Table 12.6.9.3-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 2-O

| Frequency Range | SBFD UL subband bandwidth (MHz) | OTA wanted signal mean power (dBm) (NOTE 1) | OTA interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | EISREFSENS + 6 dB | Wide Area BS: EISREFSENS_50M + 34 + ΔFR2_REFSENS | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 60 kHz SCS, BW = X (in the unit of RB) |
|  |  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 60 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: EISREFSENS depends on the SBFD BS UL subband bandwidth as specified in clause 12.6.3. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |  |

And for OTA blocking requirement for BS type 2-O, conducted limits –44 dBm should be converted to the radiated interface: -44-G = EISREFSENS_50M + 42 + ΔFR2_REFSENS

Proposal 3: It is proposed to correct the FR2-1 OTA blocking requirement requirement as in the following table.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 42 + ΔFR2_REFSENS | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE 1: EISREFSENS and EISREFSENS_50M are given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |  |

The agreements in last meeting for FR2-1 WA BS in-band blocking is that the value and methodology shall be revisited in the maintenance. Since the requirement apply at the RIB the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and for FR2 as a biased analogue beam forming system, it is important to consider both the wanted and the interfering signal when considering a statistical worst case. From the study in the previous meeting, the offset of interfering signal power versus wanted signal at 99th percentile is comparable with existing FR2-1 OTA blocking requirements [3, 4].

### Issue 3: In-band blocking value for FR2-1 WA BS

- Agreement

  - Adopt -44 dBm as a provisional requirement. The value and the methodology to derive it shall be revisited in maintenance.

### Issue 3: In-band blocking value for FR2-1 WA BS

- Agreement

  - Adopt -44 dBm as a provisional requirement. The value and the methodology to derive it shall be revisited in maintenance.

Proposal 4: If the requirements is revised, the FR2-1 WA BS in-band blocking is defined in the range (EISREFSENS_50M + 34 + ΔFR2_REFSENS) to (EISREFSENS_50M + 42+ ΔFR2_REFSENS).

Conclusion

In this contribution we provide some update proposal for blocking requirements for SBFD.

Proposal 1: It is proposed to correct the FR1 OTA in-channel adjacent subband blocking requirement as in the following table.

Table 12.6.9.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-O

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | EISREFSENS + 6 dB | Wide Area BS: -28 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -34 - ΔOTAREFSENS | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

Proposal 2: It is proposed to correct the FR2-1 OTA in-channel adjacent subband blocking requirement as in the following table.

Table 12.6.9.3-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 2-O

| Frequency Range | SBFD UL subband bandwidth (MHz) | OTA wanted signal mean power (dBm) (NOTE 1) | OTA interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | EISREFSENS + 6 dB | Wide Area BS: EISREFSENS_50M + 34 + ΔFR2_REFSENS | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 60 kHz SCS, BW = X (in the unit of RB) |
|  |  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 60 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: EISREFSENS depends on the SBFD BS UL subband bandwidth as specified in clause 12.6.3. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |  |

Proposal 3: It is proposed to correct the FR2-1 OTA blocking requirement requirement as in the following table.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS_50M + 42 + ΔFR2_REFSENS | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE 1: EISREFSENS and EISREFSENS_50M are given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |  |

Proposal 4: If the requirements is revised, the FR2-1 WA BS in-band blocking is defined in the range (EISREFSENS_50M + 34 + ΔFR2_REFSENS) to (EISREFSENS_50M + 42+ ΔFR2_REFSENS).

References

- R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung.

- R4-2512680, Way Forward for [116][307] NR_duplex_evo_BSRF, Huawei, HiSilicon, Samsung

- R4-2511143, Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS, ZTE

- R4-2511529, On existing BS RF RX requirements for SBFD operation, Nokia