# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513773

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	SBFD BS to SBFD BS adjacent channel coexistence

Agenda Item:	7.2.4

Document for:	Approval

1 Introduction

SBFD BS to SBFD BS adjacent channel co-existence

- SBFD BS to SBFD BS adjacent channel coexistence

  - Focus on FR1 WA.

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated.

  - Focus on bands: n104, n79, n41.

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

SBFD BS to SBFD BS adjacent channel co-existence

- SBFD BS to SBFD BS adjacent channel coexistence

  - Focus on FR1 WA.

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated.

  - Focus on bands: n104, n79, n41.

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

During RAN#109, RAN4 Rel-20 5G-Adavanced package has been approved. New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 (BS RF requirement evolution) was agreed [1]. One work area is SBFD BS to SBFD BS adjacent channel coexistence.

In this contribution we provide an overview on the work objectives for this topic.

2 Discussion

2.1 General aspects

Deployments scenarios, bands, configuration requirement item and other general aspects need to be considered.

According to the WID [1], the WI objective for the work area is focusing on FR1 WA BS and on band n104, n79 and n41. It can be found the band width for these bands is around 200 ~ 700 MHz.

Table 2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Band width (MHz) |
| --- | --- | --- | --- | --- |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD | 194 |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD | 600 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | 700 |

For SBFD-SBFD coexistence, ACIR can be improved thanks to the nature of SBFD design, as shown in the figure below, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. A filter just covering operator own spectrum is required for the purpose. Wider SBFD DL sub-band (i.e. guard band) is required for filter implementation. For example for the band n104 which has 700 MHz band width, considering it is allocated to 2 ~ 3 operators, each may hold 200 MHz, 300 MHz or 400 MHz. Table 2.1-1 shows the available guard band for different operator’s spectrum.

Expected spectrum allocation and sub-band sizes

Table 2.1-1: available guard band for different operator’s spectrum

| Operator's spectrum | Achieved guard band (MHz) |  | min Ratio of transition guard/sub frequency band | Comments |
| --- | --- | --- | --- | --- |
|  | 2 operator case | 3 operator case (in middle) |  |  |
| 100 MHz | 80 | 40 | 40% | Configuration 1: 40-20-40 Configuration 2: 80-20 |
| 200 MHz | 180 | 80 | 30% | Configuration 1: SBFD 20-80+ TDD 100 |
| 300 MHz | 260 | 100 | 33.30% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100 |
| 400 MHz | 360 | N/A | 90% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100+ TDD 100 |

Proposal 1: FR1 WA BS, it is proposed to focus on the scenario that per operator spectrum is larger or equal to 100 MHz.

2.2 Co-located scenarios

Existing OTA co-location requirements are based on single worst-case scenarios, which is inherited from passive antenna system. Edge-to-edge separation d between the BS type 1-O and the co-location reference antenna isset to 0.1 m.

Figure 2.2-1: Illustration of BS type 1-O enclosure and co-location reference antenna

The BS-to-BS isolation between two co-located base stations have been evaluated from OTA measurement and OTA co-location simulation results in Rel-19 work item on NR base station RF requirement evolution. From the table below, it can be found that the coupling loss is band/frequency dependent, e.g. the difference of measurement results can be 15-20 dB for the comparison of 7 GHz and 3.5 GHz.

Table 6.4.4.2-1 in 37.941: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical Separation | SA-to-SA | M: 54 | M: 53 | M: 56 |  |
| Horizontal Separation | SA-to-SA | M: 31 | M: 34 | M: 46 |  |
|  | A-to-SA |  | M: 35 S: 55±15 | M: 51 M: 69 | M: 55 S: 70±15 |
|  | A-to-A |  |  |  | S: 77±15 |
| M: measurement results S: simulation results. |  |  |  |  |  |

Observation 1: the coupling loss is band/frequency dependent, the higher frequency larger coupling loss is observed.

The 0.1 m edge-to-edge separation is the worst assumption. It is not suitable for SBFD-SBFD co-existence since it is not a typical scenario and it will result over stringent requirements which cause design complexity and cost. As discussed in in [R4-2413283], we did some evaluation on the coupling loss. SA to SA coupling loss for n104 AAS is measured and the CDF of SA to SA coupling for Case a-2 is shown in Figure 2.2-2. Considering the robust of the massive MIMO architecture, it is not necessary to consider the percentile at 100%. It is proposed to further discuss on the suitable percentile for SBFD-SBFD coexistence case.

- Horizontal 0°

- Horizontal 0°

- Vertical 0°

- Vertical 0°

- Horizontal 180° 0°

- Horizontal 180° 0°

c) Horizontal 180° 1 m

c) Horizontal 180° 1 m

b) Vertical 0.7 m

b) Vertical 0.7 m

a) Horizontal 0° 0.4 m

a) Horizontal 0° 0.4 m

Figure 2.2-2: CDF of SA to SA coupling

Observation 2: The 0.1 m edge-to-edge separation is the worst-case assumption and it is not suitable for SBFD-SBFD co-existence.

Proposal 2: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence.

2.3 Co-existence scenarios

The grid shift assumption has been discussed extensively. It should be chose carefully since hose requirements are actually the guarantee of SBFD network performance. We feel the need to collect some measurements based on real deployment in order to cross-check the reliability of RAN4 co-ex simulation. To be specific, we extract the site location from a region where typical co-existence among operators with active NR traffic can be identified. Note that the concerning deployment is dense urban macro in C band, co-location case is also included. According to the CDF of site distance, following characteristics for such real deployment can be observed:

- Site distance is smaller than 10m @11%-tile of the CDF (co-site)

- Site distance is 10m - 20m @9%-tile of the CDF

- Site distance is 20m- 50m @25%-tile of the CDF

- Site distance is larger than 50m @55%-tile of the CDF

Observation 3: more than half of the base stations are more than 50 meters away from the interfering base stations in a real C band deployment.

In ECC report 281[2], the regulatory technical conditions for 5G MFCN operation in the band for non-synchronised TDD is analysed. It is exactly the same scenario as SBFD-SBFD co-existence, which is the base station to base station interference with non-time-aligned UL and DL transmission. Two deployment geometries are simulated, where the grid shift (separation between a victim BS and the nearest interfering BS) is 70 and 288 metres, respectively. The former case (case 1) is aligned with the assumption of 70 metre interferer-victim separation used in ECC Report 203 to derive the baseline BEM of -34 dBm/(5 MHz) for non-synchronised TDD.

Figure 16 in ECC report 281: Interferer-victim separation of 70 metres (Case 1)

Observation 4: Interferer-victim separation of 70 metres is adopted to derive ECC BEM requirement for non-synchronised TDD in the 3400-3800 MHz band.

Proposal 3: It is proposed to further discuss the interferer-victim separation or grid shift for SBFD-SBFD co-existence.

In our view eventually, the same grid shift assumption should be adopted for both TX and RX requirements derivation since both requirements are defined for SBFD BS and the same performance shall be guaranteed under the same deployment.

Proposal 4: it is proposed to adopt the same deployment scenario for both TX unwanted emission and RX blocking requirements definition for adjacent SBFD-SBFD co-existence.

2.4 RF requirements

For the deployment scenarios of SBFD BS to SBFD BS adjacent channel coexistence, additional TX unwanted emission and additional RX in-band blocking.

In Rel-19 SBFD requirement definition, the in-band blocking requirements are derived by RAN4 co-ex simulation. It is proposed to continue the simulation work for SBFD-SBFD co-existence. Simulation assumptions as in Rel-18 SBFD SI can be used as starting point.

Proposal 5: simulation assumptions as in Rel-18 SBFD SI can be used as starting point for SBFD-SBFD co-existence.

  - The scenarios to be studied are Scenario 1, defined in TR 38.858

  - The case to be studied is Case 3

FFS on frequency, AAS antenna model and transmission power.

For TX unwanted emission, we can run the co-ex simulation to get a desired BS-BS ACIR level for less than 5% UL throughput loss. And then the additional unwanted emission limit to guarantee less than 5% degradation can be derived.

Proposal 6: it is proposed to run ACIR simulation to derive additional unwanted emission limit.

For in-band blocking, the same as Rel-19, the power level of interference from the DL adjacent channel interference will be simulated. Beside the grid shift should be decided, the percentage values may need more discussion, since only the interfering signal power is considered and the possibility for the wanted signal is weak and interfering signal is strong simultaneously would be much lower. 99% was used in Rel-19 simulation.

Proposal 7: it is proposed to further discuss and decide on the percentage values of blocking CDF to be used for in-band blocking limits.

2.5 Expected feasible filtering performances

Based on the discussion in clasue 2.1 on the expected spectrum allocation. To consider the operator’s spectrum is larger than 100 MHz and the implementation aspects. 80 MHz offset could be used as starting point. The feasible attenuation for different spectrum cases are shown in Table 2.2-2.

Table 2.2-1: Filtering performance evaluation

| Operator's spectrum | Filtering performance at 80 MHz Freq offset to the operators’ spectrum edge |
| --- | --- |
| 100MHz | 30 ~ 35dB attenuation |
| 200MHz | 45 dB attenuation |
| 300MHz | 45 dB attenuation |
| 400MHz | 45 dB attenuation |

Observation 5: 80 MHz offset from the spectrum block edge for unwanted emission could be used as starting point.

Conclusion

In this contribution we provide consideration on SBFD-SBFD coexistence objective.

Proposal 1: FR1 WA BS, it is proposed to focus on the scenario that operator's spectrum is larger or equal to 100 MHz.

Observation 1: the coupling loss is band/frequency dependent, the higher frequency larger coupling loss is observed.

Observation 2: The 0.1 m edge-to-edge separation is the worst-case assumption and it is not suitable for SBFD-SBFD co-existence.

Proposal 2: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence.

Observation 3: more than half of the base stations are more than 50 meters away from the interfering base stations in a real C band deployment.

Observation 4: Interferer-victim separation of 70 metres is adopted to derive ECC BEM requirement for non-synchronised TDD in the 3400-3800 MHz band.

Proposal 3: It is proposed to further discuss the interferer-victim separation or grid shift for SBFD-SBFD co-existence.

Proposal 4: it is proposed to adopt the same deployment scenario for both TX unwanted emission and RX blocking requirements definition for adjacent SBFD-SBFD co-existence.

Proposal 5: simulation assumptions as in Rel-18 SBFD SI can be used as starting point for SBFD-SBFD co-existence.

  - The scenarios to be studied are Scenario 1, defined in TR 38.858

  - The case to be studied is Case 3

Proposal 6: it is proposed to run ACIR simulation to derive additional unwanted emission limit.

Proposal 7: it is proposed to further discuss and decide on the percentage values to be used for in-band blocking limits.

Observation 5: 80 MHz offset from the spectrum block edge for unwanted emission could be used as starting point

References

- RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

- ECC Report 281, Analysis of the suitability of the regulatory technical conditions for 5G MFCN operation in the 3400-3800 MHz band