3GPP TSG-RAN WG4 Meeting #116-bis												R4-2513779

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:			6.9.2.2

Source:	Huawei, HiSilicon

Title:	on MIMO OTA tests

Document for:	approval

# Introduction

The WF [1] in RAN4 #116 did not conclude on how to combine the CTMT for MIMO performance metric. This contribution provides a new proposal.

# Discussion

Initial measurement on UMa channels from [2] is copied below and shows that the ratios between 10%-ile, 50%-ile and 90%-ile are not 9:5:1. CTMT measurement for UMi channels may manifest different ratios.

Table 1: Comparison of CTMT with CDF values from individual TP curves

| CDF %-ile | CDF of TP Curves [Mbps] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CTMT [Mbps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DML-Left |  |  |  | DMP |  |  |  | DMSU |  |  |  | DML-Right |  |  |  | AVG |  |
|  | 0° | 90° | 180° | 270° | 0° | 90° | 180° | 270° | 0° | 90° | 180° | 270° | 0° | 90° | 180° | 270° |  |  |
| 10 | 62.7 | 63.2 | 59.9 | 56.8 | 57.9 | 63.2 | 59.2 | 63.2 | 48.7 | 49.5 | 45.7 | 54.0 | 64.8 | 53.5 | 69.2 | 69.1 | 58.8 | 56.5 |
| 50 | 157.4 | 157.3 | 169.9 | 165.6 | 157.1 | 157.7 | 157.2 | 161.0 | 125.0 | 150.3 | 148.6 | 131.6 | 173.6 | 157.3 | 156.5 | 164.8 | 155.7 | 156.4 |
| 90 | 381.2 | 435.4 | 429.3 | 378.5 | 395.1 | 382.7 | 393.2 | 392.7 | 431.1 | 439.3 | 378.3 | 407.0 | 439.3 | 416.8 | 378.3 | 426.4 | 406.5 | 420.7 |

Perhaps CTMT should not be combined into a single value. It may be more practical to specify CTMT values as a set at 10%-ile, 50%-ile and 90%-ile as MIMO OTA performance metrics. In this way, performances in various scenarios are clear and transparent to operators or customers. For example, 10%-ile represents cell edge or deep fade cases, 90%-ile shows the best while 50%-ile denote typical cases. This is better than combining them into a single value and enables operators or customers to make an infomed choice on device performance.

The issue with combining three values at 10%-ile, 50%-ile and 90%-ile into a single metric was highlighted in [2]. In such case, it is impossible to distinguish the performance of two devices with a similar combined metric. In contrast, it is relatvely straightforward to tell the performance differences between two devices using the throughput values at 10%-ile, 50%-ile and 90%-ile.

Proposal 1: use three separate values of 10%-ile, 50%-ile and 90%-ile from a combined data pool of UMa and UMi channels as a set to represent MIMO OTA performance.

In addition, a device should meet or be above all three values in order to be 3GPP MIMO performance compliant.

# Conclusion

This contribution provides the following proposal.

Proposal 1: use three separate values of 10%-ile, 50%-ile and 90%-ile from a combined data pool of UMa and UMi channels as a set to represent MIMO OTA performance.

# Reference

- R4-2512693 Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3

- R4-2509913 On TMT and CTMT metric definitions