3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513781

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Inputs to 4Tx based on 26dBm PAs and PC1.5 for FDD

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.3	HPUE [NR_UE_RF_Ph5-Core]

Document for:	Approval

# 1	Background

In RAN#109, a new 5GA Release 20 WI [1] was agreed for FR1 enhancements and focusses on HPUE for FWA and FDD. In this contribution, based on our work on PC2 FDD and NTN HPUE, we provide our preliminary views on feasibility.

# Discussion

## 2.1 HPUE objectives in R20 new WI: UE RF enhancements for NR FR1

The new Release 20 WI UE RF enhancements for NR FR1 [1] emphasizes on extended HPUE support, the corresponding objectives are copied below in italics.

High power UE (HPUE) for NR single carrier operation

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

  - This work is to start in 2026 Q4

The PC1.5 1Tx and PC1 2Tx objectives will only begin in Q4 2026 thus, in this contribution we provide our preliminary input on the first two HPUE cases.

## 2.2 4Tx based on 26dBm PAs

PC1 4Tx based on 4x26dBm PAs has already been introduced for NTN in 38.101-5 in Release 19 [3] and in [2], we provided our input on MPR depending on ACLR and 4TX antenna isolation.

The best case specified MPR in [3] is 0.5dB for inner DFT-s-OFDM QPSK is 0.5dB based on 34dB ACLR target and ≥15dB antenna isolation (any 1 antenna to the other 3 antennas). Our data in [2] showed that at least 15dB antenna isolation is needed to target 0dB MPR for inner and for large channel bandwidths, there is no margin to go beyond 31dBm at least for 34dB ACLR and ≥15dB antenna isolation.

Based on our measurements, to enable up to 32dBm for DFT-s-OFDM QPSK inner, it would require that ACLR is relaxed to 31dBc AND antenna isolation is ≥20dB. If the latter may be expected in FWA implementations, the current ACLR is 31dBc for PC1.5 and 37dB for PC1. If we understand that based on coexistence simulations for bands >2.5GHz the 32dBm 4Tx can be <37dB, it is yet unclear that it can be relaxed down to 31dBc.

Since Band n41 is one of the example bands, it requested A-MPR for NS_04 and NS_47 should be evaluated

Proposal on 4x26dBm PA power class:

- Assuming ≥20dB antenna isolation and 34dB ACLR target, 31dBm PC1 can be reused and ACLR specified at 37dB for <2.5GHz and 34dB for >2.5GHz. No need to create a new power class

- 32dBm can only be reached if coexistence simulations >2.5GHz show that 31dB ACLR is sufficient, and antenna isolation is ≥20dB.

  - Even so, creating a new power class is for 1dB improvement for a limited set of allocation seems unnecessary and alternatively, considering PC1 boosting for inner DFT-s-OFDM QPSK seems less complex.

- A-MPR for NS_04 and NS_47 should be evaluated for Band n41

## 2.3 2Tx PC1.5 for FDD

2Tx PC1.5 is already specified for TDD bands including Band n39 which is just below 2GHz. Given that the ACLR of 31dB is based on Band 41 coexistence and the UE MPR assumes 10dB isolation, it is safe to assume the same ACLR and antenna isolation for FDD bands which UL is >1.7GHz. However, for lower bands, it is important to validate the ACLR target and the antenna isolation.

Since n1 and n25 are example bands:

- NS_05, NS_05U, NS_48 and NS_49 PC1.5 A-MPR should be evaluated for Band n1

- NS_03 and NS_03U PC1.5 A-MPR should be evaluated for Band n25

- Like for PC2 we can assume that NS_100 is not required for Bands n1 and n25 HPUE.

Proposal on 2Tx PC1.5 for FDD definition:

- 31dB ACLR and 10dB antenna isolation used for 2Tx PC1.5 TDD bands can be reused for FDD bands which UL is >1.7GHz

- FFS if 31dB ACLR and 10dB antenna isolation used for 2Tx PC1.5 TDD bands can be reused for FDD bands which UL is <1.7GHz

- NS_05, NS_05U, NS_48 and NS_49 PC1.5 A-MPR should be evaluated for Band n1

- NS_03 and NS_03U PC1.5 A-MPR should be evaluated for Band n25

- Like for PC2 we can assume that NS_100 is not required for n1 and n25 HPUE.

Separately from the reuse of the PC1.5 TDD bands for MPR and ACLR, the other aspect related to PC1.5 FDD is the need to evaluate the RSD as it was done for PC2. However, we believe that similar to what was done for inter-band MSD for HPUE, there are means to simplify the RSD calculations by using a fixed set of parameters like, interference increase from PC3 to PC1.5 and a fixed main path to diversity path imbalance. Also, only 2Tx RSD is needed.

Proposal on 2Tx PC1.5 FDD RSD:

- Only 2Tx RSD is specified for PC1.5: it should be clarified that only 2Tx PC1.5 will apply to FDD bands in the band power class table.

- RSD simplification is investigated in a similar way than for MSD simplification based on:

  - Fixed interference level increase

  - Fixed Main to diversity imbalance (FFS if 6dB used for MSD simplification is adequate or higher value is needed.

  - And directly derived from PC3 1Tx REFSENS or PC2 2Tx RSD.

- Given the large duplex distance, RSD should be minimal for Band n1 but important for Band n25 that has significant de-sense for CBW>25MHz (both from REFSENS and PC2 2Tx RSD).

# Conclusion

In this contribution, we provide our preliminary views on 6x26dBm HPUE and 2Tx PC1.5 for FDD based on our experience on 4Tx PC1 for NTN, PC1.5 2Tx for TDD and PC2 1/2Tx for FDD.

Proposal on 4x26dBm PA power class:

- Assuming ≥20dB antenna isolation and 34dB ACLR target, 31dBm PC1 can be reused and ACLR specified at 37dB for <2.5GHz and 34dB for >2.5GHz. No need to create a new power class

- 32dBm can only be reached if coexistence simulations >2.5GHz show that 31dB ACLR is sufficient, and antenna isolation is ≥20dB.

  - Even so, creating a new power class is for 1dB improvement for a limited set of allocation seems unnecessary and alternatively, considering PC1 boosting for inner DFT-s-OFDM QPSK seems less complex.

- A-MPR for NS_04 and NS_47 should be evaluated for Band n41

Proposal on 2Tx PC1.5 for FDD definition:

- 31dB ACLR and 10dB antenna isolation used for 2Tx PC1.5 TDD bands can be reused for FDD bands which UL is >1.7GHz

- FFS if 31dB ACLR and 10dB antenna isolation used for 2Tx PC1.5 TDD bands can be reused for FDD bands which UL is <1.7GHz

- NS_05, NS_05U, NS_48 and NS_49 PC1.5 A-MPR should be evaluated for Band n1

- NS_03 and NS_03U PC1.5 A-MPR should be evaluated for Band n25

- Like for PC2 we can assume that NS_100 is not required for n1 and n25 HPUE.

Proposal on 2Tx PC1.5 FDD RSD:

- Only 2Tx RSD is specified for PC1.5: it should be clarified that only 2Tx PC1.5 will apply to FDD bands in the band power class table.

- RSD simplification is investigated in a similar way than for MSD simplification based on:

  - Fixed interference level increase

  - Fixed Main to diversity imbalance (FFS if 6dB used for MSD simplification is adequate or higher value is needed.

  - And directly derived from PC3 1Tx REFSENS or PC2 2Tx RSD.

- Given the large duplex distance, RSD should be minimal for Band n1 but important for Band n25 that has significant de-sense for CBW>25MHz (both from REFSENS and PC2 2Tx RSD).

# References

[1] RP-252901 New WID: UE RF enhancements for NR FR1, Phase 5, vivo, RAN#109

[2] R4-2511320 MPR and antenna isolation evaluation for 4Tx PC1, Skyworks Solutions Inc., RAN4#116

[3] R4-2511790 Draft CR to TS 38.101-5 for section 6.1 and 6.2, Samsung, RAN4#116