3GPP TSG-RAN WG4 Meeting #116bis	R4-2513782

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	4.8.1

Source:	MediaTek Inc.

Title:	Discussion on remaining issues for low NR band CA via switching

Document for:	Discussion

# Introduction

Based on the WF [1], we provide our perspectives on the unresolved issues in this contribution.

- Discussion

According to the WF [1], RAN4 agreed to specify DL measurement channels.

Issue 1: Downlink reference measurement channels Agreement: - RAN4 can continue to discuss how to specify the DL RMC for this feature as part of the Performance phase of the work item

We provide a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE supporting LBCA. In our DL RMC design, we try to reuse most of parameter values in existing TS38.101-1 RMCs and just make some necessary changes when considering new features introduced in LBCA.

Proposal 1: Try to reuse most of parameter values in existing TS38.101-1 RMCs when considering LBCA:

- PDCCH length: Two symbols PDCCH in DL slots with PDSCH

- PDSCH mapping type: Type A

- Periodicity of TRS: 20ms

- Number of additional DMRS: 2

There are several new parameters for RMC to be defined in new RMC when considering LBCA, such as LBCA-SwitchingPattern, LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-Duration-SCelltoPCell.

| Figure 1. FDD to SDL switch with the switching gap on the switched-from carrier | Figure 2. SDL to FDD switch with the switch occurring before the start of the UL slot |
| --- | --- |

Based on TS38.768, the actual switching gap length for FDD to SDL shall be larger than “Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period” and the the actual switching gap length for SDL to FDD shall be larger than “Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period”. By considering the worst case of switching period Tswitch with 140us. We propose to use 3 symbols for LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-Duration-SCelltoPCell.

Proposal 2: By considering the worst case of switching period Tswitch with 140us. For DL RMC, we propose to use 3 symbols for LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-Duration-SCelltoPCell.

For LBCA-SwitchingPattern, following RAN1 agreements, 40ms periodicity with bitmap can be configured as per UE based RRC configuration.  We propose to use ‘0000000000, 1111111111, 0000000000, 1111111111’ which can facilitate 20ms transmission of SSB and TRS.

Proposal 3: To facilitate 20ms transmission of SSB and TRS, we propose to set the LBCA-SwitchingPattern as ‘0000000000, 1111111111, 0000000000, 1111111111’ in DL RMC.

Based on proposals 1, 2 and 3, we provide the illustration of reference channel in Figure 3. Also, the reference channel parameters and detail for Fixed reference channel are provided in Table 1 and 2, respectively.

Figure 3. Illustration for reference channel

Proposal 4: We propose RMC Table 1 and 2 for DL CA switching test configuration

Table 1. reference channel parameters for LBCA

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Switching Pattern for Low-band CA | LBCA-SwitchingPattern |  | A bitmap of slots | 0000000000, 1111111111, 0000000000, 1111111111 |
|  | Period of Switching Pattern |  | Slots | 40 |
|  | Slots on the PCell |  |  | A bit value of ‘0’ in LBCA-SwitchingPattern |
|  | Slots on the SCell |  |  | A bit value of ‘1’ in LBCA-SwitchingPattern |
| DL slots on the PCell | Slot Location |  |  | mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 9, 22, 23, 24, 25, 26, 27, 28, 29 |
|  | CORESET frequency domain allocation |  |  | Full BW |
|  | CORESET time domain allocation |  |  | 2 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  |  | Type A |
|  | PDSCH start symbol index (S) |  |  | 2 |
|  | Number of consecutive PDSCH symbols (L) |  |  | 12 for mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 9 for mod(i, 40) = 9, 29 |
|  | PDSCH PRB bundling |  | PRBs | 2 |
|  | Dynamic PRB bundling |  |  | false |
|  | Overhead value for TBS determination |  |  | 0 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
|  | DMRS type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | FDM between DMRS and PDSCH |  |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) |  | Slots | 2 for mod(i, 40) = 2, 3, 4, 5, 6, 7, 22, 23, 24, 25, 26, 27 13 for mod(i, 40) = 8, 9, 28, 29 |
|  | Number of HARQ Processes |  |  | 4 |
|  | CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  |  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  |  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 0 for CSI-RS resource 1 and 2 1 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  | TCI state #0 |
|  | PTRS configuration |  |  | PTRS is not configured |
| DL slots on the PCell | Slot Location |  |  | mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 19, 32, 33, 34, 35, 36, 37, 38, 39 |
|  | CORESET frequency domain allocation |  |  | Full BW |
|  | CORESET time domain allocation |  |  | 2 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  |  | Type A |
|  | PDSCH start symbol index (S) |  |  | 2 |
|  | Number of consecutive PDSCH symbols (L) |  |  | 12 for mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 9 for mod(i, 40) = 19, 39 |
|  | PDSCH PRB bundling |  | PRBs | 2 |
|  | Dynamic PRB bundling |  |  | false |
|  | Overhead value for TBS determination |  |  | 0 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
|  | DMRS type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | FDM between DMRS and PDSCH |  |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) |  | Slots | 8 for mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 19, 32, 33, 34, 35, 36, 37, 38 |
|  | Number of HARQ Processes |  |  | 4 |
|  | CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  |  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  |  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  | TCI state #0 |
|  | PTRS configuration |  |  | PTRS is not configured |
| NOTE 1: Slot i is slot index per 4 frames |  |  |  |  |

Table 2. Fixed reference channel for receiver requirements (SCS 15 kHz, FDD, QPSK 1/3) under LBCA

| Parameter |  | Unit | Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| Subcarrier spacing |  | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 |
| Subcarriers per resource block |  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| PCell | Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 12296 | 14088 |
|  | For slot i where mod(i, 40) = 9, 29 | Bits | 672 | 1128 | 2280 | 3496 | 4608 | 5760 | 6912 | 8064 | 9224 |
|  | Transport block CRC |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
|  | For slot i where mod(i, 40) = 9, 29 | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
|  | LDPC base graph |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | For slot i where mod(i, 40) = 9, 29 |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |
|  | For slot i where mod(i, 40) = 9, 29 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | Bits | 3240 | 5400 | 5400 | 11232 | 17064 | 22896 | 28728 | 40608 | 34560 |
|  | For slot i where mod(i, 40) = 9, 29 | Bits | 2160 | 3600 | 7488 | 11376 | 15264 | 19152 | 23040 | 27072 | 31104 |
| SCell | Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 12296 | 14088 |
|  | For slot i where mod(i, 40) = 19, 39 | Bits | 672 | 1128 | 2280 | 3496 | 4608 | 5760 | 6912 | 8064 | 9224 |
|  | Transport block CRC |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
|  | For slot i where mod(i, 40) = 19, 39 | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
|  | LDPC base graph |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | For slot i where mod(i, 40) = 19, 39 |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |
|  | For slot i where mod(i, 40) = 19, 39 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | Bits | 3240 | 5400 | 5400 | 11232 | 17064 | 22896 | 28728 | 40608 | 34560 |
|  | For slot i where mod(i, 40) = 19, 39 | Bits | 2160 | 3600 | 7488 | 11376 | 15264 | 19152 | 23040 | 27072 | 31104 |
| Max. Throughput averaged over 4 frame |  | Mbps | 0.756 | 1.283 | 2.586 | 3.933 | 5.299 | 6.674 | 8.044 | 9.413 | 10.784 |

Issue 2: Uplink reference measurement channels

Agreement:

-	RAN4 can further discuss whether an UL RMC is needed for this feature

Though in RAN1 agreement, the uplink transmission timing shall refer to DL slot timing. However, for the uplink transmission conformance test, the test equipment never acquires a triggering input from UE side to align UE side DL slot boundary. The uplink transmission timing shall be UE self-contained that there is no need to specify an UL RMC.

Proposal 5: There’s no need to specify UL RMC

In last RAN4 meeting, CA_n12-n29, CA_n14-n29, CA_n28-n67 and CA_n29-n71 were also introduced in addition to CA_n5-n29 assuming there will be no further basket WI needed in the future release. However the UE coexistence requirements are not specified to protect adjacent UE among inter-UEs in the dedicated bands. For example, the n12 single carrier UE will be operated at a time and the low band CA via switching capable UE will be operated in n29 to receive the DL signaling at the same time. Then, the n12 aggressor UE will be interfered to n29 operating LBCA via switching capable UE. RAN4 need to define the UE-coexistence requirements to consider this coexistence scenarios for CA_n12-n29 and CA_n28-n67 CA since 3GPP did not specify the coexistence requirements to protect the victim LBCA capable UE from the single carrier aggressor UE.

Proposal 6: RAN4 need to define the UE-coexistence requirements to consider this coexistence scenarios for CA_n12-n29 and CA_n28-n67

Proposal 7: RAN4 agree on the CR in [2] R4-2513780 capturing proposal 1~6

# Conclusion

In this contribution, we propose as following:

Proposal 1: Try to reuse most of parameter values in existing TS38.101-1 RMCs when considering LBCA:

- PDCCH length: Two symbols PDCCH in DL slots with PDSCH

- PDSCH mapping type: Type A

- Periodicity of TRS: 20ms

- Number of additional DMRS: 2

Proposal 2: By considering the worst case of switching period Tswitch with 140us. For DL RMC, we propose to use 3 symbols for LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-Duration-SCelltoPCell.

Proposal 3: To facilitate 20ms transmission of SSB and TRS, we propose to set the LBCA-SwitchingPattern as ‘0000000000, 1111111111, 0000000000, 1111111111’ in DL RMC.

Proposal 4: We propose RMC Table 1 and 2 for DL CA switching test configuration

Table 1. reference channel parameters for LBCA

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Switching Pattern for Low-band CA | LBCA-SwitchingPattern |  | A bitmap of slots | 0000000000, 1111111111, 0000000000, 1111111111 |
|  | Period of Switching Pattern |  | Slots | 40 |
|  | Slots on the PCell |  |  | A bit value of ‘0’ in LBCA-SwitchingPattern |
|  | Slots on the SCell |  |  | A bit value of ‘1’ in LBCA-SwitchingPattern |
| DL slots on the PCell | Slot Location |  |  | mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 9, 22, 23, 24, 25, 26, 27, 28, 29 |
|  | CORESET frequency domain allocation |  |  | Full BW |
|  | CORESET time domain allocation |  |  | 2 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  |  | Type A |
|  | PDSCH start symbol index (S) |  |  | 2 |
|  | Number of consecutive PDSCH symbols (L) |  |  | 12 for mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 9 for mod(i, 40) = 9, 29 |
|  | PDSCH PRB bundling |  | PRBs | 2 |
|  | Dynamic PRB bundling |  |  | false |
|  | Overhead value for TBS determination |  |  | 0 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
|  | DMRS type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | FDM between DMRS and PDSCH |  |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) |  | Slots | 2 for mod(i, 40) = 2, 3, 4, 5, 6, 7, 22, 23, 24, 25, 26, 27 13 for mod(i, 40) = 8, 9, 28, 29 |
|  | Number of HARQ Processes |  |  | 4 |
|  | CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  |  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  |  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 0 for CSI-RS resource 1 and 2 1 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  | TCI state #0 |
|  | PTRS configuration |  |  | PTRS is not configured |
| DL slots on the PCell | Slot Location |  |  | mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 19, 32, 33, 34, 35, 36, 37, 38, 39 |
|  | CORESET frequency domain allocation |  |  | Full BW |
|  | CORESET time domain allocation |  |  | 2 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  |  | Type A |
|  | PDSCH start symbol index (S) |  |  | 2 |
|  | Number of consecutive PDSCH symbols (L) |  |  | 12 for mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 9 for mod(i, 40) = 19, 39 |
|  | PDSCH PRB bundling |  | PRBs | 2 |
|  | Dynamic PRB bundling |  |  | false |
|  | Overhead value for TBS determination |  |  | 0 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
|  | DMRS type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | FDM between DMRS and PDSCH |  |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) |  | Slots | 8 for mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 19, 32, 33, 34, 35, 36, 37, 38 |
|  | Number of HARQ Processes |  |  | 4 |
|  | CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  |  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  |  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  |  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  | TCI state #0 |
|  | PTRS configuration |  |  | PTRS is not configured |
| NOTE 1: Slot i is slot index per 4 frames |  |  |  |  |

Table 2. Fixed reference channel for receiver requirements (SCS 15 kHz, FDD, QPSK 1/3) under LBCA

| Parameter |  | Unit | Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| Subcarrier spacing |  | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 |
| Subcarriers per resource block |  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| PCell | Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 12296 | 14088 |
|  | For slot i where mod(i, 40) = 9, 29 | Bits | 672 | 1128 | 2280 | 3496 | 4608 | 5760 | 6912 | 8064 | 9224 |
|  | Transport block CRC |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
|  | For slot i where mod(i, 40) = 9, 29 | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
|  | LDPC base graph |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | For slot i where mod(i, 40) = 9, 29 |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |
|  | For slot i where mod(i, 40) = 9, 29 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 0, 1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 2, 3, 4, 5, 6, 7, 8, 22, 23, 24, 25, 26, 27, 28 | Bits | 3240 | 5400 | 5400 | 11232 | 17064 | 22896 | 28728 | 40608 | 34560 |
|  | For slot i where mod(i, 40) = 9, 29 | Bits | 2160 | 3600 | 7488 | 11376 | 15264 | 19152 | 23040 | 27072 | 31104 |
| SCell | Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 12296 | 14088 |
|  | For slot i where mod(i, 40) = 19, 39 | Bits | 672 | 1128 | 2280 | 3496 | 4608 | 5760 | 6912 | 8064 | 9224 |
|  | Transport block CRC |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
|  | For slot i where mod(i, 40) = 19, 39 | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
|  | LDPC base graph |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | For slot i where mod(i, 40) = 19, 39 |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
|  | Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |
|  | For slot i where mod(i, 40) = 19, 39 | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
|  | For slot i where mod(i, 20) = 10, 11 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | For slot i where mod(i, 40) = 12, 13, 14, 15, 16, 17, 18, 32, 33, 34, 35, 36, 37, 38 | Bits | 3240 | 5400 | 5400 | 11232 | 17064 | 22896 | 28728 | 40608 | 34560 |
|  | For slot i where mod(i, 40) = 19, 39 | Bits | 2160 | 3600 | 7488 | 11376 | 15264 | 19152 | 23040 | 27072 | 31104 |
| Max. Throughput averaged over 4 frame |  | Mbps | 0.756 | 1.283 | 2.586 | 3.933 | 5.299 | 6.674 | 8.044 | 9.413 | 10.784 |

Proposal 5: There’s no need to specify UL RMC

Proposal 6: RAN4 need to define the UE-coexistence requirements to consider this coexistence scenarios for CA_n12-n29 and CA_n28-n67

Proposal 7: RAN4 agree on the CR in [2] R4-2513780 capturing proposal 1~6

# Reference

- R4-2511862, WF on UE RF requirements for LB CA via switching, RAN4#116, Apple

- R4-2513780, (NR_LBCA_Sw-Core) Missing requirements for low NR band CA via switching in TS38.101-1, RAN4#116bis, MediaTek