3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513790

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	On BS conformance requirements for SBFD-capable BS

Agenda item:			6.13.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In RAN4#116, CR was agreed to TS38.104 to introduce BS RF requirement for SBFD-capable BS, which has been approved in the CR package [RP‑252418] in RAN#109. In this contribution, we would like to provide viewpoints on BS conformance requirements for SBFD-capable BS.

# 2. Discussion

## 2.1 Manufacture declaration

Based on previous discussion, the following WF [3][4] is agreed for manufacture declaration for SBFD-capable BS.

##### Issue 5-1-3: Declaration of supported SBFD configuration Agreement: - Declaration of supported SBFD configuration: - At least the following declarations shall be captured in TS 38.141-1/-2: - TX power for SBFD; - Bands supported for SBFD; - Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; - SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. - Other declarations are not precluded ##### Issue 10: BS conformance testing - manufacturer declaration - Way forward - FFS in next meeting - All the declarations in the WF is needed - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS. - It is necessary to add the definitions of output power requirements for SBFD capable BS to TS 38.141-1 and TS 38.141-2. - It is necessary to add the definition of total power dynamic range requirement for SBFD capable BS to TS 38.141-1 and TS 38.141-2. - The definitions of EVM and averaged EVM for SBFD capable BS need to be added to TS 38.141-1 and TS 38.141-2. - The OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS. - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS. - The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2.

Accordingly, the following new manufacture declaration related to SBFD-capable BSs shall be introduced.

Proposal 1: The following new manufacture declaration related to SBFD-capable BSs shall be introduced:

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xx | Operating bands for SBFD operation | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in SBFD operation. Declarations shall be made per antenna connector for TAB connector for BS type 1-H. |  | x |
| D.xx | NR supported transmission bandwidth configurations for SBFD DL subband(s) and UL subbands for SBFD operation | NR supported transmission bandwidth configurations for SBFD DL subband(s) and UL subbands for SBFD operation. Declared per supported operating band, per antenna connector for TAB connector for BS type 1-H. |  | x |
| D.xx | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) for SBFD operation | Conducted rated carrier output power, per single band connector or multi-band connector for SBFD operation. Declared per supported operating band, per antenna connector for TAB connector for BS type 1-H. (Note 1, 2, 7) | x | x |
| D.xx | Rated total output power (Prated,t,AC, or Prated,t,TABC) for SBFD operation | Conducted total rated output power for SBFD operation. Declared per supported operating band, per antenna connector for TAB connector for BS type 1-H. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1, 2, 7) | x | x |

## 2.2 Measurement uncertainties and test requirements

For SBFD-capable BS, for all existing requirements, the existing maximum test system uncertainty can be reused.

Proposal 2: For SBFD-capable BS requirement which has corresponding existing requirements for legacy TDD BS, the existing maximum test system uncertainty (e.g., clause 4.1.2 of TS 38.141-1) can be reused.

For the newly defined SBFD-specific requirement, including OTA transient period between SBFD and non-SBFD, In-channel adjacent subband leakage and In-channel adjacent subband blocking requirements, the following corresponding maximum test system uncertainty can be reused.

Proposal 3: For newly defined SBFD-specific requirement, the maximum test system uncertainty shall be specified as:

- OTA transient period between SBFD and non-SBFD shall follow the uncertainty for transmitter transient period, i.e., NA.

- In-channel adjacent subband leakage shall follow the uncertainty for ACLR in which BW should be replaced by DL subband

- In-channel adjacent subband blocking shall follow the uncertainty for in-band blocking

# 3. Conclusion

In this contribution, we have provided we would like to provide viewpoints on BS conformance requirements for SBFD-capable BS:

Proposal 1: The following new manufacture declaration related to SBFD-capable BSs shall be introduced:

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xx | Operating bands for SBFD operation | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in SBFD operation. Declarations shall be made per antenna connector for TAB connector for BS type 1-H. |  | x |
| D.xx | NR supported transmission bandwidth configurations for SBFD DL subband(s) and UL subbands for SBFD operation | NR supported transmission bandwidth configurations for SBFD DL subband(s) and UL subbands for SBFD operation. Declared per supported operating band, per antenna connector for TAB connector for BS type 1-H. |  | x |
| D.xx | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) for SBFD operation | Conducted rated carrier output power, per single band connector or multi-band connector for SBFD operation. Declared per supported operating band, per antenna connector for TAB connector for BS type 1-H. (Note 1, 2, 7) | x | x |
| D.xx | Rated total output power (Prated,t,AC, or Prated,t,TABC) for SBFD operation | Conducted total rated output power for SBFD operation. Declared per supported operating band, per antenna connector for TAB connector for BS type 1-H. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1, 2, 7) | x | x |

Proposal 2: For SBFD-capable BS requirement which has corresponding existing requirements for legacy TDD BS, the existing maximum test system uncertainty (e.g., clause 4.1.2 of TS 38.141-1) can be reused.

Proposal 3: For newly defined SBFD-specific requirement, the maximum test system uncertainty shall be specified as:

- OTA transient period between SBFD and non-SBFD shall follow the uncertainty for transmitter transient period, i.e., NA.

- In-channel adjacent subband leakage shall follow the uncertainty for ACLR in which BW should be replaced by DL subband

- In-channel adjacent subband blocking shall follow the uncertainty for in-band blocking

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2512665, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE, RAN#116.

[4] R4-2512680, “Way Forward for [116][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, Samsung, RAN#116.

[5] R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, RAN#116.