3GPP TSG-RAN WG4 Meeting #116bis		R4-2513798

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:		4.19.3

Source:		Samsung

Title:	Discussion on RRM core maintenance of Rel-19 SBFD

Document for:		Discussion

- Introduction

In last RAN4#116 meeting, RAN4 finalized the core part discussion on RRM core requirements for Rel-19 SBFD. There are no open issues. According to further conclusion in RAN1 and RAN2, in this contribution, we provide our view of the RRM core maintenance of Rel-19 SBFD.

- Discussion

In last RAN4#116 meeting, RAN4 finalized the RRM core requirements for Rel-19 SBFD. All agreements are captured in latest WF [1]. There is one item related to RAN1 final conclusion, it is captured in [1] as below:

#### Issue 1-1-1: Requirement applicability for basic UE operation - Agreement - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

In last RAN1#122 meeting, for the discussion of SBFD UE feature list, RAN1 achieve the consensus in [2]:

The simultaneous DL reception in 2 DL subbands, it is a basic UE feature, which means all SBFD UEs shall support simultaneous DL reception in 2 DL subbands. There is no such basic UE operation as above. Thus, RAN4 no need to add such corresponding requirements for UE not supporting simultaneous DL reception in 2 DL subbands. RAN4 just keep current core requirements for UE supporting simultaneous DL reception in 2 DL subbands.

Proposal 1: No need to add such corresponding requirements for UE not supporting simultaneous DL reception in 2 DL subbands in RAN4. The core requirements in current spec are applied to all SBFD UEs.

In this RAN4#116bis meeting, RAN4 received an LS from RAN2 in [3]. In the LS, RAN2 evaluated the specification impact to the scenario of simultaneous configuration of SBFD and inter-band NR-DC:

RAN2 would like to thank RAN1 for the LS on simultaneous configuration of SBFD and DC in R1-2504858. RAN2 has evaluated the specification impact to the scenario of simultaneous configuration of SBFD and inter band NR-DC as indicated in the LS, and made the following agreement: RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion..

The progress is that RAN1 send a reply LS to RAN2 to capture RAN1 evaluation of the specification impact to the scenario of simultaneous configuration of SBFD and inter-band NR-DC in [4]. In RAN1 reply LS, RAN1 has no strong view to support the scenario of simultaneous configuration of SBFD and inter-band NR-DC.

Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly. It is up to RAN2 to decide whether or not to support simultaneous configuration of SBFD and DC.

In last RAN2#131 meeting, RAN2 approved one LS related to DC support in [3].

In RAN2#131 discussion:

Proposal 1 RAN2 to conclude that a. Whether SBFD and DC can be supported is up to RAN4 decision b. The issue doesn’t block the completion of the WI in RAN2 c. Further spec changes in RAN2 can be triggered based on RAN4 decision. Discussions - InterDigital think from R2 point of view we can support SBFD + DC. QC agree. - Samsung, CATT, Xiaomi agree with the proposal. - QC think we can send LS to RAN4. LG E, Xiaomi agree. - CATT, HW think R4 is discussing so no need to send LS. RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion. [Post131][223][SBFD] LS on simultaneous configuration of SBFD and DC (ZTE) Intended outcome: Approve the LS Deadline: Short

Agreements on other aspects RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion..

Now, the LS in [3] is replied to RAN1 and RAN4. No final decision was made in RAN2. From RRM perspective, in order to support DC, we need to add some RRM requirements for PSCell for SBFD if the SBFD is deployed in the DC. Take an example, in current RLM/BFD/CBD requirements, there are separate RRM requirements for PSCell. In SBFD requirements for PCell, RAN4 achieved to use NR-U approach for CSI-RS based L1 measurements. If it is decided to support the case of simultaneous configuration of SBFD and inter-band NR-DC, RAN4 to discuss whether/how to use enhance the RRM requirements for PSCell measurement of CSI-RS based L1 measurements. E.g. including RLM and BFD for PSCell measurement of CSI-RS based L1 measurements if SBFD operation is in DC.

Proposal 2: If it is decided to support the case of simultaneous configuration of SBFD and inter-band NR-DC, RAN4 to discuss whether/how to use enhance the RRM requirements for PSCell measurement of CSI-RS based L1 measurements. E.g. including RLM and BFD for PSCell measurement of CSI-RS based L1 measurements if SBFD operation is in DC.

- Conclusion

In this contribution, we provide our views of RRM core maintenance of Rel-19 SBFD:

Proposal 1: No need to add such corresponding requirements for UE not supporting simultaneous DL reception in 2 DL subbands in RAN4. The core requirements in current spec are applied to all SBFD UEs.

Proposal 2: If it is decided to support the case of simultaneous configuration of SBFD and inter-band NR-DC, RAN4 to discuss whether/how to use enhance the RRM requirements for PSCell measurement of CSI-RS based L1 measurements. E.g. including RLM and BFD for PSCell measurement of CSI-RS based L1 measurements if SBFD operation is in DC.

- Reference

[1] R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei

[2] R4-2513004, LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#122, RAN1

[3] R4-2513014, Reply LS on simultaneous configuration of SBFD and DC, RAN2

[4] R1-2504858, Reply LS on simultaneous configuration of SBFD and DC, RAN1

[5] R4-2512352 Big CR for RRM requirements of Sub-band full duplex, Huawei, Samsung