3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513818

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.8.4.2.2

Source: 	Nokia

Title: 	Simplification of BS TRP test methods

Document for:	Approval

# 1.	Introduction

The revised WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at TSG RAN#109 [1]. The objective of this WI on simplification of BS TRP test methods is revised as follow:

- TRP simplification: Simplify BS TRP estimation methods (e.g. Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods.

This topic was discussed at TSG RAN4#116 [2] and the WF was agreed [3]. This contribution provides proposals to progress this topic in RAN4 according to the discussion and agreed WF.

# 2.	Discussion

It was agreed in the WF [3]:

Issue 2-2: Reference method and preferred method - Agreement: - The anechoic chamber to be clarified as the preferred test method for TRP measurements (as the reverberation chamber is already considered as the alternative test method); - Further discuss whether the “preferred” terminology is the right one. - Further work on CR to TR 37.941 to clarify definition of the anechoic chamber. - Enhancements to the Annex I (TRP sampling grids) to be further detailed to address Spherical Equal Angle Grid; related modifications to Annex I to be merged with the above chamber modifications in sub-bullet a. - Further discuss whether the “reference” terminology is the right one.

2.1	Terminology

The open issues discussed between ETSI and the European Commission for the drafting of the Harmonised Standards were shared in TFES(25)83027 [4], in which one of the most frequent issues preventing the citation of Harmonised Standards was reported as ‘Alternative test methods’:

Note that it was clearly stated that ‘a preferred test method now always needs to be identified if a Harmonised Standard is to be listed.’ Hence the ‘preferred’ terminology is used in ETSI and the European Commission, and thus the same terminology should also be used instead of the ‘reference’ terminology in the RAN4 specifications to avoid unnecessary ambiguity in ETSI and the European Commission.

2.2	Definition of the anechoic chamber

Since this is a definition of the ‘anechoic chamber’, it should be placed within clause 3.1 ‘Definitions of Terms’ in TR 37.941 [5]. The proposed wordings are as follows which are used in the draft CR in [6].

anechoic chamber: shielded OTA measurement chamber including IAC, CATR, or PWS.

2.3	Spherical Equal Angle Grid

It was generally agreed that the ‘preferred test method’ should also include the TPR sampling grid beside the TRP measurement chamber and spherical equal angle grid should be used. Consequently, annex I.1 of TS 38.141-2 [7] should be updated accordingly. The proposed changes are as follows which are used in the draft CR in [8].

# I.1 General The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values. The procedure in annex I.2 can provide an accurate estimate and can be applied to all TRP requirements, other procedures that are equivalent in terms of accuracy can also be applied. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex I.2 to annex I.9. When making TRP measurements the alignment between EUT and measurement antenna is important to achieve expected measurement uncertainty: 1. The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to correctly measure the TRP properly. 2. Test methods described in clauses I.5.1, I.5.2, I.10, I.11 and I.12 require angular alignment between the selected measurement grid and EUT radiation pattern in order to measure peak values in the main beams. Angular misalignment can lead to differences in the actual and measured angular positions of the intended maximum EIRP. 3. Test methods described in clause I.5.3, I.6 and I.9 are designed to be independent of rotations of the angular grid, and hence angular alignment between the measurement grid and EUT is not needed.

2.4	Enhancements to the Annex I

The notes in ERC/REC 74-01 [9] can be used as a reference for improvement in annex I of TS 38.141-2 [7] to clearly specify the pre-conditions of application and calculation of the TRP estimate when alternative test methods like orthogonal cuts grid or pre-scan are used.

Table 6: Spurious domain emission limits for the land mobile service, maritime mobile service and short range devices

| Reference number | Type of equipment | Frequency of the spurious domain emission f | Limits: mean power or, when applicable, average power during bursts duration in the reference bandwidth |
| --- | --- | --- | --- |
| 2.1.1 | Terminals and Base Stations (in transmit mode), except the equipment specified below | 9 kHz f 1 GHz | -36 dBm |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm |
| 2.1.2 | Short range devices; RLAN; CB; Cordless Telephones; Radio Microphones (all systems in transmit mode) | f within the bands: 87.5-118 MHz, 174-230 MHz, 470-694 MHz | -54 dBm |
|  |  | 9 kHz f 1 GHz (except 87.5118 MHz, 174230 MHz, 470694 MHz) | -36 dBm |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm |
| 2.1.3 | Short range inductive devices operating below 30 MHz (in transmit mode) (Note 3) | 9 kHz f 10 MHz | 27 dBA/m (at 9 kHz then decreasing by 10 dB/decade) (Note 4) |
|  |  | 10 MHz < f 30 MHz | -3.5 dBA/m (Note 4) |
|  |  | f within the bands: 87.5-118 MHz, 174-230 MHz, 470-694 MHz | -54 dBm |
|  |  | 30 MHz < f 1 GHz (except 87.5118 MHz, 174230 MHz, 470694 MHz) | -36 dBm |
|  |  | 1GHz < f Fupper (see recommend 3) | -30 dBm |
| 2.1.4 | Receivers and idle/standby transmitters except the equipment specified below | 9 kHz f 1 GHz | - 57 dBm |
|  |  | 1 GHz < f Fupper (see recommends 11) | - 47 dBm |
| 2.1.5 | Short range inductive receivers and idle/standby transmitters operating below 30 MHz | 9 kHz ≤ f < 4.78 MHz | 5.5 dBµA/m, decreasing by 3 dB/octave (Note 4) |
|  |  | 4.78 MHz ≤ f < 30 MHz | -22 dBµA/m (Note 4) |
|  |  | 30 MHz ≤ f < 1 GHz | -57 dBm |
| 2.1.6 | Base Stations using AAS and beamforming with integrated antennas operating below 6 GHz (Note 5) | 9 kHz f 1 GHz | -36 dBm(Note 6) |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm (Note 6) |
| 2.1.7 | Base Stations using AAS and beamforming with integrated antennas operating above 24.25 GHz (Note 5) | 9 kHz f 1 GHz | -36 dBm (Note 5) (Note 6) |
|  |  | 1 GHz < f 18 GHz | -30 dBm (Note 5) (Note 6) |
|  |  | 18 GHz < f Fupper (see recommends 3) | -20 dBm/10 MHz (other limits apply for specific frequency separations, see Figure 7) (Note 6) |
| 2.1.8 | Terminals operating above 24.25 GHz using AAS and beamforming with integrated antennas | 9 kHz f 1 GHz | -36 dBm (Note 6) |
|  |  | 1 GHz < f 7.25 GHz | -30 dBm (Note 6) |
|  |  | 7.25 GHz < f Fupper (see recommends 3) | -13 dBm/MHz and -10 dBm/100 MHz (Note 6) (Note 7) (Note 8) |
| Note 1: ƒ is the frequency of the spurious domain emission Note 2: For systems that use digital modulation and narrow-band high power (1 Watt) analogue modulated systems, the reference bandwidth is specified in section 2 of this annex, while for any other analogue modulation the reference bandwidth specified in recommend 4 is applicable. Note 3: Limits for Wireless Power Transmission for Electric Vehicles systems are under study and may need further consideration. Note 4: Levels are H-field limit at 10 m distance, measured by shielded loop antenna as specified by CISPR. Note 5 : There is currently no information on base stations using AAS and beamforming with integrated antennas operating between 6 and 24.25 GHz. These could be considered in a future revision as necessary. Note 6: For Terminals and Base Stations using AAS and beamforming with integrated antennas, the metric for unwanted emission is defined as Total Radiated Power (TRP): and applies separately for the transmitting and receiving phases of operation. where PD(r,,) is the power density in W/m2 at a distance r of two orthogonal polarizations. Over-the-air spatial parameters are specified in a Cartesian coordinate system (x, y, z) using spherical coordinates (r,,). is the angle in the x/y plane and it is between the x-axis and the projection of the vector onto the x/y plane and is defined between -180° and +180°.  is the angle between the projection of the vector in the x/y plane and the vector and is defined between -90° and +90. The TRP measurement procedure for emissions in the spurious domain with large frequency separation is slightly different from that of the wanted signal and unwanted emissions closer to the wanted signal. For the wanted signal, if the directivity is known, it is enough to measure the peak e.i.r.p. and adjust it with the directivity to obtain TRP. Care must be taken for using this method for unwanted emissions since its directivity can be different from the wanted signal directivity, in particular at large frequency separations from the carrier. If the directivity is not known but it can be verified that the emission is from the antenna array, TRP can be obtained by measuring e.i.r.p. in the cardinal cuts and using pattern multiplication to extrapolate the e.i.r.p. values in other directions. Pattern multiplication exploits the fact that the array factor of a rectangular array can be separable in two factors along the two symmetry planes. If none of the above options are applicable, the final option is to measure the e.i.r.p. on a full-sphere grid with the reference angular step. The measured values can then be integrated to obtain the TRP. All the above options are accurate methods for TRP assessment provided that the mentioned pre-conditions are fulfilled. For spurious emissions at larger frequency separations from the carrier, it is important to first identify the frequencies with notable emissions for further measurement. This is achieved through a pre-scan which can be performed on a very sparse grid. If no notable emission is identified at a certain frequency during the pre-scan, no further investigation is needed for that frequency. For frequencies which are identified for further measurement, a number of orthogonal cuts (two or three) or a sparse spherical grid can be used. These methods are based on the assumption that spurious emissions at larger frequency separation are not fully correlated and are less directive. For spurious emissions, an upper bound assessment for TRP is enough to ensure the compliance and the exact TRP value is of less importance. Therefore, a systematic correction factor is added to the estimated TRP value in order to ensure an overestimate with a certain confidence. The systematic correction factor is dependent on the chosen grid type and angular step. Orthogonal cut measurements should be measured with the reference angular steps. The sparse full sphere measurement can be performed on angular steps larger than the reference step and smaller than 15 degrees. A larger angular step results in a shorter measurement time while it imposes a larger systematic correction. For studies, the TRP needs to be reflected in terms of conducted power (P). The link between conducted power and TRP is based on antenna efficiency that could be transformed in terms of additional losses (L). The relation between these terms are P=TRP+L. (L needs to be defined with a positive number) Note 7: Both limits are simultaneously applicable. Note 8: Limits in the frequency band 8.5-10.5 GHz might be reconsidered, if necessary when this Recommendation is revised. |  |  |  |

# 3.	Conclusion

This contribution has provided proposals to progress the objective of the WI for TRP simplification in RAN4, the proposals are summarized as follows.

Proposals:

1)	The ‘preferred’ terminology should be used instead of the ‘reference’ terminology in the RAN4 specifications to avoid unnecessary ambiguity in ETSI and the European Commission.

2)	The definition of the ‘anechoic chamber’ should be placed within clause 3.1 ‘Definitions of Terms’ in TR 37.941 [5] as:

anechoic chamber: shielded OTA measurement chamber including IAC, CATR, or PWS.

3)	Annex I.1 of TS 38.141-2 [7] should be updated to define spherical equal angle grid as the TRP sampling grid for the ‘preferred test method’.

4)	Use the notes in Table 6 of ERC/REC 74-01 as a reference for improvement in annex I of TS 38.141-2.

# References

[1]	RP-252578, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, Huawei, Nokia, Ericsson.

[2]	R4-2511451, “Topic summary for [116][304] NR_BS_RF_Part3_OTA_TRP”, Moderator (ZTE).

[3]	R4-2512580, “Way Forward for [116][304] NR_BS_RF_Part3_OTA_TRP”, ZTE.

[4]	TFES(25)83027, “Open issues discussed with the European Commission – updates”, Ericsson Limited.

[5]	3GPP TR 37.941 v19.0.0: “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”.

[6]	R4-2513819, “Draft CR on definition of anechoic chamber”, Nokia.

[7]	3GPP TS 38.141-2 v19.2.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.

[8]	R4-2513820, “Draft CR on TRP sampling grid for the preferred test method”, Nokia.

[9]	ERC/REC 74-01, “ERC/REC 74-01 of 1998 on unwanted Emissions in the Spurious Domain corrected May 2022 (SE#91)”.