3GPP TSG-RAN WG4 Meeting # 116Bis		R4-2513855

Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	7.8.3

Source:	Apple

Title:	Initial Views on RF requirements  for LB CA via switching with 2 DLs

WI/SI:	NR_LBCA_Sw_enh-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction and Scope

In the recent RAN plenary meeting (3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025, 2025), discussions were held on enhancements for Rel-19 low NR band aggregation via switching (NR_LBCA_Sw) feature. As a result, an agreement was made to introduce a new WI in Rel-20 ( NR_LBCA_Sw_enh) to handle enhancements to the feature [1]. The main objectives of the new WI are highlighted below:

The core part of WI:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

  - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

  - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw

- Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this discussion paper, we will share our initial views on the UE RF requirements, using CA_n12A-n29A and CA_n28A-n67A as examples of the band combination to support this feature.

# Discussion

  - Low NR band aggregation via switching with 2 DLs

In Rel-19, low NR band aggregation via switching was introduced to address challenges that exist for LB-LB inter-band carrier aggregation when traditional aggregation techniques (single antenna with multiplexers) are used. Only CA combinations involving an SDL band and a FDD band were considered. Under this switching, when the SCell ( operation is triggered, the UE needs to switch to the SCell, and during the operation period there is no simultaneous Tx/Rx between the PCell and the SCell.

With the goal to increase the UE DL throughput, enhancements to this feature was recently introduced in Rel-20 by allowing two downlink (DL) carriers to be simultaneously enabled. Under this enhancement, the UE can switch between case 1 and case 3 as described below and captured in Figure 1:

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1

Figure 2.1-1:	Simplified model for realizing CA aggregation between n5 and n29 through switching

Figure 2.1-2:	Examples of CA combos supporting NR_LBCA_Sw-enh

  - Reference RF front-end architecture

As captured in Figure 2.1-2 shown above, the two DLs are either adjacent (CA_n28-n67) or relatively very close in frequency (CA_n12-n29). This implies that a new wider RX filter could be designed to cover both DLs. Ass a result of this, the following UE architecture could be considered.

Observation 1:	The two DLs are either adjacent (CA_n28-n67) or relatively very close in frequency

(CA_n12-n29). This implies that a new wider RX filter could be designed to cover both DLs.

Figure 2.1-2:	Architecture for combos supporting NR_LBCA_Sw-enh

  - Initial views on the switching period

During the band switching process, several events have to occur within the UE baseband, transceiver, and RF front end. Depending on the UE implementation, the main events that could potentially contribute to the switching period are the following:

- PLL settling and relocking (or retuning), depending on the PLL architecture

- Antenna tuning

- RFFE parameters loading, including ET parameters

- RFIC programming and calibration,

As captured in Figure 2.1-1, n29 spectrum is withing the n12 spectrum. Therefore, the existing n12 antenna could be used for n29 without any antenna tuning. Likewise, antenna tuning may not be needed for CA_n28-n67 since n67 .

Observation 2:	Because n29 is located within n12 band, antenna tuning may not be required when switching between n12 and n29. Likewise, antenna tuning is not required for CA_n28-n67.

Proposal 2:	To accommodate different UE implementations, we propose using the same switching period values used in Rel-19 (35uSec, 70uSec, and 140uSec) as a baseline to determine the periods for Rel-20.

# 3	Conclusions

Observation 1:	The two DLs are either adjacent (CA_n28-n67) or relatively very close in frequency

(CA_n12-n29). This implies that a new wider RX filter could be designed to cover both DLs.

Observation 2:	Because n29 is located within n12 band, antenna tuning may not be required when switching between n12 and n29. Likewise, antenna tuning is not required for CA_n28-n67.

Proposal 2:	To accommodate different UE implementations, we propose using the same switching period values used in Rel-19 (35uSec, 70uSec, and 140uSec) as a baseline to determine the periods for Rel-20.

# 4	References

RP-252953, “New WID on enhancement of low NR band carrier aggregation via switching”, by RAN4 Chair (Apple),  3GPP TSG RAN Meeting #109, Prague, Beijing, China, September 15-18, 2025