# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513863

Prague, Czech Republic, 13th to 18th October 2025

Agenda Item:	6.8.4.3

Source:	Ericsson

Title:	On Transmitter Spurious Emission Co-existence BS conformance discussion

Document for:	Approval

# 1	Introduction

LS from ETSI MSG TFES [1] was received with a request to re-evaluate co-existence Tx spurious emission requirement levels for BS. It was acknowledged by RAN4 in the response LS [2].

The co-existence spurious emission requirement was originally derived in 1999 for UTRA. The technical background at that time was very much focused on the introduction of UTRA FDD/TDD operating at 2 GHz. The assumptions used as references to the original requirement definition can be found in a contribution [3] presented at RAN4#113 in Orlando. Over the years RAN4 have defined new operating bands below and above 2 GHz using the same requirement level. In current specification, bands are defined within the range 410 MHz up to 7125 MHz with the same requirement level, except for a few occasions (e.g. protection of GSM). Therefore, it is now time to re-evaluate the relevance of transmitter co-existence spurious emission requirement levels.

The objective can be summarized as:

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES

- Improve transmitter co-existence spurious emission requirements if needed.

  - Note: start the work after March 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

In this contribution, we share our views on the performance impact in the conformation specification.

# 2	Discussion

In the current BS RF specifications, additional spurious emissions requirements are defined to protect other networks operating in different frequency bands within the same geographical area. These requirements may apply for the protection of systems operating in frequency ranges outside the BS downlink operating band. TR 25.942 describes RF coexistence requirements from a spectrum-sharing and inter-system interference perspective, establishing performance thresholds to ensure acceptable operation between different network elements (e.g., BS–UE and BS–BS) and with other radio services.

In our companion contribution [3], we share our calibration results and other consideration to progress on the simulation assumptions. This contribution analyses the potential impact on both TS 38.141-1 and TS 38.141-2 if the TR 25.942-derived Tx spurious emission coexistence requirements for FR1 victim frequencies up to 7.125 GHz, as evaluated in recent coexistence simulations.

## 2.1	Impact on TS 38.141-1

Section 6.6.5.5.1.3 of TS 38.141-1 specifies band-specific Tx spurious emission limits applied for the protection of system operating in frequency ranges other than the BS downlink operating band.

| Section | Description | Impact |
| --- | --- | --- |
| 6.6.5.5.1.3 | BS spurious emissions limits for BS for co-existence with systems operating in other frequency bands | Limits will change based on simulation results. Test methods and measurement bandwidths would remain unchanged |

## 2.2	Impact on TS 38.141-2

Section 6.7.5.4.5 of TS 38.141-2 specifies band-specific Tx spurious emission limits applied for the protection of system operating in frequency ranges other than the BS downlink operating band.

| Section | Description | Impact |
| --- | --- | --- |
| 6.7.5.4.5 | BS spurious emissions test limits for BS for co-existence with systems operating in other frequency bands | Limits will change based on simulation results. Test methods and measurement bandwidths would remain unchanged |

The only performance impact will be on the test requirements on Clause 6.6.5.5.1.3 TS 38.141-1 and Clause 6.7.5.4.5 TS 38.141-2, based on the spurious emission limits derived from the system level evaluations.

In our companion contribution [5], we present an overview on the challenges we have with current BS RF specification structure in relation to transmitter co-existence spurious emission requirements and provide some potential approaches to consider improving the specification structure. The performance impact is interlinked with the RAN4 task to work on framework simplification for co-existence requirements as the same requirements are written in such a way that they are defined per operating band. This have resulted in very long tables with the same requirement level applicable for many bands, except for a few bands for GSM. With the number of supported bands today we have reached the point where readability suffer, effort to include a new band increases and effort for specification maintenance increases.

The approach with band specific requirement levels worked fine when few bands were defined. But now the management of the specification structure format is not sustainable anymore.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The only performance impact will be on the test requirements on Clause 6.6.5.5.1.3 TS 38.141-1 and Clause 6.7.5.4.5 TS 38.141-2, based on the spurious emission limits derived from the system level evaluations.

# References

[1]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[3]	  R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

[4]	R4-2513862, “On Transmitter Spurious Emission Co-existence discussion”, RAN4#116bis, Ericsson

[5]	R4-2500880. “On the topic of BS RF specification structure improvements”, RAN4#116, Ericsson