3GPP TSG-RAN WG4 Meeting #116bis		                		      R4-2513900

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.10.3

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

In the RAN #103 meeting, the WID on several enhancements for NR and MR-DC RRM requirements was approval. There are two objectives need to be discussed in Rel-19, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, and the other is Fast SCell activation for UE supporting Rel-18 EMR. In this meeting, we should discuss performance part on fast SCell activation for UE supporting R18 EMR from this meeting according to schedule [1].

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

Discussion

During the discussion on core part, RAN4 has agreed that both single direct activation and MAC CE based single SCell activation are supported at SCell addition. Meanwhile, MAC CE based PUCCH SCell activation is included. Therefore, RAN4 should discuss the test cases for the requirements we have defined on core part [2].

RAN4#112bis meeting Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported. - Note: MAC CE based PUCCH SCell activation is included. RAN4#115 meeting Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

In the design of legacy test cases, known SCell and unknown SCell cases were considered for SCell Activation and deactivation in A.6.5.3.1, A.6.5.3.2, A.6.5.3.3, and known SCell and unknown SCell cases were considered for PUCCH SCell Activation and deactivation in A.6.5.3.6 and A6.5.3.7. For known SCell case, R19 did not modify the previous requirements, but only modified the requirements for the case where the target SCell is unknown and meets the conditions and the requirenebts refer to 8.3.2A. Therefore, we think that three scenarios need to be considered in the design of test cases and maybe it’s enough for UE to pass one of them due to the requirements of unknown SCell case are the same for these three scenarios and do not need the repeated validation.

RAN4 needs to guarantee test coverage because UE may only support some of scenarios. Therefore, we support to introduce test cases for all three scenarios and both FR1 and FR2. Based on these test cases, RAN4 could further discuss the applicability rule whether UE is required to pass all of them.

Support to consider direct SCell activation at SCell addition for FR2.

In the previous discussion, we discussed the issue on applicability of fast SCell activation delay requirements and reached a conclusion blew.

RAN4#113 meeting Issue 1-1-1: Applicability of fast SCell activation delay requirements Agreement: - For R19 fast SCell activation for UE supporting R18 EMR, when neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured: - Fast SCell activation delay requirements (same with conclusion of the case where either measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured) apply under the following conditions: - For EMR carriers: - Consider the case when the T331 timer hasn’t expired at the moment of initiation of RRC state transition to Connected mode. i.e., measIdleDuration-r16 is configured, and T331 is running. Otherwise, follow the legacy unknown SCell activation delay requirements

According to the agreements in core part, we should consider three configurations that:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

R18 EMR is used for the first bullet to reduce the activation delay. Meanwhile, R18 cell reselection measurement results and R16 EMR are used for bullet 2 and bullet 3 separately. In our understanding, the reporting for these configurations has been verified in previous release and the validity of configuration 1 and configuration 2 has been checked in release 18. Therefore, it can be used directly. In release 19, we need to consider test cases to verify that the SCell activation time are within the requirements stated in clause 8.3.2A, and R16 EMR, R18 eEMR and valid cell reselection measurement should be considered when designing the test cases.

In other words, test cases that cover different UE capabilities and NW configurations should be considered. In our opinion, there are three configurations, including two for R18 and one for R16. If only verifying the SCell activation under R18 configuration, can we ensure that EMR without validity verification can also complete the fast SCell activation process. Based on this, we suggest considering the design of test cases for all three configurations mentioned above.

Proposal 2: Support to define test case for the configuration that UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode.

As for details of testing, we can merge and modify them based on the existing test procedures. Fast SCell activation based on R18 EMR is a fusion of two testing processes, EMR reporting and fast Scell activation. In the existing protocol, we have validated the above two measurement processes separately, as summarized in the table below.

Table 1. Existing Scell activation test cases

| Feature | Side Condition | FR-group | Test case |
| --- | --- | --- | --- |
| Scell activaiton/deactivation | known | FR2 | A5.5.3.2 |
| Scell activaiton/deactivation | unknown | FR2 | A5.5.3.5 |
| Direct Scell activation | known | FR2 | A5.5.3.7 |
| PUCCH Scell activation/deactiation | known | FR2 | A5.5.3.9 |
| PUCCH Scell activation/deactivation | unknown | FR2 | A5.5.3.10 |
| Scell activaiton/deactivation | known | FR1 | A6.5.3.1 |
| Scell activation/deactivaiton | unknown | FR1 | A6.5.3.3 |
| Direct Scell activation | known | FR1 | A6.5.3.4 |
| PUCCH Scell activation/deactiation | known | FR1 | A6.5.3.6 |
| PUCCH Scell activation/deactivation | unknown | FR1 | A6.5.3.7 |

Table 2. Existing EMR/Cell reselection measurement report test case

| Feature | UE behaviour | FR group | Test case |
| --- | --- | --- | --- |
| Legacy EMR test | With reporting | FR1 | A6.6.9.1 |
| Idle mode fast CA/DC eEMR | Without reporting | FR1 | A6.6.9.2 |
| Idle mode fast CA/DC cell reselection | Without reporting | FR1 | A6.6.9.3 |
| Idle mode fast CA/DC cell reselection | With reporting | FR1 | A6.6.9.4 |
| Idle mode fast CA/DC eEMR | Without reporting | FR2 | A7.6.23.1 |
| Idle mode fast CA/DC cell reselection | Without reporting | FR2 | A7.6.23.2 |
| Idle mode fast CA/DC cell reselection | With reporting | FR2 | A7.6.23.3 |

Basic principle is to reuse the legacy test cases as much as possible. The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle/Inactive mode fast CA/DC reporting test case

Proposal 3: The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle/Inactive mode fast CA/DC reporting test case.

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposals:

Proposal 1: Support to consider direct SCell activation at SCell addition for FR2.

Proposal 2: Support to define test case for the configuration that UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode.

Proposal 3: The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle/Inactive mode fast CA/DC reporting test case.

References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT