3GPP TSG RAN WG4 Meeting #116-bis	R4-2513908

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			6.19.3.1

Source:	vivo

Title:	Discussion on RRM performance requirements of (e)RedCap NTN

Document for:	Discussion

- Introduction

In the previous meeting, companies provided preliminary analysis on performance requirements and RRM test case design for NTN in NR Phase 3, which has been captured in the latest WF [1]. In this paper, we will present our further analysis on RRM performance requirements of (e)RedCap NTN.

- Discussion

#### Issue 2-1-1: Measurement accuracy requirement Agreement: - FFS: RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location - For L1/L3 measurement accuracy, RAN4 to follow the agreements reached in RAN4 #114-bis meeting. - For measurement accuracy for NW verified location, RAN4 to follow the agreements reached in RAN4 #114 meeting. - RAN4 to define new sections for measurement accuracy requirements with suffix ‘10.X’ for (e)RedCap UE with NTN. - Further to discuss the side conditions.

First, regarding (e)Redcap UEs with FR1-NTN, the definition of performance requirements primarily focuses on measurement accuracy requirements. Based on the contribution submitted by companies in the previous meeting, it has a command understanding of the accuracy requirements that need to be defined, which are exactly the requirements included in the aforementioned agreement.

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

#### Issue 2-1-3: How and where to capture the test cases for (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: The existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN. - Proposal 1a: RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on. - Proposal 1b: Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 2: RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements. - Proposal 2a: RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs. - Proposal 3: - RAN4 to define new sections for test cases with suffix ‘A.X’. - Proposal 4: For RedCap UEs in NTN scenario, - To incorporate clauses for RedCap in NTN within Clause A.14, - Some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Regarding where to capture this, we prefer that RAN4 to define new sections for test cases related to (e)RedCap UEs with FR1-NTN. In the previous meeting, we introduced new UE capabilities for (e)RedCap UEs with FR1-NTN. For Rel-19 UEs capable of (e)RedCap with FR1-NTN, they only need to be tested in the new test cases defined in Rel-19 for (e)RedCap NTN, and are not required to comply with the existing ones (e.g., legacy test cases for NTN). Therefore, to distinguish from the existing test cases defined in Clause A.14 and to make the spec drafting clearer and know which set of tests to follow, we think defining a new chapter is a better approach.

As for how to capture this, in order to avoid redundant duplication with the existing spec, RAN4 can adopt the "refer to" method for the reusable parts of the existing content when drafting the performance specification.

Proposal 2: On How and where to capture the test cases for (e)RedCap UE with FR1-NTN,

- RAN4 to define new sections for test cases with suffix ‘A.X’

- RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents

Specifically, regarding the candidate test list in the WF [1] summarized by the moderator, which includes the following FFS-related mobility test cases, as we mentioned above, for Rel-19 UEs capable of (e)RedCap with FR1-NTN, the relevant performance is tested through the new test cases defined for RedCap NTN. Therefore, to ensure test coverage, similar to legacy NTN, the following test cases also need to be defined.

- FFS: Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for 1Rx/2Rx RedCap UE with NTN - FFS: Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - FFS: Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

Proposal 3: RAN4 need to define the following FFS-related mobility test cases:

- Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN

- Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements

- Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN

- Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

- Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

- Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

- Summary

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

Proposal 2: On How and where to capture the test cases for (e)RedCap UE with FR1-NTN,

- RAN4 to define new sections for test cases with suffix ‘A.X’

- RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents

Proposal 3: RAN4 need to define the following FFS-related mobility test cases:

- Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN

- Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements

- Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN

- Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

- Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

- Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN

- References

[1] R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT