3GPP TSG-RAN WG4 Meeting #116bis	R4-2511009

Prague, Czech Republic, Oct 13 – 17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ENDC_RF_Ph4) draft CR to TS 38.101-2: Corrections for UE RF enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update the UE capabilities according to RAN2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the tetative UE capabilies to align with RAN2 spec. - fr2-MPR-ImprovementDownlinkIndependent-r19 changed as mpr-DL-Independent-r19 - fr2-MPR-ImprovementActivationDependent-r19 changed as mpr-ActivateDependent-r19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specs are not aligned between RAN4 and RAN2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2.1, 6.2A.2.3, 6.2A.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

6.2A.2.2	Maximum output power reduction for power class 1

6.2A.2.2.1	Maximum output power reduction for power class 1 intra-band contiguous UL CA

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA+∆MPR, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Tables 6.2A.2.2-1 and 6.2A.2.2-2. If mpr-DL-Independent-r19 is supported and the intra-band contiguous CA is configured with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2.

If mpr-ActivateDependent-r19 is supported and the activated subset of CCs in the intra-band contiguous CA configuration comprises single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2.

∆MPR for FR2-1 256 QAM as specified in Table 6.2.2.1-5 applies.

The MPR table 6.2A.2.2-1 depends on the parameter ‘bandwidth basis’. If mpr-DL-Independent-r19 is supported, UL BWchannel_CA is used as bandwidth basis. If mpr-ActivateDependent-r19 is supported and all activated CCs form a contiguous block in both UL and DL, the bandwidth basis is the Cumulative aggregated channel bandwidth (CABW) as calculated from the activated subset of the CA configuration. If mpr-DL-Independent-r19 and mpr-ActivateDependent-r19 are both supported and all activated UL CCs form a contiguous block, the bandwidth basis is the UL aggregated channel BW as calculated from the activated subset of the CA configuration. Otherwise, Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | Bandwidth basis |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |  |  |

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 7.0 | ≤ 5.0 | ≤ 2.0 | ≤ 2.0 |
| QPSK | ≤ 8.0 | ≤ 6.0 | ≤ 3.0 | ≤ 3.0 |
| 16 QAM | ≤ 8.0 | ≤ 6.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall be derived instead as MAX(MPR1, MPR2) if mpr-DL-Independent-r19 is not supported, and as MPR2 if mpr-DL-Independent-r19 is supported, where:

MPR1 shall be determined from Table 6.2.2.1-1 if CABW  200 MHz, from Table 6.2.2.1-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.1-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if UL BWchannel_CA > 200 MHz.

If mpr-ActivateDependent-r19 is supported, and the activated CCs form a contiguous block in both UL and DL, the CABWactivated is the CABW calculated on the activated subset of configured CCs. In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous activated CCs, and whose CABWactivated  400 MHz, MPRWT_C_CA shall instead be derived as MAX(MPR1a, MPR2a) if mpr-DL-Independent-r19 is not supported, and as MPR2a if mpr-DL-Independent-r19 is supported, where:

-	MPR1a shall be determined from Table 6.2.2.1-1 if CABWactivated  200 MHz, from Table 6.2.2.1-2 if CABWactivated > 200 MHz.

-	MPR2a shall be determined from Table 6.2.2.1-1 if UL aggregated channel BW as calculated on the activated subset of the CA configuration  200 MHz, from Table 6.2.2.1-2 otherwise.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.1-1 and Table 6.2.2.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If mpr-DL-Independent-r19 is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL BWchannel_CA assuming lowest SCS among all configured CCs. If mpr-ActivateDependent-r19 is supported and all activated CCs form a contiguous block in both UL and DL, NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth calculated on the activated subset of CCs, assuming lowest SCS among all activated CCs. If mpr-DL-Independent-r19 and mpr-ActivateDependent-r19 are supported and all activated CCs form a contiguous block in both UL and DL, NRB_agg_C is the number of the aggregated RBs within the activated subset of the fully allocated UL BWchannel_CA bandwidth assuming lowest SCS among all activated CCs.

<Next Change>

6.2A.2.3	Maximum output power reduction for power class 2

For power class 2, MPR (except 256 QAM) specified in sub-clause 6.2A.2.4.1 applies for intra-band contiguous UL CA and sub-clause 6.2A.2.4.2 applies for intra-band non-contiguous UL CA.

Table 6.2A.2.3-1: (Void)

For FR2-1 256 QAM, for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth, the following rule for MPR applies:

MPR = MPRC_CA+∆MPR

MPRC_CA is defined in Table 6.2A.2.3-2. If mpr-DL-Independent-r19 is supported and the intra-band contiguous CA is configured with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.2-2 and 6.2.2.2-3.

If [mpr-ActivateDependent-r19] is supported and the activated subset of CCs in the intra-band contiguous CA configuration comprises a single UL CC, MPRC_CA shall be determined from Tables 6.2.2.2-2 and 6.2.2.2-3.

△MPR as specified in Table 6.2.2.1-5 applies.

The MPR table 6.2A.2.3-2 depends on the parameter ‘bandwidth basis’. If mpr-DL-Independent-r19 is supported, UL BWchannel_CA is used as bandwidth basis. If mpr-ActivateDependent-r19 is supported and all activated CCs form a contiguous block in both UL and DL, the bandwidth basis is the Cumulative aggregated channel bandwidth (CABW) as calculated from the activated subset of the CA configuration. If mpr-DL-Independent-r19 and mpr-ActivateDependent-r19 are both supported and all activated UL CCs form a contiguous block, the bandwidth basis is the UL aggregated channel BW as calculated from the activated subset of the CA configuration. Otherwise, Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis.

Table 6.2A.2.3-2: Maximum power reduction (MPRC_CA) for UE power class 2 in FR2-1

|  |  | Bandwidth basis |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: Refer to clause 6.1 for 256QAM applicability. |  |  |  |  |  |

For FR2-1 256 QAM, for intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA+∆MPR, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If mpr-DL-Independent-r19 is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL BWchannel_CA assuming lowest SCS among all configured CCs. If mpr-ActivateDependent-r19 is supported and all activated CCs form a contiguous block in both UL and DL, NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth calculated on the activated subset of CCs, assuming lowest SCS among all activated CCs. If mpr-DL-Independent-r19 and mpr-ActivateDependent-r19 are supported and all activated CCs form a contiguous block in both UL and DL, NRB_agg_C is the number of the aggregated RBs within the activated subset of the fully allocated UL BWchannel_CA bandwidth assuming lowest SCS among all activated CCs.

For FR2-1 256QAM, for intra-band non-contiguous UL CA, the following rule for MPR applies:

MPR = max(MPRNC_CA+∆MPR, -8*A +10.0)

Where:

MPRNC_CA is derived from table 6.2A.2.3-3.

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs.

Table 6.2A.2.3-3: MPRNC_CA for UE power class 2 in FR2-1

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |  |  |

For inter-band carrier aggregation with uplink assigned to two NR bands, the MPR for each configured UL band in the UL CA band combination is:

MPRinter-band_CA = max(MPRSingleBand, MPRPA-PA)

Where:

MPRSingleBand is the MPR specified in clause 6.2.2.2 for the allocation and modulation type in that band

MPRPA-PA is specified in Table 6.2A.2.3-2 and applies only when both bands have non-zero UL RB allocations, 0 dB otherwise.

Table 6.2A.2.3-2: MPRPA-PA for Inter-band ULCA in FR2 for PC2

| NR CA Band | Value (dB) | Condition |
| --- | --- | --- |
| CA_n257A-n259A | Max(0, 6 - 10*log10(Max(1.0, LRB,min*12*SCS/1e6))) | LRB,min = Min (LRB,n257 , LRB,n259 ), where LRB,n is the number of non-zero power UL RBs in band ‘n’ |

6.2A.2.4	Maximum output power reduction for power class 3

6.2A.2.4.1	Maximum output power reduction for power class 3 intra-band contiguous CA

For power class 3, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Tables 6.2A.2.4-1 and 6.2A.2.4-2. If mpr-DL-Independent-r19 is supported and the intra-band contiguous CA is configured with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.3-1 and 6.2.2.3-2.

If mpr-ActivateDependent-r19 is supported and the activated subset of CCs in the intra-band contiguous CA configuration comprises a single UL CC, MPRC_CA shall be determined from Tables 6.2.2.3-1 and 6.2.2.3-2.

The MPR table 6.2A.2.4-1 depends on the parameter ‘bandwidth basis’. If mpr-DL-Independent-r19 is supported, UL BWchannel_CA is used as bandwidth basis. If mpr-ActivateDependent-r19 is supported and all activated CCs form a contiguous block in both UL and DL, the bandwidth basis is the Cumulative aggregated channel bandwidth (CABW) as calculated from the activated subset of the CA configuration. If mpr-DL-Independent-r19 and mpr-ActivateDependent-r19 are both supported and all activated UL CCs form a contiguous block, the bandwidth basis is the UL aggregated channel BW as calculated from the activated subset of the CA configuration. Otherwise, Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis.

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3 in FR2-1

|  |  | Bandwidth basis |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). |  |  |  |  |  |

Table 6.2A.2.4-2: Maximum power reduction (MPRWT_C_CA) for UE power class 3 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 |
| QPSK | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 |
| 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2) if mpr-DL-Independent-r19 is not supported, and as MPR2 if mpr-DL-Independent-r19 is supported, where:

MPR1 shall be determined from Table 6.2.2.3-1 if CABW  200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

If mpr-ActivateDependent-r19 is supported, and the activated CCs form a contiguous block in both UL and DL, the CABWactivated is the CABW calculated on the activated subset of configured CCs. In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous activated CCs, and whose CABWactivated  400 MHz, MPRWT_C_CA shall instead be derived as MAX(MPR1a, MPR2a) if mpr-DL-Independent-r19 is not supported, and as MPR2a if mpr-DL-Independent-r19 is supported, where:

-	MPR1a shall be determined from Table 6.2.2.3-1 if CABWactivated  200 MHz, from Table 6.2.2.3-2 if CABWactivated > 200 MHz.

-	MPR2a shall be determined from Table 6.2.2.3-1 if UL aggregated channel BW as calculated on the activated subset of the CA configuration  200 MHz, from Table 6.2.2.3-2 otherwise.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3-1 and Table 6.2.2.3-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If mpr-DL-Independent-r19 is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL BWchannel_CA assuming lowest SCS among all configured CCs. If mpr-ActivateDependent-r19 is supported and all activated CCs form a contiguous block in both UL and DL, NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth calculated on the activated subset of CCs, assuming lowest SCS among all activated CCs. If mpr-DL-Independent-r19 and mpr-ActivateDependent-r19 are supported and all activated CCs form a contiguous block in both UL and DL, NRB_agg_C is the number of the aggregated RBs within the activated subset of the fully allocated UL BWchannel_CA bandwidth assuming lowest SCS among all activated CCs

<<End of change>>