# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513927

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on RF requirements for HPUE

# Agenda Item:   7.1.4

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WID on UE RF requirement enhancements for the deployment in NR FR1 was approved [1]. The objective for HPUE of this WID is:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

The first two bullets, i.e., 4Tx for CPE/FWA UE with 4x26dBm PA configurations and 2Tx for PC1.5 handheld and FWA UE with 2x26dBm PA configurations, will be discussed since this meeting. In this contribution, we provide some of our considerations on RF requirements.

- Discussion

## 2.1	4Tx for CPE/FWA UE with 4x26dBm PA configuration

It has been confirmed in the objectives that PC1/[PC1b] 4x4 UL MIMO and TxD is supported for TDD single carrier with the maximum output power up to 32dBm. In Rel-18, 4Tx for PC1.5 CPE/FWA UE with 4x23dBm, 2x23dBm+2x26dBm, 4x26dBm PA configurations were discussed and corresponding 4-port UL MIMO and TxD requirements were specified in TS38.101-1, which can be referred to.

### UL MIMO and TxD

For the RF requirements related to UL MIMO, RAN4 has already defined UL MIMO requirements for multiple transmit antenna connectors up to a maximum of four in TS38.101-1, where some of the requirements apply to the sum from all UE transmit antenna connectors and other requirements apply to each transmit antenna connector. The RF requirements for UL MIMO in current TS38.101-1 are summarized In Table 1.

Table 1 UL MIMO RF requirement types in TS38.101-1

| RF requirements for UL MIMO | Apply to |
| --- | --- |
| - UE maximum output power - MPR/A-MPR - Minimum output power - Configured transmitted power - OBW/Out of band emission/SE | Apply to the sum from all UE transmit antenna connectors |
| - Frequency error - Transmit OFF power - Transmit ON/OFF time mask - Transmit modulation quality (EVM, carrier leakage, IBE, etc..) - Transmit intermodulation | Apply to each transmit antenna connector |
| - Time alignment error - Coherent UL MIMO | Difference between any two different transmit antenna connectors |

Then for this new CPE/FWA UE with 4x26dBm PA configuration, the existing approach can be reused. For the requirements that are defined on each transmit antenna connector can keep unchanged, such as frequency error/Transmit OFF power, TAE and coherent UL MIMO. For the requirements that apply to the sum from all UE transmit antenna connectors, such as minimum output power and OBW/out of band emission/SE requirements, they are agnostic of power class and number of transmit antenna connectors. But the sum power from all UE transmit antenna connectors is subject to the power class.

The power class for 4x26dBm PA configurations haven’t been decided yet and it will be discussed in this meeting. As we discussed in our paper [2] in this meeting, if introducing a new power class, the co-existence simulation should be studied first for ACLR evaluation to guarantee a UE with higher power class cannot cause high interference for the existing system. Accordingly, a series of new RF requirements for this new power class should be specified, which is a quite new work.

With the same PC1 power class, the MPR/A-MPR requirements should also be specified for 4Tx CPE/FWA UE with 4x26dBm PA configuration considering the more severe RIMD products caused by 4Tx, which would make it difficult for a UE to meet OOB emission requirements. Consequently, larger MPR/A-MPR requirements may be needed.

Therefore, different power class decision will cause different workload in RAN4. The RF requirements discussion would largely depend on which power class is adopted.

Observation 1. Reusing legacy PC1 ans introducing a new power class will cause different workload in RAN4. The RF requirements discussion would largely depend on which power class is adopted.

- Proposal 1: The requirements of Frequency error, Transmit OFF power, Transmit ON/OFF time mask, Transmit modulation quality, Transmit intermodulation keep unchanged for 4x26dBm PA configurations, regardless of which power class is adopted.

- Proposal 2: There are no changes for Rx requirements for 4x26dBm PA configurations, regardless of which power class is adopted.

- Proposal 3: For the requirements depends on sum of power from all UE transmit antenna connectors, e.g. MPR/A-MPR, OOB emission requirements, it should be postponed until RAN4 decides the power class.

For signal antenna port with 4Tx diversity, the requirements for TxD can be derived from UL MIMO requirements.

Proposal 4: The requirements for TxD can be derived from UL MIMO requirements.

### ULFPTx

ULFPTx feature is introduced in Rel-16 as UL MIMO enhancement, and for 4Tx PC1.5 UL MIMO in Rel-18, ULFPTx requirements were defined. In the WID, due to the maximum output is up to 32dBm (actually there are two choices, which are 31dBm or 32dBm.), for ULFPTX, it seems only mode-1 is considered.

Proposal 5: To achieve 31dBm or 32dBm maximum output, only ULFPTx mode-1 is considered.

## 2.2	PC1.5 with 2Tx for FDD bands for handheld and FWA UE

PC1.5 with 2Tx for FDD bands for handheld and FWA UE are going to be introduced in this WID since FDD bands are also important for some operators considering data throughput for cell edge users is not satisfactory. This is not the first time to introduce HPUE for FDD bands. In Rel-17, PC2 HPUE for FDD bands was introduced. In current TS38.101-1, PC1.5 with 2Tx for TDD bands, such as n41, n77, n78, n79, and related 2x2 UL MIMO and TxD requirements are specified. These requirements are agnostic of NR bands. If RAN4 would like to introduce new FDD bands for PC1.5 UE with 2Tx, some legacy requirements should also apply, including maximum output power and MPR.

Proposal 6: The MPR requirements of PC1.5 2Tx requirements for TDD bands can be applied to FDD bands for PC1.5 UE with 2Tx.

One difference between PC1.5 2Tx FDD and PC1.5 2Tx TDD is A-MPR requirement. A-MPR is band-specific requirement which need to be discussed one by one for FDD bands.

Proposal 7: A-MPR needs to be discussed for PC1.5 2Tx for FDD bands.

Another difference between PC1.5 2Tx FDD and PC1.5 2Tx TDD is that RSD requirements should be defined for 2Tx FDD due to the self-interference impacts on Rx from its own Tx. There are two example bands, n1 and n25, to be defined in the WID. In current TS38.101-1, the RSD requirements from PC3 to PC2 for FDD bands n1 and n25 for UE supporting Tx Diversity are defined as:

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n25 |  | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |

It can be found that 0dB RSD is defined for n1 due to large Tx-Rx frequency range. Therefore, for PC1.5 2Tx UE in band n1, it could be foreseen the RSD=0dB for band n1 due to large Tx-Rx frequency range. For band n25, it can be further discussed.

In addition to that, with our experience in the past, there will be more new FDD bands introduced in the spec to support PC1.5 2Tx. It was a time-consuming work in terms of the PC2 2Tx FDD discussion in Rel-18/19. Thus to learn a lesson from PC2 2Tx FDD, we think it would be better to come up with a general approach to defined the RSD requirements from PC3 to PC1.5.

Thus we have the following proposals:

Proposal 8: It is proposed that RSD=0 dB from PC3 to PC1.5 for PC1.5 2Tx FDD band n1.

Proposal 9: It is proposed to study a general approach to defined the RSD requirements from PC3 to PC1.5 for all PC1.5 2Tx FDD bands.

- Conclusion

Observations:

Observation 1. Reusing legacy PC1 ans introducing a new power class will cause different workload in RAN4. The RF requirements discussion would largely depend on which power class is adopted.

Proposals for 4x26dBm HPUE

- Proposal 1: The requirements of Frequency error, Transmit OFF power, Transmit ON/OFF time mask, Transmit modulation quality, Transmit intermodulation keep unchanged for 4x26dBm PA configurations, regardless of which power class is adopted.

- Proposal 2: There are no changes for Rx requirements for 4x26dBm PA configurations, regardless of which power class is adopted.

- Proposal 3: For the requirements depends on sum of power from all UE transmit antenna connectors, e.g. MPR/A-MPR, OOB emission requirements, it should be postponed until RAN4 decides the power class.

Proposal 4: The requirements for TxD can be derived from UL MIMO requirements.

Proposal 5: To achieve 31dBm or 32dBm maximum output, only ULFPTx mode-1 is considered.

Proposals for PC1.5 FDD HPUE

Proposal 6: The MPR requirements of PC1.5 2Tx requirements for TDD bands can be applied to FDD bands for PC1.5 UE with 2Tx.

Proposal 7: A-MPR needs to be discussed for PC1.5 2Tx for FDD bands.

Proposal 8: It is proposed that RSD=0 dB from PC3 to PC1.5 for PC1.5 2Tx FDD band n1.

Proposal 9: It is proposed to study a general approach to defined the RSD requirements from PC3 to PC1.5 for all PC1.5 2Tx FDD bands.

- Reference

- RP-252962 New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109, Beijing, CN, Sept. 2025.

- R4-25013925 Discussion on general aspects for 4Tx HPUE, ZTE, RAN4#116bis, Prague, CZ, Oct. 2025.