3GPP TSG-RAN WG4 Meeting #116bis                                                                      R4-2513934

Prague, CZ, 13th-17th Oct., 2025

Title:	LS to RAN5 on testability for LP-WUR

Response to:	-

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN4

To:	RAN5

Cc:	-

Contact person:	liu.ke10@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN4 has finished the core part discussion on LP-WUS/WUR and decided to use 1% miss detection rate (MSD) as performance metric for LP-WUS conformance test. Considering that LP-WUR is not able to feedback to NW/TE directly, LP-WUR can be tested based on main receiver’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods). RAN4 would like RAN5 to design the details of the test with consideration of following suggestions from RAN4:

-  For the case when LP-WUR supporting both OOK waveform and OFDM waveform, only test with OFDM waveform could be performed.

-  For the case when LP-WUR supporting RRC_IDLE/INACTIVE state only or RRC_CONNECTED state only, the test should be done accordingly.

-  For the case when LP-WUR supporting both RRC_IDLE/INACTIVE state and RRC_CONNECTED state, specific test mode could be designed by RAN5.

    - Testing time reduction of LP-WUR RF test cases could be considered.

-  The following settings could be used for Rx requirements related testing:

    - LP-WUS, LPSS and SSB share the same EPRE.

    - LPSS shares the same chip rate and same sequence length as LP-WUS.

    - LPSS shares the same length in OFDM symbols as LP-WUS.

    - LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition.

    - SSB is transmitted at 20 ms periodicity.

The above information are just suggestions from RAN4, it is up to RAN5 to decide the final test issue.

2	Actions

To RAN5

ACTION:  RAN4 respectfully asks RAN5 to take above suggestions into account.

3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 Meeting #117		17 – 21 November 2025				Dallas, US

TSG-RAN WG4 Meeting #118		9 – 13 February 2026				Gothenburg, SE