3GPP TSG RAN WG4 Meeting #116-bis    		                          R4-2513940

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			7.6.4.2

Source:			Samsung

Title:			Discussion on HD-FDD for NTN Core Part

Document for:		Discussion

Introduction

In the RAN#109, the new WID [1] was approved for studying enhanced requirements for NR NTN and IoT NTN Phase 2. The WID includes HPUE for NTN, HD-FDD for Ka/Ku bands, and DL CA for intra-band Intra-SAN. This contribution discusses the impact of HD-FDD for FR1-NTN core part operation on RAN4 RRM.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

- Discussion

In the RAN4#116 RRM meeting [2], the impact on NR handover due to HD-FDD was analyzed. The conclusion is that HD-FDD has no spec impact on NR handover. Considering that VSAT with FR1-NTN does not introduce any special characteristics for handover, reusing the previous conclusion is recommended.

Issue 1-3-2: The impact on NR Handover due to HD-FDD - Proposal 1 (CATT, Xiaomi, CMCC, Samsung, Nokia, ZTE): For the impact on NR handover including HO, CHO and satellite switch for HD-FDD, the principle in option 1 is accepted. - Concern: HW, Nokia - Proposal 2 (HW, CMCC, Nokia, CATT): Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE. - Concern: No company. Agreement: No spec impact

Proposal 1: RAN4 to reuse the conclusion that HD-FDD VSTA with FR1-NTN has no spec impact on NR handover.

RAN1 has specified the principle for collision handling for HD-UE in NTN. RNA4 has further clarified the requirement of collision handling for HD-UE in NTN based on the RAN1 principle. HD-FDD VSAT Phase 1 is based on RedCap FR1-NTN using HD-FDD in paired spectrum. According to the big CR of HD-FDD redcap UE in FR1-NTN [3], the relevant RRM requirements affected by HD-FDD are listed as follows:

- Minimum requirement for CSI-RS based RLM/BFD

- Minimum requirement for L1 indication based RLM/BFD

- Scheduling availability of UE during RLM/BFD

- Scheduling availability of UE performing measurements

- CSI-RS based L1-RSRP Reporting

- Requirements Applicability of UE Rx-Tx time difference measurements

For the specific impact on above RRM requirements, RAN4 should consider the requirement applicability for HD-FDD VSAT UE with FR1-NTN bands.

Proposal 2: RAN4 to discuss whether the legacy requirements for HD-FDD redcap UE in FR1-NTN can be reused for HD-FDD VSAT UE in FR1-NTN, and the related RRM requirements that may be analyzed are listed below:

- Minimum requirement for CSI-RS based RLM

- Minimum requirement for L1 indication based RLM/BFD

- Scheduling availability of UE during RLM/BFD

- Scheduling availability of UE performing measurements

- CSI-RS based L1-RSRP Reporting

- Requirements Applicability of UE Rx-Tx time difference measurements

Proposal 3: RAN4 to consider the requirement applicability for HD-FDD VSAT UE with FR1-NTN bands.

In the current specs, "nonTerrestrialNetwork-r17" is used to indicate that UE support NTN capabilities, and "hd-FDDRedCap-r17" is used to indicate that the UE support HD-FDD RedCap capabilities. In the last meeting, RAN4 defined a new UE capability () for (e)RedCap UE with NTN. However, the spec lacks implicit or explicit indication of UE capability for VSAT UE with NTN. So RAN4 need to define new UE capabilities for HD-FDD VSAT UE with NTN.

Issue 1-4-1: Whether and how to design the UE capabilities for (e)RedCap UE with NTN - Proposal 1: RAN4 need to decide that whether Rel-19 requirements defined for (e)RedCap UE with NTN will apply for R17/18 UEs declaring support for RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support for NTN (nonTerrestrialNetwork-r17) respectively. - Option 1: Apply (E///, Samsung, vivo) - Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - FFS: The Rel-19 requirement can be release independent from Rel-17. - FFS: The LS to RAN2 for necessary UE capability description update to enable (e)RedCap UE with FR1-NTN and the corresponding CR to TS 38.307 are needed. - The release independent approach is not applied to Rel-19 new RAN1 UE features. - Option 2: Does not apply (Nokia) - Option 2a: RAN4 to introduce new capability to declare support over RedCap over NTN. (Samsung, vivo, HW, Nokia, CATT, Apple, MTK) - Option 2b: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - FFS: The LS to RAN2 for necessary UE capability description update to enable (e)RedCap UE with FR1-NTN is needed. Agreement: RAN4 to introduce new capability to indicate the support over RedCap over NTN.

Proposal 4: RAN 4 to define new UE capabilities for HD-FDD VSAT UE with NTN.

Conclusion

This contribution discusses the impact of HD-FDD for FR1-NTN core part operation on RAN4 RRM. The following are the proposed points in this contribution.

Proposal 1: RAN4 to reuse the conclusion that HD-FDD VSTA with FR1-NTN has no spec impact on NR handover.

Proposal 2: RAN4 to discuss whether the legacy requirements for HD-FDD redcap UE in FR1-NTN can be reused for HD-FDD VSAT UE in FR1-NTN, and the related RRM requirements that may be analyzed are listed below:

- Minimum requirement for CSI-RS based RLM

- Minimum requirement for L1 indication

- Scheduling availability of UE during RLM

- Minimum requirements for CSI-RS based BFD

- Scheduling availability of UE during BFD

Proposal 3: RAN4 to consider the requirement applicability for HD-FDD VSAT UE with FR1-NTN bands.

Proposal 4: RAN 4 to define new UE capabilities for HD-FDD VSAT UE with NTN.

Reference

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2

[2] Chair notes RAN4#116 v09

[3] R4-2509295, Big CR for RRM requirements of NR NTN Phase 3