TSG-RAN Working Group 4 (Radio) meeting #116-bis	R4-2513943

Prague, EU, 13th to 17th October 2025

Source:	Ericsson

Title:	Overview of Rel-20 BS RF evolution WI phase 2

Agenda item:	7.2.2

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#116 in Bangalore) all findings from Rel-19 BS RF evolution WI related to co-location improvements were captured in a CR to TR 37.941 and big CR to 38.104 with introduction of SBFD was approved. Work related to re-evaluation of spurious emission got an extension and remaining work packages start conformance phase.

To evolve BS RF requirements in general and evolve SBFD support RAN agreed a new WID for Rel-20 with a second phase of BS RF evolution including further improvements for co-location requirements and evolution of SBFD requirements. The new work item (NR_BS_RF_req_evo_Ph2-Core) have two separate objectives: Enhancement of co-location requirements and SBFD BS to SBFD BS adjacent channel co-existence.

In this contribution we present an overview of the work packages and some observations on how to proceed the work at coming meetings.

# Discussion

At RAN#109 a new WI was created to resolve all technical issues related to co-location requirement in current specifications. The work starts at RAN4#116-bis in Prague and is supposed to run until end of Rel-20. The WI includes both RF core work and conformance work, as indicated in Table 2-1.

Table 2-1: Work plan

| Meeting | Date | RF core TU allocation | Performance TU allocation | Note |
| --- | --- | --- | --- | --- |
| RAN#109 | Sep 2025 |  |  | WID created |
| RAN4#116-bis | Oct 2025 | 0.5 |  | First RAN4 meeting, initial discussion |
| RAN4#117 | Nov 2025 | 0.5 |  | Determine simulation parameters |
| RAN#110 | Dec 2025 |  |  |  |
| RAN4#118 | Feb 2026 | 1.0 |  |  |
| RAN#111 | Mar 2026 |  |  |  |
| RAN4#118-bis | Apr 2026 | 1.0 |  | Simulation phase start |
| RAN4#119 | May 2026 | 1.0 |  |  |
| RAN#112 | Jun 2026 |  |  |  |
| RAN4#120 | Aug 2026 | 1.0 |  |  |
| RAN#113 | Sep 2026 |  |  | Requirement derivation phase |
| RAN4#120-bis | Oct 2026 | 1.0 |  |  |
| RAN4#121 | Nov 2026 | 1.0 |  | Last RAN4 for BS RF core |
| RAN#114 | Dec 2026 |  |  |  |
| RAN4#122 | Jan 2027 | 0.5 | 0.5 | Start of BS conformance |
| RAN#115 | Mar 2027 |  |  |  |
| RAN4#122-bis | Apr 2027 |  | 0.5 |  |
| RAN4#123 | May 2027 |  | 0.5 |  |
| RAN#116 | Jun 2027 |  |  |  |
| RAN4#124 | Aug 2027 |  | 0.5 | Last RAN4 meeting |
| RAN#117 | Sep 2027 |  |  | WI closure |

Observe that the time allocation is split between two work packages with different objectives running in parallel during the whole WI. The two work packages are Enhancement of co-location requirements and SBFD BS to SBFD BS adjacent channel co-existence, which both aim to derive new requirements based on simulation campaigns and define new test requirements.

Both work packages are building on previous work in previous release. It is expected that simulation assumptions and conclusion from outcome from previous release (e.g. simulation assumptions, models, simulation/measurement results) should be considered in Rel-20.

Observation 1: Both work packages include simulation activities which would require careful planning considering the limited TU allocation.

  - Enhancement of co-location requirements

The bands above 3 GHz in NR FR1 are becoming more important since network operators often select AAS BS for networks when new bands are adopted. For AAS BS RF specifications a number of technical issues related to NR BS type 1-O requirements have been identified. The current requirement concept using a Co-Location Reference Antenna (CLRA) and corresponding Co-Location Test Antenna (CLTA) is attached with some technical issues related to guaranteed robustness, CLTA availability, test repeatability. To make NR BS type 1-O more useful and relevant for AAS BS operating in the upper bands of FR1, RAN4 need to investigate and enhance the co-location requirements.

Work package objective:

- Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3 GHz (e.g. band 42, 43, n48, n77, n78, n79 and n104).

- Evaluate outcome from Rel-19 BS-to-BS isolation findings captured in TR 37.941, clause 6.4.4 as baseline. If needed, evaluate coupling loss based on simulation and measurements for situations where Rel-19 findings are not complete.

- Consider impact on the following OTA co-location requirements:

  - Transmitter co-location spurious emission requirement

  - Receiver co-location out-of-band blocking requirement

  - TDD OFF power requirement

  - Transmitter intermodulation requirement

- Enhance OTA co-location requirements and improve the core requirements based on reasonable expected BS-to-BS coupling loss assumption for bands in the upper region of FR1. Two options will be considered:

  - Option 1: Investigate and if possible, improve current CLRA requirement concept.

  - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any.

Observation 2: For co-location enhancement work package RAN4 to be able to determine a reasonable coupling loss assumption per requirement a simulation campaign accomplished with some measurement would be required.

More details on work required for this objective can be found in a companion contribution in [2].

  - SBFD BS to SBFD BS adjacent channel co-existence

SBFD (Sub-band Full Duplex) is an evolution technique of TDD in the 5G-Advanced. In Rel-19 TDD BS to SBFD BS co-existence study was conducted and requirements was defined in TS 38.104 and TS 38.141-1/2. In Rel-20 a work package to evolve the specifications to also support co-existence between networks where both operators are using SBFD was approved.

Compared to TDD BS to SBFD BS co-existence case, ACIR in SBFD BS to SBFD BS co-existence case could be improved and better co-existence performance might be achieved, because of the nature of SBFD design where the SBFD downlink of operator B can be adopted as guard band for filter implementation to protect the SBFD uplink of operator B from adjacent operator A.

Therefore, in Rel-20 it is proposed to specify BS TX and RX RF requirement for SBFD-capable BS for the scenario of SBFD BS to SBFD BS adjacent channel coexistence.

Work package objective:

- Evaluate SBFD BS to SBFD BS adjacent channel co-existence:

  - Focus on FR1 WA BS network deployment.

  - Consider both scenarios including SBFD capable BS being co-located (0% grid-shift) and co-existence (5% grid-shift).

  - Focus on bands: n41 (2496 to 2690 MHz), n79 (4400 to 5000 MHz), n104 (6425 to 7125 MHz).

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel co-existence. The BS RF requirements include:

    - Transmitter unwanted emission requirement 1, 2

    - Receiver in-band blocking requirement 1, 2

Note 1: Whether to comply with new requirements is declared by BS manufacturer.

Note 2: The existing TDD requirements apply for non-SBFD BSs.

Observation 3: For SBFD co-existence work package a simulation campaign is required to be able to establish relevant requirements. It is essential that relevant models and parameters are adopted for the simulation campaign.

Observation 4: For SBFD co-existence work package it is also worth to align parameters and models with previous work in Rel-19, to maintain scope of the simulation campaign reasonable and result relevant for Rel-19 SBFD requirements.

For the case when co-location is considered, the free space path loss model (or any other far-field based model) can not be used. Instead, input from the co-location enhancement work within the same WI should be considered. The information can be used to establish coupling results for the co-located case.

Observation 5: The work package related to SBFD co-existence may need input from the work package for improving co-location requirements.

More details on work required for this objective can be found in a companion contribution in [3].

# Conclusion

In this contribution we provide an overview of the time schedule and included objectives for the new work item.

Based on the information we observe following:

Observation 1: Both work packages include simulation activities which would require careful planning considering the limited TU allocation.

Observation 2: For co-location enhancement work package RAN4 to be able to determine a reasonable coupling loss assumption per requirement a simulation campaign accomplished with some measurement would be required.

Observation 3: For SBFD co-existence work package a simulation campaign is required to be able to establish relevant requirements. It is essential that relevant models and parameters are adopted for the simulation campaign.

Observation 4: For SBFD co-existence work package it is also worth to align parameters and models with previous work in Rel-19, to maintain scope of the simulation campaign reasonable and result relevant for Rel-19 SBFD requirements.

Observation 5: The work package related to SBFD co-existence may need input from the work package for improving co-location requirements.

Based on the observations we propose following:

Proposal: Initially split meeting time equally between the two work packages.

# References

[1]	RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, Moderator (Huawei)

[2]	R4-2513942, “On enhancement of BS co-location requirements”, Ericsson

[3]	R4-2513969, “On SBFD-SBFD co-existence”, Ericsson