3GPP TSG-RAN WG4 Meeting #116bis				R4-2513953

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.11.2

Source: 	CAICT

Title: 	Discussion on Rel-19 RAN4 UE feature list for NR_AIML_air

Document for:	Discussion

- Discussion

In RAN4 115, the following agreement is UE feature related [1]:

Issue 2-3: Rx beam knowledge and TCI handling

Agreement:

- if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known.

- If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown.

- Known Rx beam means no additional Rx beam sweeping is needed

- Unknown Rx beam means additional Rx beam sweeping is needed.

- Rx beam is known mean TCI state is known if conditions below are also met

The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis.

- The target TCI state is known when the following is met:

  - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1  RSRP report of the target TCI state (clause 8.10.2 of 38.133)

  - Applies for case 1

  - FFS for case 2

  - Whether RS resource is only from set B should be clarified

    - FFS on whether prediction also impacts if a TCI state is known or unknown

      - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown

      - FFS on the time delay if prediction is used as a condition for a known TCI state

Further agreements on TCI state handling in last meeting [2]:

Issue 2-3: TCI State Handling

Agreement:

- On Detectability and SNR conditions:

  - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command

  - The TCI state remains detectable during the TCI state switching period

  - The SSB associated with the TCI state remain detectable during the TCI switching period

    - SNR of the TCI state ≥ -3 dB

- Time conditions:

  - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement

- Agreement on QCL relationship

  - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

  - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

In last meeting, Rel-19 NR_AIML_air UE feature was included in Rel-19 UE feature list [3]. The details of Rel-19 NR_AIML_air UE feature should be further discussed and concluded.

We provide the detail information in the following table as starting point for further discussion:

59. NR_AIML_air

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_AIML_air | [59-1-x] | UE-side beam prediction for BM Case1 | [Knows RX beam of predicted TX beam in setA in BM-case 1 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-2 (RAN1 feature for BM-case-1) | yes | N/A | [In BM-case 1, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state.] | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |
|  | [59-1-y] | UE-side beam prediction for BM Case2 | [Knows RX beam of predicted TX beam in setA in BM-case-2 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-4 (RAN1 feature for BM-case-2) | yes | N/A | In BM-case 2, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state. | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |

Proposal 1: Use the proposed table as starting point for Rel-19 NR_AIML_air UE feature discussion.

- Conclusion

Proposal 1: Use the proposed table as starting point for Rel-19 NR_AIML_air UE feature discussion.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_AIML_air | [59-1-x] | UE-side beam prediction for BM Case1 | [Knows RX beam of predicted TX beam in setA in BM-case 1 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-2 (RAN1 feature for BM-case-1) | yes | N/A | [In BM-case 1, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state.] | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |
|  | [59-1-y] | UE-side beam prediction for BM Case2 | [Knows RX beam of predicted TX beam in setA in BM-case-2 if the predicted Tx beam is not QCL Type-D to a known TX beam] | Pre-requisitng FG: 58-1-4 (RAN1 feature for BM-case-2) | yes | N/A | [In BM-case 2, network needs to transmit additional samples of reference signal corresponding to the predicted TX beam of setA so that UE can sweep its RX beams during TCI state switch process when the RS resource for predicted Tx beam is the RS in target TCI state or QCLed to target TCI state.] | [Per Band] | [TDD] | [FR2-1 only] | N/A |  | [Optional with capability signaling] |

- Reference

[1] R4-2508080	WF on NR_AIML_air_part1, Qualcomm

[2] R4-2511890	WF on AI/ML for air interface  [116][131], Qualcomm

[3] R4-2511883	R19  RAN4 UE feature list for NR CMCC