3GPP TSG-RAN WG4 Meeting #116bis	R4-2513970

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.20.4

Source:	Ericsson

Title:	SAN demodulation requirements for IoT-NTN

Document for:	Discussion

# 1	Introduction

RAN4#116 agreed with the way forward on Rel-19 IoT-NTN phase 3 SAN demodulation requirements [1], where RAN4 will define NPUSCH format 1 demodulation requirements with the assumption the SAN receives NPUSCH format 1 from two UEs simultaneously. This contribution provides our simulation result to define the NPUSCH format 1 demodulation requirements.

# 2	Discussion

## 2.1	Simulation results

Table 1 shows our simulation result of NPUSCH format 1 with OCC2 for alignment. Since RAN4#116 agreed to measure NPUSCH format 1 throughput per UE, the summary shows SNR to achieve 70% of the maximum throughput per UE (UE1 and UE2). Our simulation result observes that the difference of SNRs between two UEs is up to 0.1dB. Therefore, it is feasible to set the same NPUSCH format 1 demodulation requirements for both the UEs.

Table 1	NPUSCH format 1 with OCC-2 simulation results for alignment.

| Tx antennas | Rx antennas | SCS | Sub-carriers | Propagation channel | FRC | Repetition number | SNR to achieve 70% of the peak rate (from UE1) | SNR to achieve 70% of the peak rate (from UE2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 2 | -4.12 | -4.17 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 2 | -7.41 | -7.38 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 8 | -13.14 | -13.24 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 8 | -15.13 | -15.19 |

Observation 1: Difference of SNRs for NPUSCH format 1 to achieve 70% of the maximum throughput between two UEs is up to 0.1dB.

Proposal 1: Set the same SNR for NPUSCH format 1 with OCC2 from two UEs.

## 2.2	How to specify NPUSCH format 1 with OCC2 demodulation requirement

The remaining open issue is how to specify the requirement of NPUSCH format 1 with OCC2. Since RAN4 has agreed to transmit NPUSCH format 1 with OCC sequence indices 0 and 1 simultaneously and measure the SNR to achieve 70% of the maximum throughput per UE (OCC sequence index), we propose to specify the SNR for each OCC sequence index as shown in Table 2 and Table 3 for 3.75kHz single tone and 15kHz single tone, respectively.

Proposal 2: RAN4 specify the SNR to achieve 70% of the maximum throughput of NPUSCH format 1 for OCC sequence indexes 0 and 1, which correspond to one UE and another UE, respectively.

Table 2	Example of NPUSCH format 1 with OCC2 demodulation requirements for 3.75kHz single tone.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | OCC sequence index | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| Note 1: NPUSCH format 1 with OCC sequence indices 0 and 1 are transmitted simultaneously. |  |  |  |  |  |  |  |  |  |  |  |

Table 3	Example of NPUSCH format 1 with OCC2 demodulation requirements for 15kHz single tone.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | OCC sequence index | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128 Hz | 256 | 8 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128 Hz | 256 | 8 | 0 | 70% | TBD |
|  |  |  |  |  |  |  |  |  | 1 | 70% | TBD |
| Note 1: NPUSCH format 1 with OCC sequence indices 0 and 1 are transmitted simultaneously. |  |  |  |  |  |  |  |  |  |  |  |

# 3	Summary

Observations:

Observation 1: Difference of SNRs for NPUSCH format 1 to achieve 70% of the maximum throughput between two UEs is up to 0.1dB.

Proposals:

Proposal 1: Set the same SNR for NPUSCH format 1 with OCC2 from two UEs.

Proposal 2: RAN4 specify the SNR to achieve 70% of the maximum throughput of NPUSCH format 1 for OCC sequence indexes 0 and 1, which correspond to one UE and another UE, respectively.

# References

- R4-2512645, “Way Forward for [116][332] IoT_NTN_Ph3_demod”, ZTE Corporation.

# Appendix

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Table A.16.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 | A16-3 | A16-4 | A16-5 | A16-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 | 15 | 15 | 15 |
| Number of allocated subcarriers | 1 | 1 | 3 | 6 | 12 | 12 |
| Diversity | No | No | No | No | No | No |
| Modulation | BPSK | BPSK | QPSK | QPSK | QPSK | QPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 | 3 / 0 | 7 / 0 | 9 / 0 | 5/4 |
| Payload size (bits) | 32 | 32 | 40 | 104 | 136 | 424 |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 | 5 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 2/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.22 | 0.44 | 0.56 | 0.34 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 288 | 288 | 288 | 1440 |
| Total symbols per resource unit | 96 | 96 | 144 | 144 | 144 | 720 |
| Channel estimation length (ms) Note 1 | 16 | 4 | 4 | 4 | 2 (when repetition = 2) 4 (when repetition > 2) | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |  |