3GPP TSG-RAN WG4 Meeting #116bis	R4-2513976

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.2.3

Source:	Ericsson

Title:	UE demodulation requirements for non-colocated CA Phase 2

Document for:	Discussion

# Introduction

This contribution discusses the UE demodulation requirements for the intra-band non-collocated EN-DC/NR-CA deployment Phasae2 [1].

Specify the corresponding performance requirements for type 4a/4b capable FWA UE. - RRM performance requirements - PDSCH demodulation performance requirements

# Discussion

## Rel-19 Type 4 UE

In Rel-18, RAN4 introduced Type 2 UE where UE can receive two component carriers in the intra-band EN-DC/NR-CA with the following capabilities:

- Reception power difference between 2 CCs is up to 25dB,

- Reception timing difference between 2 CCs (MRTD) is up to 33us, and

- Maximum MIMO layers are up to 2 for each CC.

In Rel-19, 3GPP approved a WI to specify ‘Type 4’ UE [1], where the capabilities of reception power difference and timing difference are the same as Type 2 UE, but the maximum MIMO layers per CC is up to 4.

According to WI summary [2], it is assumed the Type 4 UE is operated in the following band combinations, which is same as Rel-18.

- EN-DC: DC_42_n77 and DC_42_n78

- NR-CA: CA_n77 and CA_n78

Type 4 UE consists of two sub-types according to the RF architecture, called Type 4a and Type 4b, as summarized in Table 1. UE Type 4a is applicable only for EN-DC; it can receive the DL signal with 4 receive antennas for NR carrier, but with 2 receive antennas for LTE carrier. On the other hand, UE Type 4b is applicable for both EN-DC and NR-CA; it can receive the DL signal with 4 receive antennas for both the carriers.

Table 1	Type 4a/4b UE RF architecture (written based on [3]).

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4a | 1 (NR) | 4 | 4 | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and LNA => FWA only |
|  | 2 (LTE) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 4 | 4 | 4Rx | NR-DC EN-DC | 25dB full range | Requires 8 antennas and LNA => FWA only |
|  | 2 (LTE/NR) | 4 | 4 | 4 | 4Rx |  |  |  |

In Rel-18 WI, RAN4 defined UE demodulation requirements for Type 2 UE for NR-CA scenario only, because UE demodulation requirements in TS 38.101-4 specify the UE demodulation performance of NR carrier only. Following the conclusion on the Rel-18 UE demodulation performance, RAN4 only needs to define Type 4 UE demodulation requirements for NR-CA, which implies RAN4 should focus on Type 4a.

Observation 1: In Rel-18, RAN4 defined Type 2 UE PDSCH demodulation requirements for TDD+TDD NR-CA scenario.

Proposal 1: Define Type 4a UE PDSCH demodulation requirement for 2CC NR-CA scenario, with the configuration FR1 TDD SCS=30kHz + FR1 TDD SCS=30kHz.

In Rel-18 Type 2 UE demodulation requirements, RAN4 set the following configurations:

- CC#1: Rank 1 and MCS4 in 64QAM table,

- CC#2: Rank 2 and MCS21 with 256QAM MCS table, and

- Timing offset of the SCell from the PCell to 33us.

Observation 2: Rel-18 Type 2 UE demodulation requirement configures rank 1 with MCS4 in 64QAM table (QPSK 0.30) on for one carrier and rank 2 with MCS21 in 256QAM table (256QAM 0.69) for another carrier.

Observation 3: Rel-18 Type 2 UE demodulation requirement configures the timing offset of the SCell from the PCell to 33us.

We propose to reuse the Type 2 configuration except for MCS and MIMO layers. According to the WID, we propose to configure 3 or 4 MIMO layers for at least one carrier.

Proposal 2: RAN4 defines Type 4 UE demodulation requirement, where at least one CC configures rank 3 or rank 4.

Proposal 3: RAN4 defines Type 4 UE demodulation requirement where the timing offset of the SCell from the PCell is 33us.

## UE demodulation requirements for Type 4 UE

For Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank of two CCs so that the difference of two SNRs is less than 25dB, where the SNRs are average of companies’ impairment results [4]. We propose using the same methodology to derive MCS/rank and SNR to define Type 4 UE demodulation requirements.

Observation 4: In Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank so that the difference of SNRs of two CCs is less than 25dB.

Proposal 4: RAN4 sets the MCS/rank of two CCs for Type 4 UE demodulation requirements so that the difference of SNRs of two CCs is less than 25dB, where the SNRs are average of companies’ impairment results.

Table 2 and Table 3 show our simulation results to list the MCS/rank candidates for Type 4 UE demodulation requirement. In Rel-18, RAN4 set rank 2 with MCS21 in 256QAM table for one CC and rank 1 with MCS4 in 64QAM table for another CC. Since Type 4 UE can use 4Rx compared with 2Rx, one candidate option is rank 4 with MCS21 in 256QAM table for one CC and rank 2 with MCS4 in 64QAM table for another CC. According to our simulation result, the difference between two SNRs is 24.5dB, which is within 25dB. Note, in our simulation, we set Tx EVM to 3% and static channel model for both the cases according to the Rel-18 assumption.

Table 2	PDSCH simulation results (for alignment) for CC#1.

| Rank | MCS (64QAM table) | SNR to achieve 70% of maximum throughput (ideal results) |
| --- | --- | --- |
| 1 | 4 | -7.0 |
|  | 5 | -6.2 |
| 2 | 4 | -4.3 |
|  | 5 | -3.3 |

Table 3	PDSCH simulation results (for alignment) for CC#2.

| Rank | MCS (256QAM table) | SNR to achieve 70% of maximum throughput (ideal results) |
| --- | --- | --- |
| 4 | 21 | 20.2 |
|  | 22 | 21.2 |
|  | 23 | 22.7 |
|  | 24 | 23.6 |
|  | 25 | 25.3 |
|  | 26 | 24.9 |

Proposal 5: Assume Tx EVM=3% for both QPSK and 256QAM as same assumption as Rel-18 Type 2 UE demodulation requirements.

Proposal 6: Configure CBW/SCS=40MHz/30kHz and 4x4 static channel in TS 38.101-4 B.1.1.

Proposal 7: RAN4 should evaluate the following MCS/rank combinations for Type 4 UE demodulation requirements: rank 2 with MCS4/MCS5 in 64QAM MCS table for CC#1 and rank 4 with MCS21/MCS22 in 256QAM MCS table for CC#2.

Table 4 summarizes our proposal for Type 4 UE demodulation requirements.

Table 4: UE demodulation requirement for Type 4 UE in non-colocated NR-CA scenario.

| Carrier number | CBW/SCS | MCS | MCS table | Rank | Propagation condition | Antenna configuration | Metric | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CC#1 | 40MHz / 30kHz | 4 or 5 | 64QAM MCS table | 2 | Static channel specified in Annex B.1.1 | 4x4 | 70% of maximum throughput | SNR1 |
| CC#2 | 40MHz / 30kHz | 21 or 22 | 256QAM MCS table | 4 | Static channel specified in Annex B.1.1 | 4x4 | 70% of maximum throughput | SNR2 (<SNR1+25dB) |
| Note 33us timing difference between PCell and SCell. |  |  |  |  |  |  |  |  |

# Summary

Observations:

Observation 1: In Rel-18, RAN4 defined Type 2 UE PDSCH demodulation requirements for TDD+TDD NR-CA scenario.

Observation 2: Rel-18 Type 2 UE demodulation requirement configures rank 1 with MCS4 in 64QAM table (QPSK 0.30) on for one carrier and rank 2 with MCS21 in 256QAM table (256QAM 0.69) for another carrier.

Observation 3: Rel-18 Type 2 UE demodulation requirement configures the timing offset of the SCell from the PCell to 33us.

Observation 4: In Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank so that the difference of SNRs of two CCs is less than 25dB.

Proposals:

Proposal 1: Define Type 4a UE PDSCH demodulation requirement for 2CC NR-CA scenario, with the configuration FR1 TDD SCS=30kHz + FR1 TDD SCS=30kHz.

Proposal 2: RAN4 defines Type 4 UE demodulation requirement, where at least one CC configures rank 3 or rank 4.

Proposal 3: RAN4 defines Type 4 UE demodulation requirement where the timing offset of the SCell from the PCell is 33us.

Proposal 4: RAN4 sets the MCS/rank of two CCs for Type 4 UE demodulation requirements so that the difference of SNRs of two CCs is less than 25dB, where the SNRs are average of companies’ impairment results.

Proposal 5: Assume Tx EVM=3% for both QPSK and 256QAM as same assumption as Rel-18 Type 2 UE demodulation requirements.

Proposal 6: Configure CBW/SCS=40MHz/30kHz and 4x4 static channel in TS 38.101-4 B.1.1.

Proposal 7: RAN4 should evaluate the following MCS/rank combinations for Type 4 UE demodulation requirements: rank 2 with MCS4/MCS5 in 64QAM MCS table for CC#1 and rank 4 with MCS21/MCS22 in 256QAM MCS table for CC#2.

# References

- RP-240838, “Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)”, KDDI.

- RP-252837, “WI summary for WI Core part: Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)”, KDDI.

- R4-2303696, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI.

- R4-2401752, “Summary of simulation results for UE demodulation requirements for non-colocated NR-CA deployment scenario”, Ericsson.

# Appendix

## Type 4 UE types

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4a | 1 (NR) | 4 | 6 total | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and FWA |
|  | 2 (LTE/NR) | 2 |  | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 8 total | 4 | 4Rx | NR-CA EN-DC | 25dB full range | Requires 8 antennas and LNA |
|  | 2 (LTE/NR) | 4 |  | 4 | 4Rx |  |  |  |