# 3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513986

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	4.21.1

Source: 	Ericsson

Title:	Remaining issues on Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, the Rel-19 NES enhancement WI has completed. There are some of issues mentioned can be discussed in maintenance stage [1]. In this contribution, we will discuss these maintenance issues on Rel-19 NES.

- OD-SSB

Interruption

In last meeting, RAN4 agreed the interruption requirement directly in the CR. However, we noticed that the interruption occasions and length are not aligned in different sections. From our understanding, when UE supports Rel-19 OD-SSB capability, UE shall follow the OD-SSB periodicity. In other words, UE doesn’t need to wait the SMTC and the max SMTC among active serving cells to perform the SCell activation.

In Rel-15, the interruption is extended with SMTC duration due to the AGC retuning should based on the SMTC occasions together with all the active serving cells [2]. When UE supports Rel-19 OD-SSB capability, UE shall response the OD-SSB immediately. The related RF retuning action should also follow the same method, and UE needs to retune the AGC based on OD-SSB directly. Therefore, the OD-SSB based SCell activation interruption for intra-band should follow the same principles of inter-band SCell activation.

RAN4 #113 Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another iss

Proposal 1: RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

- OD-SIB1

In RAN4 #116 meeting, RAN4 agreed one issue related OD-SIB1 can be discussed in maintenance stage as follow.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 RAN4 agreements in R4-2508441 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.. Agreement on Ad hoc: - Keep current agreement and if companies think additional requirements are needed discuss in maintenance stage.

From our understanding, current agreement for OD-SIB1 on paging interruption has clear captured in the spec as follow.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. The interruption time could be longer due to on demand SIB1 acquisition if the target NR cell is an NES cell and UE is capable of on demand SIB1 acquisition. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. If the target NR cell is an NES cell and UE is capable of on demand SIB1 acquisition, TSI-NR could be longer due to UL WUS transmission and RAR reception for on demand SIB1 acquisition of NES cell.

Proposal 2: RAN4 to insist on the previous agreement about OD-SIB1 and no new change is needed.

- Summary

In this contribution, we have discussed the remaining issues of NES enhancement requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

Proposal 2: RAN4 to insist on the previous agreement about OD-SIB1 and no new change is needed.

- References

- R4-2508277, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- R4-1810013, “Misalignment Issue on SCell Activation Delay and Interruption requirements”, MediaTek Inc.