3GPP TSG-RAN4 Meeting #116bis	R4-2513994

# Prague, Czech Republic 13th – 17th October 2025

Title:	Views on SBFD-to-SBFD coexistence

Source:	Qualcomm Incorporated

Agenda item:	7.2.4

Document for:	Approval

# Introduction

Following the Rel20 BS RF enhancement WI [1] approved in RAN#109, we have the following objective regarding SBFD-capable BS to SBFD-capable BS in the adjacent channel:

SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

In this paper, we provide in this paper our preliminary views on SBFD BS SBFD coexistence aspects envisioned in RAN4 during Release 20.

# General discussion

During Rel-18 study item [2] and Rel-19 work item [3] on SBFD-capable BS requirements, the focus within RAN4 on specifying different Tx and Rx requirements for SBFD-capable BSs given with the assumption that the adjacent channel is a legacy TDD BS. That assumption stems from the fact that in already existing NR bands, deployment of SBFD necessitates ensuring protection and coexistence with the already deployed TDD BSs. For adjacent channel coexistence between TDD and SBFD-capable BS, it was discussed and captured in [2] that the ACIR enhancement is limited by the ACLR of TDD BS. Coexistence was shown to be feasible for 100% grid-shift assumption, which was the baseline assumption during Rel-18 and Rel-19 in RAN4.

Observation 1: Work in Rel-18 and Rel-19 considered TDD to SBFD adjacent channel coexistence, which is in general limited by legacy TDD BS requirements, especially regarding ACIR enhancements.

In Rel-20 work item [2], the above objective is extended to account for cases of green-field spectrum deployments, where SBFD-capable BSs can be deployed with another SBFD-capable BS in the adjacent channel. Through such scope, RAN4 can quantify what is the exact ACIR needed in order to meet the SBFD UL degradation (i.e., case 3 in [2]) target without any restriction on existing TDD deployments. To quantify this, RAN4 needs to discuss if coexistence studies based on the framework and baseline considered in [2] could be leveraged at the start to define expected level of ACIR enhancement needed.

Proposal 1: RAN4 to discuss if adjacent channel coexistence framework in TR 38.858 could be leveraged to quantify expected level of ACIR enhancement needed to enable feasible SBFD-to-SBFD adjacent channel deployments.

In order to have efficient discussions in Rel-20 work, it would be viable if RAN4 could agree on which Tx and Rx requirements would be determined via the adjacent channel coexistence work and which ones might not need simulation work. By doing so, it would make a clear direction for RAN4 as well as make it have enough time to agree on controversial simulation parameters.

Proposal 2: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence.

# Conclusion

In this contribution, our observations and proposals for the Rel-20 SBFD-to-SBFD coexistence can be summarized as follows:

Observation 1: Work in Rel-18 and Rel-19 considered TDD to SBFD adjacent channel coexistence, which is in general limited by legacy TDD BS requirements, especially regarding ACIR enhancements.

Proposal 1: RAN4 to discuss if adjacent channel coexistence framework in TR 38.858 could be leveraged to quantify expected level of ACIR enhancement needed to enable feasible SBFD-to-SBFD adjacent channel deployments.

Proposal 2: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence.

# References

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Huawei (Moderator), RAN Meeting #109, Beijing, China.

- TR 38.858, Study on evolution of NR duplex operation, 3GPP.

- RP-240789 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon (Moderator), RAN Meeting #103.