3GPP TSG-RAN4 Meeting #116bis	R4-2513996

# Prague, Czech Republic 13th – 17th October 2025

Title:	Coexistence considerations for High Power UEs

Source:	Qualcomm Incorporated

Agenda item:	7.1.3

Document for:	Approval

# Introduction

Following the Rel20 UE RF enhancement WI [1] approved in RAN#109, we have the following objective regarding High power UE (HPUE) for NR single carrier operation:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

In this paper, we provide in this paper our preliminary views on the coexistence aspects related to HPUE Fixed Wireless Access (FWA) in RAN4 Rel-20.

# General discussion

## Background for PC1 ACLR derivation

Historically, PC1 was introduced in LTE Release 11 to address HPUE for public safety operations in the US for band 14 (i.e., 700 MHz) to enable higher achievable data rate with broader coverage, which are essential for Public Safety broadband systems. The coexistence study as well as details pertaining to specification of  the HPUE Tx and Rx requirements are documented in [2]. In this section, we will focus on the coexistence component and how the current ACLR (i.e., 37 dB) was derived as an outcome of the work in [2]. The adjacent channel coexistence investigated the impact of band 14 UL transmissions on the adjacent band 13 BS reception. The adopted framework considered the following methodology to derive the level of ACIR (i.e., band 14 HPUE ACLR and band 13 BS ACS) needed to ensure protection of band 13’s UL operation:

- Run the B14 UL to B13 UL coexistence study assuming power class 3 (+23dBm) UE is deployed in B14, obtain the B13 system performance degradation results as the baseline

- Run the B14 UL to B13 UL coexistence study assuming [+33dBm] power class UE is deployed in B14, obtain the B13 system performance degradation results

- Quantify the degradation resulting from HPUE band 13 operation and needed ACIR values.

Observation 1: Current PC1 ACLR requirement in TS 38.101-1 is based on the coexistence work carried out for the US band 14 public safety deployments, which is summarized in TR 36.837.

Observation 2: Band 14 ACLR was developed in RAN4 based on adjacent channel coexistence studies with public safety handheld devices in mind and not FWA/ CPEs.

Adjacent channel coexistence study in [2] followed LTE simulation framework given in TR 36.942 with some modifications to account for larger (rural) public safety deployments which are summarized below:

- Different cell sizes for the Band 14 and Band 13 deployments, to account for rural public safety deployments.

- Different power control sets/ configurations to investigate impact of UL power control on the adjacent channel interference

- HPUE was considered as typical handheld UE as the use case was public safety devices (i.e., no special consideration for FWA/ CPE characteristics).

In addition, the BS antenna assumptions in [2] assumed no beamforming/ AAS at the BS side as well as an omni-directional model at the HPUE side. In summary, given the different objectives between the work in [2] and the Release 20 objectives listed in [1], it is our view that evaluating the coexistence study with more updated system parameters and assumptions to determine the ACLR for FWA/CPE as given in [1] is the viable way forward in this work item.

Proposal 1: RAN4 to agree on new set of adjacent channel coexistence parameters to capture realistic deployments of FWA/ CPE devices for Rel-20 and beyond.

## FWA adjacent channel coexistence considerations

### Simulation scenarios

The objective in Release 20 work is to revisit the existing PC1 ACLR requirement in TS 38.101-1. In this regard, the FWA ACLR should be the target simulation goal which typically implies that the study must consider the impact of aggressor’s network FWA UL transmissions on victim’s network reception. From [1], we have the following TDD exemplary bands to be considered, n41, n77/n78, n79, n104. Additionally, we believe that RAN4 should consider the case of synchronized TDD operation, where the victim network’s UL transmission (i.e., victim device is the receiving BS) will be impacted by the aggressor’s network UL transmissions (i.e., aggressor device is the transmitting FWA UE). This means that asynchronized operation, i.e., impact of inter-UE or inter-BS cross link interference is out of scope at this stage.

Observation 3: Since FWA ALCR is the target requirement to investigate, the relevant scenario would be the one in which both victim and aggressor networks operate in UL, and the impact of FWA UE ACLR on the victim’s base station UL reception is assessed.

In addition to the transmission direction, it is important also to agree on victim and aggressor network device type assumptions. For the aggressor network, FWA/ CPE are considered. On the other hand, it is a viable direction to assume handheld UE for the victim network, since handheld is the typical UE device considered in RAN4 adjacent channel coexistence.

Proposal 2: RAN4 to consider FWA network UL transmissions as interference to victim network’s base station UL reception from handheld UEs for the Release 20 coexistence work.

### FWA Deployment and network layout

In contrast to the assumptions for band 14 studies [2], Release 20 considerations for PC1 deployments for FWA and CPE are targeted more for C-band deployments (i.e., exemplary bands n41, n77/n78, n79, n104 as mentioned in [1]). Accordingly, urban macro deployments are more suitable compared to rural deployments with large cell sizes (~ 8km as considered in [2]). Thus, we recommend RAN4 to consider hexagonal grid with 19 macro sites and 3 sectors per site with wrap around and an inter-side distance (ISD) of 750m in the coexistence study. In regard to the considered indoor and outdoor ratios, given FWA deployments, it is a viable assumption to consider FWAs are mixture of indoor and outdoor, since FWA are typically mounted on windows/ balconies, but also could be indoor to serve other indoor use cases. For the victim handheld network, the typical assumption of 70% outdoor while 30% indoor could be considered.

Proposal 3a: RAN4 to consider as a starting point UMa deployments with preliminary ISD = 750m.

Proposal 3b: RAN4 to consider as a starting point, a mixture of 70/30% indoor/outdoor ratio assumption for both aggressor network FWA UEs as well as victim network handheld UEs.

In addition to the indoor/outdoor ratio, height distribution of the FWAs is important to be discussed in RAN4. Typically for outdoor UEs, RAN4 assumes the UE height to be fixed at 1.5m, while for indoor UEs, it is uniformly distributed between 1.5m and 22.5m, following the UMa propagation model given in TR 38.901. For the FWA case, having all FWAs at a fixed height of 1.5m does not make much sense from practical deployment point of view. A more realistic approach would assume that the FWAs are deployed between 1.5m and 22.5m to account for possible FWA deployments at windows, rooftops, and balconies. 

Proposal 4: RAN4 to consider as a starting point for outdoor FWA deployment height a uniform distribution between 1.5m and 22.5m.

Regarding network layout, given that we are investigating multi-operator coexistence scenarios, it is plausible that RAN4 would investigate both coordinated (i.e., 0% grid shift) and un-coordinated layouts.

Proposal 5: RAN4 to consider both coordinated and un-coordinated network layouts.

### Beamforming assumptions

The outcome of the adjacent channel coexistence work can be heavily influenced by the beamforming assumptions at the UE and BS. In LTE studies for PC1 [2], no AAS was considered at the BS side and the beamforming gain consisted only of the single elemental gain. With AAS (i.e., active beamforming), the BS has the capability to direct energy towards directions of interest and minimize interference into unwanted directions. In addition, AAS parameters and underlying model for beamforming at the BS has been well studied in RAN4. One recent example is the work in TR 38.922, where RAN4 has studied the different RF parameters as well as the BS AAS parameters for three different frequency ranges (4 GHz, 8GHz, and 15 GHz) to provide a response to WP5D regarding WRC-27 sharing studies in ITU-R. Particularly, the 4.4 - 4.8 GHz parameters were based on defined band n79, and the different Tx and Rx requirements for both BS and UE as well as the BS AAS parameters were captured in [3]. Based on the aforementioned points, adopting the AAS parameters for the C-band parameters is a good starting point and RAN4 could further discuss if different parameters could be further investigated, e.g., more antenna elements at the BS compared to what was given in [3], while maintaining the same BS EIRP.

Proposal 6: RAN4 to consider BS AAS beamforming assumptions following the BS AAS parameters given in R4-2410576 for the FWA coexistence study.

Moving to the UE, an omni-directional model was assumed in the LTE study [2]. For C-band however, a more directive antenna model is more appropriate. For example, the element radiation pattern in 38.901 could be considered with half power BWs between 90 and 120 degrees.

Proposal 7: RAN4 to discuss antenna directivity for each of the FWA UE’s Tx chains and which model to be used in the coexistence study.

Recall also that the main assumption in [1] considers 4Tx for CPE/FWA UE with 4x26dBm PA configurations. It is plausible that the FWA/ CPE will exploit some design freedom in whether to favor spatial diversity or whether to focus on range. The impact of this design diversity can be captured by studying multiple FWA antenna arrangements.

One such example set is to study a pair of FWA antenna arrangements illustrated in the figure below. UE type A has one antenna at each of the 4 vertical faces of the simplified UE (1111 arrangement) and UE type B has its antennas clustered in 2020 arrangement. Other variants are also possible.

A

B

Example FWA topologies with directional antennas

Proposal 8: RAN4 to identify discrete antenna arrangements on the FWA UE targeted for the study.

Based on the above discussion points, we propose Table 1 as a starting point for the adjacent channel coexistence work in this work item.

Tabel 1 Adjacent channel coexistence simulation parameters

| Category | Parameter | Value |
| --- | --- | --- |
| Network configuration | ISD | 750m |
|  | CHBW | 100 MHz |
|  | Propagation model | UMa in 38.901 (Pathloss, LOS probability, and O-to-I penetration loss) |
|  | Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
|  | Victim network | TDD UL (UE to BS) with handheld PC3 |
|  | Victim Network Outdoor/Indoor Ratio | 30/70% |
|  | Aggressor network | TDD UL (UE to BS) with FWA (same BW as victim) |
|  | Multi-operator layout | Coordinated (0% Grid Shift), uncoordinated |
|  | Shadowing correlation | Between cells: 1.0, Between sites: 0.5 |
|  | Additional shadowing @ FWA | 0 dB |
|  | FWA/PC3 Ratio | 100/0% as a starting point |
| FWA configuration | Outdoor/Indoor ratio | 70/30% as a starting point |
|  | Low/ high penetration loss | 50% low loss and 50% high loss |
|  | FWA distribution | Uniform distribution within each sector |
|  | Height distribution | Uniform distribution between 1.5 and 22.5m |
|  | FWA-BS minimum distance | 35 m |
|  | Beamforming | TBD |
|  | Additional Losses | 0 dB |
| gNB configuration | AAS parameters | See R4-2410576 as a starting point |
|  | Antenna height | 25m |
|  | BS noise figure | 5 dB |

Proposal 9: RAN4 to adopt table 1 as a starting point for the adjacent channel coexistence work.

## Additional UE coexistence requirements

Apart from ACLR requirements, another aspect that RAN4 should investigate is the spurious emissions for UE co-existence, given in Table 6.5.3.2-1 in TS 38.101-1. For example, for band n41, the coexistence requirement for NR band n79 is -50 dBm/1MHz. Historically, such limit is based on early LTE work with conservative assumptions on the distance between UEs to be 0.7 to 1.5m depending on frequency band (See R4-080710 for more details).

Proposal 10: RAN4 to check spurious emissions for UE co-existence for investigated bands in this WI to make sure that the limits reflect practical deployments and updated assumptions.

# Conclusion

In this contribution, we provided our views on the coexistence aspects for the single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations. Our observations and proposals can be summarized as follows:

Observation 1: Current PC1 ACLR requirement in TS 38.101-1 is based on the coexistence work carried out for the US band 14 public safety deployments, which is summarized in TR 36.837.

Observation 2: Band 14 ACLR was developed in RAN4 with public safety handheld devices in mind and not FWA/ CPEs.

Proposal 1: RAN4 to agree on new set of adjacent channel coexistence parameters to capture realistic deployments of FWA/ CPE devices for Rel-20 and beyond.

Observation 3: Since FWA ALCR is the target requirement to investigate, the relevant scenario would be the one in which both victim and aggressor networks operate in UL, and the impact of FWA UE ACLR on the victim’s base station UL reception is assessed.

Proposal 2: RAN4 to consider FWA network UL transmissions as interference to victim network’s base station UL reception from handheld UEs for the Release 20 coexistence work.

Proposal 3a: RAN4 to consider UMa propagation model with preliminary ISD = 750m.

Proposal 3b: RAN4 to consider as a starting point, a mixture of 70/30% indoor/outdoor ratio assumption for both aggressor network FWA UEs as well as victim network handheld UEs.

Proposal 4: RAN4 to consider as a starting point outdoor FWAs height as uniformly distributed between 1.5m and 22.5m to account for possible FWA deployments.

Proposal 5: RAN4 to consider both coordinated and un-coordinated network layouts.

Proposal 6: RAN4 to consider BS AAS beamforming assumptions following the BS AAS parameters given in  R4-2410576 for the FWA coexistence study.

Proposal 7: RAN4 to discuss antenna directivity for each of the FWA UE’s Tx chains.

Proposal 8: RAN4 to identify discrete antenna arrangements on the FWA UE targeted for the study.

Proposal 9: RAN4 to adopt table 1 as a starting point for the adjacent channel coexistence work. Table 1:

| Category | Parameter | Value |
| --- | --- | --- |
| Network configuration | ISD | 750m |
|  | CHBW | 100 MHz |
|  | Propagation model | UMa in 38.901 (Pathloss, LOS probability, and O-to-I penetration loss) |
|  | Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
|  | Victim network | TDD UL (UE to BS) with handheld PC3 |
|  | Victim Network Outdoor/Indoor Ratio | 30/70% |
|  | Aggressor network | TDD UL (UE to BS) with FWA (same BW as victim) |
|  | Multi-operator layout | Coordinated (0% Grid Shift), uncoordinated |
|  | Shadowing correlation | Between cells: 1.0, Between sites: 0.5 |
|  | Additional shadowing @ FWA | 0 dB |
|  | FWA/PC3 Ratio | 100/0% as a starting point |
| FWA configuration | Outdoor/Indoor ratio | 70/30% as a starting point |
|  | Low/ high penetration loss | 50% low loss and 50% high loss |
|  | FWA distribution | Uniform distribution within each sector |
|  | Height distribution | Uniform distribution between 1.5 and 22.5m |
|  | FWA-BS minimum distance | 35 m |
|  | Beamforming | TBD |
|  | Additional Losses | 0 dB |
| gNB configuration | AAS parameters | See R4-2410576 as a starting point |
|  | Antenna height | 25m |
|  | BS noise figure | 5 dB |

Proposal 10: RAN4 to check spurious emissions for UE co-existence for investigated bands in this WI to make sure that the limits reflect practical deployments and updated assumptions.

# References

- RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, vivo (Moderator), RAN Meeting #109, Beijing, China.

- TR 36.837, Public safety broadband high power User Equipment (UE) for band 14, 3GPP.

- R4-2410576, LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4.