3GPP TSG-RAN WG4 Meeting #116bis	R4-2514057

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	RRM core requirements and test framework for AI in beam management

Agenda Item:	6.11.4

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #116, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Core requirements

In terms of the status report RP-252916, the below leftovers shall be addressed.

- RRM core requirement and testing framework for beam management Beam prediction KPIs - Beam ID prediction KPIs - Measurement period for inference - Rx beam knowledge and TCI state handling - Requirements based on relative difference - Absolute RSRP accuracy applicability - Simulation results - Measurement error modelling - LCM - Test system channel model - Test system setup

In the following content, we focus on the issues listed in the above framework.

### Issue 2-2: Prediction report delay

Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

In the light of the above agreement, the measurement delay in the prediction report delay refers to the delay of all measurements that validate the inference. Recall a previous agreement as follows:

- Agreement: - Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) - Case 2: postpone discussion until RAN1 concludes

For BM-case1, it is straightforward: the UE typically uses one L1-RSRP measurement to obtain a single prediction in the spatial domain.

For BM-case2, the number of L1-RSRP measurements depends on the adopted parameters, such as reference signal configurations, UE movement, the test channel, and so on.

Refer to agreement in RAN1 #122, which is duplicated in the below:

Agreement Support the following on the reporting condition of CSI reporting for inference - For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

It appears that after CSI report reconfiguration, i.e., with no historical measurement results, the UE must measure K times RS resources in Set B to produce the inference, where K is provided by the UE.

K may vary depending on UE capability, however RAN4 could not verify variants of K. From conformance testing perspective, RAN4 may define a common configuration, for example, four times measurement time instances with respect to a reference test configuration, which includes reference signal configurations,  UE movement, propagation channel and so on.

Observation 1: Measurement delay may depend on various configurations and applied channel. RAN4 shall define a single number/value with respect to a common configuration.

Proposal 1: Measurement delay is defined as follows:

- For BM-case1, Measurement delay is one shot L1-RSRP measurement delay.

- For BM-case2, Measurement delay is 4 * (L1-RSRP measurement delay).

Proposal 2: RAN4 shall define a reference configuration, e.g., reference signal configurations,  UE movement, channel for test and so on, to verify the prediction accuracy based on the agreed measurement delays.

Inference time is understood as the delay between the most recent measurement RS occasion and the time instant when the inference becomes valid.

Regarding the agreement ‘aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay, The reporting delay includes the inference delay’ , recall similar requirements for aperiodic report in TS38.133 as follow:

#### 9.5.3.3 Aperiodic Reporting Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

TS38.214 V18.7.0 has added the impact due to inference into UE CSI computation time by adding d and d' for AI case, as duplicated in the below:

- (Z3+d,Z3'+d'), with (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', where d and d' are according to UE reported capability as defined in [13, TS 38.306] and different values for d and d' may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report, or

Z is the interval from the end of the last symbol of the triggering PDCCH to the earliest uplink symbol where the UE can transmit the CSI report. In other words, Z is the minimum delay required for PDCCH decoding -> CSI processing -> uplink transmission.

Z': The interval from the end of the last relevant aperiodic measurement symbol to the earliest uplink symbol where the UE can transmit the corresponding CSI report. In other words, Z' is the minimum delay required for PDCCH decoding -> measurement acquisition -> CSI processing -> uplink transmission.

And, the added parameters: d and d' may be treated as the extra time delay for CSI processing time due to prediction. Between them, d' may be regarded as the inference time, e.g., a UE generates inference in d' after from the end of the last relevant aperiodic measurement symbol.

Observation 2: RAN4 to confirm that inference time is the delay between the most recent measurement RS occasion and the time instant when the inference becomes valid.

Proposal 3: The parameter d' for UE CSI computation in TS38.214  may be regarded as the inference time.

From a technical perspective, the inference time is assumed as previously described. The subsequent question is how RRM requirements specify inference time. Given that Clause 9.5.3.3 ‘Aperiodic Reporting’ in TS38.133 already refers to TS38.214, where prediction (i.e., inference time) is considered part of UE CSI computation, RAN4 does not need to define inference time separately.

Proposal 4: For aperiodic report, RAN4 to define the overall prediction reporting delay, which shall be defined in TS38.133, at least includes measurement delay + reporting delay. Wherein,

- Measurement delay: shall be specified by RAN4, and

- Reporting delay: as defined in Clause 9.5.3.3	‘Aperiodic Reporting’ in TS38.133 (without modification), which doesn’t need to restated.

Although it is agreed that ‘for semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion’, we consider this interpretation to be more of a technical perspective rather than a requirements-structure perspective. Since we already have Clause 9.5.3.1 ‘Periodic Reporting’ and Clause 9.5.3.2 ‘Semi-Persistent Reporting’ in existing TS38.133, in the conventional way, it is not conventionally necessary to define an explicit formula that introduces ‘time for the first available reporting occasion’ as a separate parameter.

Proposal 5: For semi-persistent and periodic report, RAN4 to define the overall prediction reporting delay, which shall be defined in TS38.133,  includes measurement delay +inference delay+time for the first available reporting occasion. Wherein,

- Measurement delay: shall be specified by RAN4, and

- Inference time: shall be specified by RAN4, as per UE capability, and

- Time for the first available reporting occasion: as defined in Clause9.5.3.1 ‘Periodic Reporting’ and Clause9.5.3.2‘Semi-Persistent Reporting’ in existing TS38.133, which doesn’t need to restated.

### Issue 2-3: TCI State Handling

The following agreement was reached in RAN4#116 meeting:

Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

First of all, the essential assumption of applying prediction in TCI state is: The RS resource for prediction is the RS in target TCI state or QCLed to the target TCI state.

Proposal 6: RAN4 to clarify that the use case of  prediction in TCI state is:

- The RS resource for prediction is the RS in target TCI state or QCLed to the target TCI state.

Another question that must be clarified is how to interpret the condition ‘The SSB associated with the TCI state remain detectable’.

The RS resource for prediction may or may not be the SSB associated with the TCI state.

- If the RS resource for prediction is the SSB associated with the TCI state, the (predicted) SSB associated with the TCI state remain detectable based on prediction.

- If the RS resource for prediction isn’t the SSB associated with the TCI state, the (measured) SSB associated with the TCI state remain detectable based on measurement.

Proposal 7: The term “detectable” in the condition ‘The SSB associated with the TCI state remain detectable’ should be clarified as follows:

- If the RS resource for prediction is the SSB associated with the TCI state, the (predicted) SSB associated with the TCI state remain detectable based on prediction.

- If the RS resource for prediction isn’t the SSB associated with the TCI state, the (measured) SSB associated with the TCI state remain detectable based on measurement.

Our understanding on the agreement: ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement’ is that the a beam prediction is valid for 1280 ms from the time instant when the prediction/inference is available or the time instant when the UE report the prediction results.

We observe that the phrase in the agreements ‘the last transmission of the RS resource for measurement’ directly inherited from the existing statement in Clause 8.10.2 of TS38.133 ‘the last transmission of the RS resource for beam reporting or measurement’ has vague meaning. It can’t precisely distinguish ‘the RS resource for the legacy measurement’ and the ‘the RS resource for prediction purpose measurement’.

In our opinion, to make it be clearer, the statement can be updated to:

- ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement for prediction.’, or

- ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement for prediction.’

Proposal 8: Update known condition to one of the following options:

Option 1: ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement for prediction.’

Option 2: ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement for prediction.’

Following the definition of the known and unknown conditions, the next step is to define the TCI state switching delay with prediction. As per the existing delay requirements in Clause 8.10 of TS38.133, the following delays are specified:

- MAC-CE based TCI state switching delay

  - Specifically, MAC-CE based TCI state switching delay in HST FR2 scenarios.

- DCI based TCI state switching delay

- RRC based TCI state switching delay

Since the known/unknown condition is applicable to MAC-CE based and RRC based TCI state switching, it is understood that the WI aims to define prediction only for these two cases by introducing a prediction based known/unknown condition.

Observation 3: Prediction only applies for MAC-CE based and RRC based TCI state switching, with prediction based known/unknown condition, DCI based TCI state switching delay isn’t impacted.

Between that, as a reference, MAC-CE based TCI state switch defines the delay as duplicated in the below:

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state - TSSB-proc = 2 ms; - TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise. If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Apparently, in conventional way, It is mandatory for the UE to measure SSB for consequent operations after the reception of a MAC-CE command. Specifically, the UE shall be able to measure the first SSB transmission under the known condition, or measure SSB/CSI-RS for L1-RSRP plus the first SSB transmission under the unknown condition.

We observe that prediction may occur prior to receiving the MAC-CE command and/or after receiving the MAC-CE command during the TCI state switching period.

- Before the TCI state switching period,  the predicted Tx beam in Set A might not be transmitted/configured for UE to measure with respect to the prediction condition.

- During the TCI state switching period,

  - The first SSB shall always be measured, as we understand that timing synchronization might not be within the current prediction scope, and the first SSB may not have a mandatory QCL relationship (e.g., QCL-typeA or TypeD) with the predicted Tx beam in Set A.

    - If the predicted Tx beam in Set A is the first SSB, it shall be transmitted and configured for the UE to measure (this SSB must have a QCL-TypeA or QCL-TypeD relationship with the target TCI state).

    - If the predicted Tx beam in Set A is not the first SSB, it does not need to be transmitted or configured for measurement.

  - Regarding L1-RSRP measurement, which is only valid for the unknown condition, we understand that:

    - If The predicted Tx beam in Set A is the RS for L1-RSRP, the predicted Tx beam in Set A shall be transmitted and configured for UE to measure.

    - If The predicted Tx beam in Set A isn’t the RS for L1-RSP, the predicted Tx beam in Set A doesn’t need to be transmitted/configured for UE to measure.

Observation 4: The TCI state switch delay may account for the predicted Tx beam in Set A during the switching period. Its transmission can be summarized as follows:

- The predicted Tx beam in Set A shall be transmitted/configured for UE to measure, if the predicted Tx beam in Set A is the first SSB and/or the RS for L1-RSRP, otherwise

- the predicted Tx beam in Set A doesn’t need to be transmitted/configured for UE to measure, if the predicted Tx beam in Set A isn’t the first SSB and/or the RS for L1-RSRP.

Although certain optimizable cases can be observed by incorporating the predicted Tx beam in Set A into the L1-RSRP measurement for the TCI state switch delay, it is preferable to define only a more common case where the L1-RSRP measurement for the unknown condition is independent of the predicted Tx beam in Set A.

Proposal 9: RAN4 to confirm that prediction only impacts the definition of known conditions in Clause 8.10.2 of TS38.133, and does not impact the fundamental delay requirements specified for MAC-CE based TCI state switching in Clause 8.10.3 of TS38.133 and for RRC based TCI state switching in Clause 8.10.4 of TS38.133.

Proposal 10: RAN4 to confirm that prediction based known conditions for TCI state are also applicable for Clause 8.10.3A ‘MAC-CE based TCI state switch delay in HST FR2 scenarios’ of TS38.133.

### Issue 2-9: Metrics/KPIs for beam ID prediction

Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

The summary of this issue in RAN4 meeting #115 is presented as follows:

Issue 2-1: Metrics/KPIs for beam ID prediction Proposals - Option 1: adopt N - N=2 - Further study the actual value - N>2 - N= K2+1 - Option 2: Do not use N, remove the FFS on top-N - Option 3: x<3dB - Option 4: further study x based on simulations - Option 5: Others (e.g. different distributions, etc) - Recommended WF - Option 1/2 and 3/4 to be discussed separately

Meanwhile, recall the WF in RAN4 meeting #114bis as the below:

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Furthermore, recall the WF in RAN4 meeting #114 as the below:

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Proposals - Option 1: - In case of the AI/ML model with label (beam ID) output type - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted. - In case of the AI/ML model with label (beam ID) + RSRP output type - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously. - Option 2: Option 2 with a combination of Top-K/1 and Top 1/K - Option 2a: update option 2 to Top-N/K, the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". - Option 3: Option 3 as it does not have the issue of UE failing if there are multiple beams within a small RSRP margin - Option 4: combinations of options, concrete proposals should be disucssed - Recommended WF - To be discussed The number of options should be downselected to make some progress or a clear path to choose an option should be discussed/agreed

First of all, regarding the baseline metric for beam prediction:

"A correct prediction is considered valid if the maximum ground-truth RSRP among the top K predicted beams is greater than or equal to the ground truth RSRP of the strongest genie aided beam(s) minus x dB."

In our view, this metric evaluates beam prediction accuracy by comparing the ground truth RSRP of the predicted beams against that of the genie aided beams. While this approach provides an agreed reference metric, it raises two concerns:

- Concern 1: Determination of the parameter K

- Concern 2: Determination of the parameter x

Regarding the first concern, although the choice of K may depend on practical implementation, it is critical for assessing prediction correctness.

It is observed that the top-K beams are ranked in a specific order, even when L1-RSRP is not reported by the model-according to the agreement reached in the RAN1 #121 meeting as follows:

Agreement For UE-sided model, for BM-Case 1 and BM-Case 2, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information.

However, the interpretation of the ranking information for the top-K beams remains unresolved in RAN1. One proposal suggests that the predicted top-K beams should be defined as the best K beams based on either the predicted RSRP values, or the probability of each beam in Set A being the top-1 beam.

Observation 5: For the case that only beam ID is reported, the order of top-K beams indicates the order of RSRP or the probability of each beam in Set A to be the Top-1 beam of implicitly.

Returning to the test metrics, the ranking information is not considered in the beam prediction metrics defined in RAN4. For the baseline beam prediction metric, the ground-truth RSRP of each beam among the top-K predicted beams must be evaluated to identify the single beam with the maximum ground-truth RSRP.

The following example illustrates the issue related to the choice of parameter K.

Assume K=2, with two reported beams ordered as the 1st (rank 1) beam and the 2nd (rank 2) beam. Among these, the ground-truth RSRP of the 2nd beam is higher and meet the test metrics.  Under the current metric definition, the 1st beam is not required to pass any validation in this case, e.g., the AI/ML model has a defect causing the 1st beam to have a significantly lower ground-truth RSRP than the 2nd beam, it still passes the evaluation.

This creates a risk that the network might select the 1st beam based on the UE’s top-K beam report, potentially leading to degraded performance due to suboptimal or poor beam selection. Furthermore, as K increases, the evaluation criterion becomes more ‘relaxed’, leading to greater variability and inconsistency in UE performance.

Some opposing views argue that since the 2nd beam meets the test metrics, there is no need to impose constraints on the 1st beam—considering it an acceptable error tolerance within the top-K beam report.

In our opinion, this approach can lead to degraded performance in the field. The NW may select one or more beams from the top-K list whose quality is not guaranteed, potentially resulting in incorrect configurations or commands. Even if the network performs subsequent measurement procedures to verify the predicted beam quality, such steps introduce inevitable delays—contradicting the design target of applying AI/ML model for beam management..

Furthermore, from the RAN4 perspective, requirements should remain deterministic. RAN4 must ensure that all UEs are evaluated against consistent performance criteria; otherwise, it becomes difficult for the network to manage configurations across plenty of UEs.

Yet, ensuring the accuracy of all beams reported by the UE is essential, whether determined through prediction or legacy measurement. If error tolerance were acceptable for all reported beams, there would be no need to define strict accuracy requirements for each beam. However, in the legacy measurement solution, measurement accuracy is still defined for every beam. Based on this principle, prediction accuracy should not be more relaxed than measurement accuracy.

Observation 6: Only considering the baseline:

- Top-K/1 (%), and

- The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

may result in a wrong, e.g., bad quality, beam being adopted by NW, and results in degraded performance than the legacy measurement solution.

Below, we briefly express our views with respect to the enhancement proposals based on the baseline metric for beam prediction.

Approach 1:  Verifying both K=1 case and K>1 case, for top K/1 (%) based on the baseline of the metrics for beam prediction. Given the above analysis in a straightforward and intuitive way, the K=1 case serves as a proper complement to the K > 1 case. Specifically, the K=1 case requires the UE being able to report the best (rank 1) beam with sufficient ground-truth RSRP, regardless of whether there exists another beam with higher ground-truth RSRP in the K > 1 case.

Approach 2:  Verify more than one beam, for example, N beams within the top-K predicted beams. The purpose of considering N beams here is to leverage the prediction accuracies of multiple predicted beams to enhance the baseline: ‘the beam with maximum ground-truth RSRP among top-K predicted beams’.  From high level perspective, the approach shall consider the below:

- The verification shall guarantee the ground-truth RSRP of the N beams not lower than a threshold (e.g., -x dB) with respect to the ground-truth RSRP of the top-K beams.

  - The exact formula can be studied if the general approach is agreed.

- Along with the first bullet, we’re open to apply Top-K/1 (%) or Top-K/N (%) together with ground-truth RSRP verification.

- Wherein, the N beams may have highest ground-truth RSRP or be highest ranked among top-K beams.

However, Approach 2 deviates somewhat from the baseline, as it introduces N, which could be regarded as a variant of the Top-K/N (%) method.

Observation 7: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, rather than only one or some of them, is advantageous for enhancing reliability and robustness in beam management.

Proposal 11:  Enhance the baseline metric for beam prediction by testing both cases: K = 1 and K = 2 or 3, with the beam prediction metrics baseline as follow:

- ‘Top-K/1 (%)’, and

- ‘The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB’.

Regarding the second concern, the parameter ‘x’ dB, which essentially represents a margin factor, it may be influenced by measurement errors as well as other prediction errors occurring during both training and inference phases. Therefore, it is expected that ‘x’ should be chosen to match or closely approximate the current measurement error. While TR 38.843 references 1 dB as a margin for simulations, this value remains subject to doubt.

Proposal 12: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors, [1] dB may be taken as a baseline.

Furthermore, regarding AI/ML models supporting RSRP reporting, a key issue was raised in the last meeting: Should both RSRP and beam ID be subject to testing?

To our understanding, for AI/ML models that output both a label (i.e., beam ID) and RSRP, it is essential that test metrics account for both prediction accuracy and RSRP accuracy.

An objection to this approach may be that test metrics could follow the legacy method, where only RSRP is verified. However, this may be applicable only when testing a single beam or a very limited number of beams with deterministic beam ID/index information. In contrast, beam prediction test typically involves the UE reporting a set of beams, each predicted, indexed, and sorted in descending order of RSRP.

Using the top-1 predicted beam as an example, two accuracy requirements can be distinguished:

- RSRP accuracy: The difference between the predicted L1-RSRP and the ground-truth L1-RSRP of the top-1 predicted beam should be less than or equal to y dB.

- Beam accuracy: The difference between the ground-truth RSRP of the top-1 predicted beam and that of the top-1 genie-aided beam should be less than or equal to y dB.

If only RSRP accuracy is assessed —without any comparison to the genie-aided beam (i.e., the ideal beam determined with full knowledge)—there is no guarantee that the top-1 predicted beam is meaningfully related to the optimal beam since no comparison to genie-aided beam. In such case, a poorly performing prediction model could still satisfy RSRP accuracy requirements while selecting a beam that is far from optimal, undermining the purpose of accurate beam prediction.

Observation 8: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlying problem is that the Top-1/top-K predicted beam is far away from the Top-1/top-K genie-aided beam since no comparison to genie-aided beam is performed.

Proposal 13: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously.

Furthermore, It is predictable that adding RSRP accuracy to beam ID prediction demands higher model precision and robustness.

Observation 9: For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type.

### Issue: Absolute RSRP accuracy

Yet there exists a leftover regarding the absolute RSRP accuracy with respect to the texts in the bracket in the previously achieved agreement: ‘[in top-K beams based on ground truth L1-RSRP]’.

In our opinion, the focus of RAN4 should be on the predicted beams—that is, the prediction accuracy should concentrate on the beams reported by the UE, rather than those provided by the genie-aided method. It's questionable to adopt ‘the top-K beams w.r.t. the ground truth RSRP’. Two concerns are provided below:

- If the requirement is only relevant to the top-K beams with respect to the ground truth RSRP, does the beam's RSRP have no requirements if it is present in the top-K predicted beams but not in the genie beams?

- Conversely, why should we need the RSRP accuracy of a beam that the UE doesn't even report if it is present in the top-K genie beams but not in the predicted beams?

Proposal 14: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting, i.e.,

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index in top-K beams based on prediction report.

### Issue 2-3: Relative RSRP accuracy

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Regarding the first FFS on ‘whether reported L1-RSRP can be measured RSRP’, the origin of the question comes from RAN1 study, which considered whether the UE could use measured RSRP instead of predicted RSRP in the prediction report, as outlined below:

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Later, in RAN1 #120, the following agreement was reached:

Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Based on this, we understand that the reported L1-RSRP shall be the predicted L1-RSRP. Furthermore, from the RAN4 perspective, we understand that the evaluation of prediction should clearly separate predicted and measured data by design. This separation prevents any bias or overlap that could compromise the integrity of prediction performance assessment.

Proposal 15: Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

### Issue : Performance monitoring

Issue 2-7: LCM Requirements - Proposals - Option 1: the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2. - Option 2: For performance monitoring for BM, the corresponding delay is defined as - Tmonitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS, - Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - Option 3:postpone discussion until RAN1/2 monitoring framework is clear

Regarding performance monitoring, based on RAN1’s agreements, as cited in the below, we may focus on Type 1, Option 2 of UE-side model monitoring.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In RAN WG1 #118, the below agreements were achieved:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN WG1 #118bis, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

In RAN WG1 #119, the below agreements were achieved:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Our understanding is that, for both BM‑case 1 and BM‑case 2, when a UE‑side AI/ML model is used under Option 2 (UE‑assisted performance monitoring), the UE should be capable of comparing the differences between the prediction results and the measurement results over a designated resource set or specific resources used for monitoring.

And further relevant agreements in RAN #121 defines the timeline of monitoring for BM-Case 1:

Agreement - At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately - The associated working assumption made in RAN1#120b will not be confirmed.

Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1)

Based on RAN1 agreements to date, we have the below observations.

- The performance monitoring will be considered for different CSI-report configurations: (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2. RAN1 will continue the discussions on the associated timeline separately.

- Measurement result on each RS occasions will be reported, there are total N measurement results.

- Report quantity RS-PAI is Np is defined as the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s).

Proposal 16: RAN4 to define prediction monitoring requirements for performance monitoring Type 1 Option 2 (UE-assisted performance monitoring)

Proposal 17: RAN4 shall take the below RAN1 agreements into performance monitoring requirements:

- Evaluation period for performance monitoring:

  - N transmission occasion of the CSI-RS/SSB resources for monitoring, at least for BM-case 1, Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig.

- Report quantity RS-PAI,

  - Np is defined as the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s).

## Testability and interoperability

### Issue 4-1: Baseline test setup

- Proposals - Option 1: Single AoA setup (different Tx beams emulated only with power difference?) - Is it possible to test if UE can also make a prediction when set B beams come from different directions? - How many Tx beams can be emulated when the spatial domain properties are lost? - Will UE need just a simple interpolation model or is a more complex model necessary to pass such a test? - Can additional (L1-)RSRP measurement accuracy requirement (e.g. spherical coverage) be used to complement Single AoA testing with directional/spatial UE performance? - Option 2: Single AoA setup with “two stage approach” - Will a UE which uses Rx beam information in the prediction model have to also implement a model which does not use the Rx beam information? - Are there any implementation constraints on a UE to execute/pass this test? - How will the TE know the UE Rx beam pattern? - Option 3: multi AoA setup– RRM test setup as baseline (including enhancement for up to 4AoA active probes) - How many Tx beams can be emulated with such a setup, what is the relationship between number of probes and number of set B Tx beams? - Can AoAs between the probes also be emulated (is it possible to “make it look like” signals are coming from other angles than where the probes are placed?) - Can a simplified CDL be emulated or only TDL from each probe? - Option 4: multi AoA setup – 3D MPAC system as baseline - How many Tx beams can be emulated with the baseline setup? - 4Tx beams with dual pol(set B?) can be emulated with 2 additional probes Pros&cons of different test setups - Option 5: multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. Single AoA; Single AoA with two stage; Multi AoA – RRM test setup as baseline; Multi AoA 3D MPAC. Tests that UE uses spatial information(UE actually makes measurement in multiple directions to make a prediction); No; Maybe?; Yes; Yes. Relevance for field performance; Low; Maybe?; Good/High; Good/High. Simplified CDL emulation; No; Yes; Yes. Test setup complexity; Low; Low; Medium; Medium/High?. Test method complexity; Low; High; Medium; Medium/High?. Reuse of test setup for other tests; Medium (multi AoA testing already exists in RRM); High?; Low/medium? Can only reuse the test setup for FR2 MIMO OTA?. - Recommended WF - Do not further discuss single AoA test setups, continue to study the two multi AoA candidate setups

Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

### Issue: Channel model

- Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others - Recommended WF - To be discussed If simplified CDL is to be further considered, more detailed proposals on what to study/analyze further should be discussed (e.g. RX PAS, number of relevant clusters to emulate, dependency on number of clusters and number of probes in the chamber, etc)

Agreement: - Use CDL-based channel model as starting point

From the discussions in meeting, it’s clear that there exist two options in last meetings.

- Option 1: only inputting L1-RSRP of beams, single AoA with CDL/TDL channel can be applied.

- Option 2: inputting beam information comprising L1-RSRP, Tx beam spatial distribution and Rx beam spatial distribution, multiple AOA with CDL channel shall be applied.

Option1 supposes that by emulating artificial Tx/Rx beam into the incoming beams, the single AoA test configuration can generate the needed L1-RSRP of beams.

Figure 1 Single AOA flow

However the practical Rx beam plays a crucial role in FR2 UE performance, a question to Option 1 is whether the difference between really tested Tx/Rx beam and the artificial Tx/Rx beam can impact AI/ML model.

Proposal 18: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model and the prediction accuracy?

The simplification of the CDL channel is achieved by constraining the directions Tx beams and and AOA/AOD. Observations regarding possible approaches to simplifying AOA/AOD can be found in R4-2500825, R4-2411254, and other relevant contributions. The simplified CDL channel consolidates similar AOA/AOD angles to enable spatial evaluation with a reduced number of probes.

Given that the maximal number of beams to be measured is 32, a important consideration is whether the 32 beams after simplified CDL channel can be produced by 8 or 12 probes thereby achieving the balance between performance and implementation complexity.

Proposal 19: RAN4 to check if the 32 beams after simplified CDL channel can be produced by [8] (if it is feasible to be realized in test bed) probes which may achieve the balance between performance and implementation complexity.

Proposal 20: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

# Conclusion

Observation 1: Measurement delay may depend on various configurations and applied channel. RAN4 shall define a single number/value with respect to a common configuration.

Observation 2: RAN4 to confirm that inference time is the delay between the most recent measurement RS occasion and the time instant when the inference becomes valid.

Observation 3: Prediction only applies for MAC-CE based and RRC based TCI state switching, with prediction based known/unknown condition, DCI based TCI state switching delay isn’t impacted.

Observation 4: The TCI state switch delay may account for the predicted Tx beam in Set A during the switching period. Its transmission can be summarized as follows:

- The predicted Tx beam in Set A shall be transmitted/configured for UE to measure, if the predicted Tx beam in Set A is the first SSB and/or the RS for L1-RSRP, otherwise

- the predicted Tx beam in Set A doesn’t need to be transmitted/configured for UE to measure, if the predicted Tx beam in Set A isn’t the first SSB and/or the RS for L1-RSRP.

Observation 5: For the case that only beam ID is reported, the order of top-K beams indicates the order of RSRP or the probability of each beam in Set A to be the Top-1 beam of implicitly.

Observation 6: Only considering the baseline:

- Top-K/1 (%), and

- The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

may result in a wrong, e.g., bad quality, beam being adopted by NW, and results in degraded performance than the legacy measurement solution.

Observation 7: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, rather than only one or some of them, is advantageous for enhancing reliability and robustness in beam management.

Observation 8: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlying problem is that the Top-1/top-K predicted beam is far away from the Top-1/top-K genie-aided beam since no comparison to genie-aided beam is performed.

Observation 9: For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type.

Proposal 1: Measurement delay is defined as follows:

- For BM-case1, Measurement delay is one shot L1-RSRP measurement delay.

- For BM-case2, Measurement delay is 4 * (L1-RSRP measurement delay).

Proposal 2: RAN4 shall define a reference configuration, e.g., reference signal configurations,  UE movement, channel for test and so on, to verify the prediction accuracy based on the agreed measurement delays.

Proposal 3: The parameter d' for UE CSI computation in TS38.214  may be regarded as the inference time.

Proposal 4: For aperiodic report, RAN4 to define the overall prediction reporting delay, which shall be defined in TS38.133, at least includes measurement delay + reporting delay. Wherein,

- Measurement delay: shall be specified by RAN4, and

- Reporting delay: as defined in Clause 9.5.3.3	‘Aperiodic Reporting’ in TS38.133 (without modification), which doesn’t need to restated.

Proposal 5: For semi-persistent and periodic report, RAN4 to define the overall prediction reporting delay, which shall be defined in TS38.133,  includes measurement delay +inference delay+time for the first available reporting occasion. Wherein,

- Measurement delay: shall be specified by RAN4, and

- Inference time: shall be specified by RAN4, as per UE capability, and

- Time for the first available reporting occasion: as defined in Clause9.5.3.1 ‘Periodic Reporting’ and Clause9.5.3.2‘Semi-Persistent Reporting’ in existing TS38.133, which doesn’t need to restated.

Proposal 6: RAN4 to clarify that the use case of  prediction in TCI state is:

- The RS resource for prediction is the RS in target TCI state or QCLed to the target TCI state.

Proposal 7: The term “detectable” in the condition ‘The SSB associated with the TCI state remain detectable’ should be clarified as follows:

- If the RS resource for prediction is the SSB associated with the TCI state, the (predicted) SSB associated with the TCI state remain detectable based on prediction.

- If the RS resource for prediction isn’t the SSB associated with the TCI state, the (measured) SSB associated with the TCI state remain detectable based on measurement.

Proposal 8: Update known condition to one of the following options:

Option 1: ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement for prediction.’

Option 2: ‘TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement for prediction.’

Proposal 9: RAN4 to confirm that prediction only impacts the definition of known conditions in Clause 8.10.2 of TS38.133, and does not impact the fundamental delay requirements specified for MAC-CE based TCI state switching in Clause 8.10.3 of TS38.133 and for RRC based TCI state switching in Clause 8.10.4 of TS38.133.

Proposal 10: RAN4 to confirm that prediction based known conditions for TCI state are also applicable for Clause 8.10.3A ‘MAC-CE based TCI state switch delay in HST FR2 scenarios’ of TS38.133.

Proposal 11:  Enhance the baseline metric for beam prediction by testing both cases: K = 1 and K = 2 or 3, with the beam prediction metrics baseline as follow:

- ‘Top-K/1 (%)’, and

- ‘The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB’.

Proposal 12: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors, [1] dB may be taken as a baseline.

Proposal 13: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously.

Proposal 14: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting, i.e.,

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index in top-K beams based on prediction report.

Proposal 15: Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Proposal 16: RAN4 to define prediction monitoring requirements for performance monitoring Type 1 Option 2 (UE-assisted performance monitoring)

Proposal 17: RAN4 shall take the below RAN1 agreements into performance monitoring requirements:

- Evaluation period for performance monitoring:

  - N transmission occasion of the CSI-RS/SSB resources for monitoring, at least for BM-case 1, Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig.

- Report quantity RS-PAI,

  - Np is defined as the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s).

Proposal 18: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model and the prediction accuracy?

Proposal 19: RAN4 to check if the 32 beams after simplified CDL channel can be produced by [8] (if it is feasible to be realized in test bed) probes which may achieve the balance between performance and implementation complexity.

Proposal 20: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

# References

- R4-2511890 WF on AI/ML, Qualcomm