3GPP TSG-RAN WG4 Meeting #116bis	R4-2514064

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements for NTN Ku band

Agenda Item:	5.10.2

Document for:	Discussion

# Introduction

This contribution outlines our point of view on on the RRM Performance requirements for NTN Ku band.

# Discussion

NTN Ku band UEs essentially include two types of UEs: mobile VSAT UEs operating with FR1-NTN numerology and mobile VSAT UEs operating with FR2-NTN numerology.

From a requirements perspective, same requirements apply to mobile VSAT UEs operating with FR2-NTN numerology as to VSAT UEs with FR2-NTN numerology defined in Rel-18.

In discussions on the core part, it was generally agreed that, unless otherwise stated, the requirements defined for FR2-NTN UEs also apply to VSAT UEs operating in cells with FR1-NTN numerology. Based on this, the actual updates to the core part for NTN Ku-band include the following requirements:

- Satellite switching with re-synchronization

- UE transmit timing for Satellite Access

Among these,

- Satellite switching with re-synchronization is applied for NTN Ku band UEs and VSAT UEs operating with FR2-NTN numerology defined in Rel-18. Since this is a new requirement, even for FR2-NTN, a dedicated test case is expected.

- UE transmit timing add a limitation for antenna type of applicable NTN Ku UE. In this case, adding a note to the existing corresponding test case is sufficient.

Given the above status, it is preferred to clarify the applicability of tests for FR2-NTN by indicating those tests are applicable for both mobile VSAT UEs operating with FR1-NTN numerology and VSAT UEs operating with FR2-NTN numerology.

Proposal 1: In performance part, add notes in the corresponding FR2-NTN test cases to clarify that they are applicable to NTN Ku band UEs, i.e., both mobile VSAT UEs operating with FR1-NTN numerology and VSAT UEs operating with FR2-NTN numerology.

Proposal 2: Design a test case for satellite switching with re-synchronization for NTN Ku band UEs and FR2-NTN UEs.

# Conclusion

Proposal 1: In performance part, add notes in the corresponding FR2-NTN test cases to clarify that they are applicable to NTN Ku band UEs, i.e., both mobile VSAT UEs operating with FR1-NTN numerology and VSAT UEs operating with FR2-NTN numerology.

Proposal 2: Design a test case for satellite switching with re-synchronization for NTN Ku band UEs and FR2-NTN UEs.

# References

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT